3GPP TSG-RAN WG4 Meeting #94-e	R4-2001352

Electronic meeting, 24 February – 6 March 2020

Agenda Item:	7.10.4

Source:	Ericsson

Title:	Discussion on UE demodulation and CSI reporting requirements for Rel-16 BL/CE UE

Document for:	Discussion

# 1	Introduction

RAN4#93 agreed with the way forward on the UE demodulation and CSI reporting requirements for Rel-16 BL/CE UEs. [1].

- RAN4 to introduce: - MPDCCH demodulation requirements due to MPDCCH performance improvement. - CSI reporting requirements due to the introduction of CSI-RS based feedback - FFS to introduce new UE demodulation requirements for - Multi-TB scheduling for PDSCH - MPDCCH/PDSCH demodulation requirements due to the use of LTE control region - No other UE demodulation and CSI reporting requirements are introduced

This contribution discussed the simulation assumption for the MPDCCH performance improvement and CSI reporting.

# 2	MPDCCH performance improvement

Improved MPDDCH introduced in Rel-16 uses CRS+DMRS for MPDCCH demodulation, although the legacy Rel-13 MPDCCH supports DMRS-based demodulation only. Since CRS is the cell-specific reference symbols, RAN1 specified the relation between the symbols transmitted on MPDCCH antenna ports (e.g., 107, 109) and CRS ports (e.g., 0, 1). For example, in the case of 2 CRS ports with distributed transmission, eNB applies the precoder matrix for single layer transmission specified in TS36.211 Table 6.3.4.2.3-1. Antenna ports 107 and 109 always use code book index 0 (i.e., 121;1) and code book index 1 (i.e., 121;-1), respectively according to RAN1 specification. For localized transmission, RAN1 agreed to support the CSI-based or reciprocity-based precoding. To avoid the impacts due to the CSI estimation error, we propose to use the fixed precoding mode for MPDCCH demodulation requirements.

Since there are different precoding for distributed transmission and localized transmission, we propose to verify the distributed transmission for CE Mode A and the localized transmission for CE Mode B, which is the same configuration as Rel-13 MPDCCH demodulation requirements.

Proposal 1: RAN4 specifies two MPDCCH demodulation requirements with CRS+DMRS:

- Distributed transmission mode, CE Mode A, 2 CRS ports, AL16, [Rep16], EPA5

- Localized transmission mode, CE Mode B, 2 CRS ports, AL24, [Rep32], ETU1

Table 1 is the proposed MPDCCH parameters and Table 2 lists the test cases.

Table 1	Proposed MPDCCH parameters.

| Parameter |  | Unit | Test 1 (CE Mode A) | Test 2 (CE Mode B) |
| --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | -3 | 0 |
|  |  | dB | -3 | 0 |
|  |  | dB | 0 | -3 |
|  | δ | dB | 3 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  | PRB | N/A | See TS36.211 6.8B.5 |
|  |  | ms | N/A | See TS36.211 6.8B.5 |
| Beamforming Pre-Coder |  |  | See TS36.211 6.8B.5 | See TS36.211 6.8B.5 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 | Port 0 and 1 |
| Number of PRB per MPDCCH Set |  |  | 4 | 2+4 |
| Transmission type |  |  | Distributed | Localized |
| Frequency hopping |  |  | Disabled | Enabled |
| Number of frequency hopping narrowbands |  |  | N/A | 4 |
| Frequency hopping offset |  |  | N/A | 1 |
| Frequency hopping interval |  | ms | N/A | 16 |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) (Note 3) |  |  | 1.5 | 1.5 |
| Maximum number of repetitions (mPDCCH-NumRepetition) |  |  | [16] | [32] |
| MPDCCH repetition number |  |  | [16] | [32] |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 | 7 |
| PDSCH TM |  |  | TM2 | TM2 |
| DCI Format |  |  | 6-1A | 6-1B |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1111111111 | 1111111111 |
| mpdcch-crs-config |  |  | Configured | Configured |
| Power offset between CRS and DMRS antenna ports of MPDCCH |  | dB | 0 | 0 |
| mpdcch-crs-localized-mapping-type |  |  | N/A | Not configured (cyclic precoder) |
| Note1: For each test, DC subcarrier puncturing shall be considered. Note2: Same precoding matrix is used for a PRB across subframes during the frequency hopping interval. Note 3: For MPDCCH UE-specific search space the formula for the start subframe k0 is given in TS 36.213 [6] clause 9.1.5. Note 4: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |

Table 2	MPDCCH test cases

| Test number | Bandwidth | Aggregation level | Reference Channel (TS 36.101) | Propagation condition | Antenna configuration | Pm-dsg (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10MHz | 16 ECCE | R.82 FDD | EPA5 | 2x1 low | 1 |
| 2 | 10MHz | 24 ECCE | R.83 FDD | ETU1 | 2x1 low | 1 |

# 3	CSI-RS based CSI reporting

For the Non-BL UE performance improvement, RAN1 has introduced the CSI-RS based feedback for non-BL UE configured with 8 CRS ports. According to the RAN1 agreements, the CSI-RS based CSI reporting is supported only on periodic CSI reporting with mode 1-1 if the UE is configured with PMI/RI reporting, the number of CSI-RS ports=8, and the UE is configured with higher layer parameter ce-csi-rs-feedback-config.

For a non-BL UEs configured with CEModeA and PUCCH mode 1-1 for 8 CSI-RS ports, both PUCCH mode 1-1 submode 1 and PUCCH mode 1-1 submode 2 corresponding to rank 1 can be used.

For the reference we show below Table 7.2.2-1E and Table 7.2.2-1D from TS36.213, corresponding to PUCCH mode 1-1 submode 1 with RI=1 and corresponds to PUCCH mode 1-1 submode 2 with RI=1, respectively. Reviewing the existing LTE CSI reporting test cases with PUCCH mode 1-1 in TS36.101, submode 1 is mostly used. We propose to define the CSI-RS based PMI reporting test with the periodic PUCCH 1-1 submode 1.

Proposal 2: RAN4 specify the CSI-RS based PMI reporting test with periodic PUCCH 1-1 submode 1.

Table 7.2.2-1E: Joint encoding of RI and  for PUCCH mode 1-1 submode 1

| Value of joint encoding of RI and the first PMI | RI | Codebook index |
| --- | --- | --- |
| 0-7 | 1 |  |

Table 7.2.2-1D: PUCCH mode 1-1 submode 2 codebook subsampling

| RI | Relationship between the first PMI value and codebook index |  | Relationship between the second PMI value and codebook index |  | total |
| --- | --- | --- | --- | --- | --- |
|  | Value of the first PMI | Codebook index | Value of the second PMI | Codebook index | #bits |
| 1 | 0-7 |  | 0-1 |  | 4 |

For the PMI reporting test, RAN4 uses the throughput ratio of followed PMI over the random PMI as follows:

γ=tuetrnd In the definition of γ, tue is TBD % of the maximum throughput obtained at SNRfollow using the precoders configured according to the UE reports, and trnd is the throughput SNRfollow with random precoding.

We propose to reuse this metric for the CSI-RS based PMI reporting tests for non-BL UE.

Proposal 3: For the CSI-RS based PMI reporting test, set metric as the throughput ratio of the follow PMI over the random PMI. FFS for the SNR test point.

Table 3 shows our proposal on the test parameters for CSI-RS based PMI reporting tests. Since this feature is only applicable for non-BL UEs, we set the antenna configuration to 8x2 XPOL high correlation.

Table 3	Test parameters for CSI-RS based PMI reporting test.

| Parameters |  | Unit | Values |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| PDSCH transmission mode |  |  | 9 |
| Propagation channel |  |  | EPA5 |
| Precoding granularity |  | PRB | 6 |
| Downlink power allocation | ρA | dB | 0 |
|  | ρB | dB | 0 |
|  | σ | dB | -3 |
|  | δ | dB | 0 |
| CRS reference signals |  |  | Antenna ports 0, 1 |
| CSI reference signals |  |  | Antenna ports 15,…,22 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |
| CSI reference signal configuration |  |  | 0 |
| Propagation condition and antenna configuration |  |  | High XP 8 x 2 |
| Beamforming Model |  |  | As specified in TS36.101 B.4.3 |
| CodeBookSubsetRestriction bitmap |  |  | TBD |
| SNR |  | dB | TBD |
|  |  | dB[mW/15kHz] | TBD |
|  |  | dB[mW/15kHz] | -98 |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Reporting mode |  |  | PUCCH 1-1 submode1 |
| Physical channel for CQI/PMI reporting |  |  | PUSCH |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |
| Reporting periodicity |  | ms | 10 |
| PMI delay |  | ms | 10 |
| cqi-pmi-ConfigurationIndex |  |  | 12 |
| ce-csi-rs-feedback-config |  |  | Configured |
| Frequency hopping |  |  | Disabled |
| Frequency hopping inverval (interval-FDD) |  |  | N/A |
| Starting OFDM symbol (startSymbolBR) |  |  | 3 |
| PDSCH repetition level |  |  | 1 |
| MPDCCH repetition level |  |  | 1 |
| Beamforming Precoder for MPDCCH |  |  | No precoding |
| Precoder update granularity for MPDCCH |  |  | N/A |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1111111111 |
| PDSCH PRB size |  | PRB | 3 |
| PDSCH MCS |  |  | [QPSK 1/2] |

# 4	Summary

Proposal 1: RAN4 specifies two MPDCCH demodulation requirements with CRS+DMRS:

- Distributed transmission mode, CE Mode A, 2 CRS ports, AL16, [Rep16], EPA5

- Localized transmission mode, CE Mode B, 2 CRS ports, AL24, [Rep32], ETU1

Proposal 2: RAN4 specify the CSI-RS based PMI reporting test with periodic PUCCH 1-1 submode 1.

Proposal 3: For the CSI-RS based PMI reporting test, set metric as the throughput ratio of the follow PMI over the random PMI. FFS for the SNR test point.

# 5	References

- R4-1915876, “Way forward on UE/BS demodulation performance for additional MTC enhancements for LTE”.

# Appendix

Table 7.2.4-1: Codebook for 1-layer CSI reporting using antenna ports 15 to 22

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 15 |  |  |  |  |  |  |  |  |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 0 - 15 |  |  |  |  |  |  |  |  |
| where |  |  |  |  |  |  |  |  |