3GPP TSG-RAN4 Meeting #94-e	R4-2001394

# Online, , 24th Feb 2020 - 6th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6800 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to clause 1-3 (General) for NR-U in 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of abbreviations and NR-U requirements applicability in general sections |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add abbreviations and applicable requirements for different NR-U operating conditions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR-U requirements are incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 3.5.2, 3.6, 3.6.7(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change 1

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

BFD	Beam Failure Detection

BFD-RS	BFD Reference Signal

BLER	Block Error Rate

BM-RS	Beam Management Reference Signal

BWP	Bandwidth Part

CA	Carrier Aggregation

CBD	Candidate Beam Detection

CCA	Clear channel assessment

CC	Component Carrier

CORESET	Control Resource Set

CP	Cyclic Prefix

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell ID

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FR	Frequency Range

HARQ	Hybrid Automatic Repeat Request

HO	Handover

L1-RSRP	Layer 1 RSRP

LBT	Listen before talk

MAC	Medium Access Control

MCG	Master Cell Group

MG	Measurement Gap

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR	New Radio

NR-DC	NR-NR Dual Connectivity

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OTDOA	Observed Time Difference Of Arrival

PBCH	Physical Broadcast Channel

PCC	Primary Component Carrier

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PRACH	Physical RACH

PSCell	Primary SCell

PSS	Primary Synchronization Signal

pTAG	Primary Timing Advance Group

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-Location

RACH	Random Access Channel

RAT	Radio Access Technology

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMSI	Remaining Minimum System Information

RRC	Radio Resource Control

RRM	Radio Resource Management

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SA	Standalone operation mode

SCC	Secondary Component Carrier

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SFN	System Frame Number

SFTD	SFN and Frame Timing DifferenceSI	System Information

SIB	System Information Block

SMTC	SSB-based Measurement Timing configuration

SpCell	Special Cell

SRS	Sounding Reference Signal

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

sTAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TDD	Time Division Duplex

TTI	Transmission Time Interval

UE	User Equipment

### UL	Uplink

Change 2

### 3.5.2	NR operating bands in FR1

NR frequency bands grouping for FR1 is specified in Table 3.5.2-1.

Table 3.5.2-1: NR frequency band groups for FR1

| Group | NR FDD |  | NR TDD |  | NR SDL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Band group notation | Operating bands | Band group notation | Operating bands | Band group notation |  | Operating bands |
| A | NR_FDD_FR1_A | n1, n70, n744, n91, n92, n93, n94 | NR_TDD_FR1_A | n34, n38, n39, n40, n50, n51 | NR_SDL_FR1_A |  | n75, n76 |
| B | NR_FDD_FR1_B | n66, n743 | NR_TDD_FR1_B | - | NR_SDL_FR1_B | - |  |
| C | NR_FDD_FR1_C | - | NR_TDD_FR1_C | n771, n78, n79 | NR_SDL_FR1_C | - |  |
| D | NR_FDD_FR1_D | n28 | NR_TDD_FR1_D | n772 | NR_SDL_FR1_D | - |  |
| E | NR_FDD_FR1_E | n2, n5, n7 | NR_TDD_FR1_E | n41, n90 | NR_SDL_FR1_E | - |  |
| F | NR_FDD_FR1_F | - | NR_TDD_FR1_F | - | NR_SDL_FR1_F | - |  |
| G | NR_FDD_FR1_G | n3, n8, n12, n20, n71 | NR_TDD_FR1_G | - | NR_SDL_FR1_G | n29 |  |
| H | NR_FDD_FR1_H | n25 | NR_TDD_FR1_H | - | NR_SDL_FR1_H | - |  |
| I | NR_FDD_FR1_I | - | NR_TDD_FR1_I | n46 | NR_FDD_FR1_I | - |  |
| : Except 3.8 GHz to 4.2 GHz. : Only 3.8 GHz to 4.2 GHz. : Except 1475.9 MHz to 1510.9 MHz. : Only when the band is confined in 1475.9 MHz to 1510.9 MHz. NOTE 5: These bands are used only in NR carrier aggregation with other NR bands according to NR CA band combinations specified in TS 38.101-1 [18] and TS 38.101-3 [20]. |  |  |  |  |  |  |  |

Change 2

## 3.6	Applicability of requirements in this specification version

In this specification,

-	‘cell’, ‘PCell’, ‘PSCell’ and ‘SCell’ refer to NR cell, NR PCell, NR PSCell, and NR SCell,

-	E-UTRA cells are referred to as ‘E-UTRA cell’, ‘E-UTRA PCell’, ‘E-UTRA PSCell’, and ‘E-UTRA SCell’,

-	E-UTRA-NR dual connectivity where E-UTRA is the master is referred to as ‘E-UTRA-NR dual connectivity’ or ‘EN-DC’.

-	NR-NR dual connectivity which involves two gNB acting as Master gNB and Secondary gNB is referred to as “NR-NR dual connectivity” or “NR-DC”. NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2.

-	‘active serving cell’ refers to PCell, PSCell and activated SCells

-	unless explicitly stated, downlink related requirements do not apply when CCA is used on a serving or neighbouring frequency on the downlink.

-	unless explicitly stated, uplink related requirements do not apply when CCA is used on the uplink by the UE

For UE configured with supplementary UL, the requirements in clause 7.1 and 7.3 shall also apply to uplink transmissions on supplementary UL.

Change 3

### 3.6.7	Requirements for operating with CCA

#### 3.6.7.1 Applicability of requirements for monitoring of frequencies operating with CCA

In RRC Idle and RRC inactive state, the requirements in table 3.6.7.1-1 shall apply to UEs which support standalone operation on cells with CCA, prior to the reselection to a serving cell operating with CCA.

In RRC connected state, the requirements in table 3.6.7.1-2 shall apply to UEs which support standalone operation on cells with CCA, and UEs which support carrier aggregation of Scells operating with CCA, prior to the PCell handover to a serving cell operating with CCA, or the addition of an Scell operating with CCA

The requirements in this section are in addition to the other requirements which apply to UEs operating without CCA on uplink or downilink serving frequencies.

Table 3.6.7.1-1 : Monitoring Requirements for UEs in RRC idle and RRC inactive state prior to reselection to a serving cell operating with CCA

| 4.2A.2.4 | Measurements of inter-frequency NR cells when CCA is used on the target cell | The requirements apply for a target cell measurements on a carrier frequency with CCA, regardless of whether the camping cell is on a carrier frequency with or without CCA. |
| --- | --- | --- |
| 4.2A.2.6 | Maximum interruption in paging reception when CCA is used on the cell which the UE is reselecting towards | The requirements apply when the cell for which SI is being received is on a carrier frequency with CCA, regardless of whether the camping cell is using CCA or not |
| 4.2A.2.6 | General requirements | The requirements apply for a target cell measurements on a carrier frequency with CCA, regardless of whether the camping cell is on a carrier frequency with or without CCA. |
| 5 | SA: RRC_INACTIVE state mobility | See corresponding subsection in section 4 |

Table 3.6.7.1-2: Monitoring requirements for UEs in RRC conencted state,  which support standalone operation on cells with CCA, and UEs which support carrier aggregation of Scells operating with CCA, prior to the PCell handover to a serving cell operating with CCA, or the addition of an Scell operating with CCA

| 9.3A | NR inter-frequency measurements when CCA is used | Applicable to neighbour frequencies with CCA |
| --- | --- | --- |
| 9.3A.1 | Introduction | Applicable to neighbour frequencies with CCA |
| 9.3A.2 | Requirements applicability | Applicable to neighbour frequencies with CCA |
| 9.3A.3 | Number of cells and number of SSB | Applicable to neighbour frequencies with CCA |
| 9.3A.3.1 | Requirements for FR1 | Applicable to neighbour frequencies with CCA |
| 9.3A.4 | Inter frequency cell identification | Applicable to neighbour frequencies with CCA |
| 9.3A.5 | Inter frequency measurements | Applicable to neighbour frequencies with CCA |
| 9.3A.6 | NR Inter frequency measurements reporting requirements | Applicable to neighbour frequencies with CCA |
| 9.3A.6.1 | Periodic Reporting | Applicable to neighbour frequencies with CCA |
| 9.3A.6.2 | Event-triggered Periodic Reporting | Applicable to neighbour frequencies with CCA |
| 9.3A.6.3 | Event-triggered Reporting | Applicable to neighbour frequencies with CCA |
| 9.3A.7 | NR Inter frequency SFTD measurement requirements | Applicable to neighbour frequencies with CCA |
| 9.3A.7.1 | Introduction | Applicable to neighbour frequencies with CCA |
| 9.3A.7.2 | SFTD Measurement delay | Applicable to neighbour frequencies with CCA |
| 9.3A.7.3 | SFTD Measurement reporting delay | Applicable to neighbour frequencies with CCA |

### 3.6.7.2 Applicability of requirements prior to configuring a serving cell with CCA

In RRC connected state, the requirements in table 3.6.7.2-1 shall apply to UEs which support standalone operation on cells with CCA, prior to the handover,  reestablishment or redirection to a serving cell operating with CCA.

In RRC connected state, the requirements in table 3.6.7.2-2 shall apply to UEs which support carrier aggregation of Scells operating with CCA, prior to the addition of an Scell operating with CCA

The requirements in this section are in addition to the other requirements which apply to UEs operating without CCA on uplink or downilink serving frequencies.

Table 3.6.7.2-1: Requirements for UEs which support standalone operation on cells with CCA, prior to the handover,  reestablishment or redirection to a serving cell operating with CCA

| 6.1A | Handover when CCA is used at least in the target cell | Applicable for PCell handover when there is CCA used in target cell, regardless if CCA is used in the serving cell or not |
| --- | --- | --- |
| 6.1A.1 | NR Handover | Applicable for PCell handover when there is CCA used in target cell, regardless if CCA is used in the serving cell or not |
| 6.1A.1.1 | Introduction | Applicable for PCell handover when there is CCA used in target cell, regardless if CCA is used in the serving cell or not |
| 6.1A.1.2 | NR FR1 - NR FR1 Handover | Applicable for PCell handover when there is CCA used in target cell, regardless if CCA is used in the serving cell or not |
| 6.1A.1.2.1 | Handover delay | Applicable for PCell handover when there is CCA used in target cell, regardless if CCA is used in the serving cell or not |
| 6.1A.1.2.2 | Interruption time | Applicable for PCell handover when there is CCA used in target cell, regardless if CCA is used in the serving cell or not |
| 6.2.1A and subsections | RRC Re-establishment with CCA | Applicable when there is CCA used in the target cell regardless if CCA was used in the PCell prior to reestablishment |
| 6.2.1.2.1 | UE Re-establishment delay requirement | Applicable when there is no CCA used in the target cell regardless if CCA was used in the PCell prior to reestablishment |
| 6.2A.1.2.1 | UE Re-establishment delay requirement | Applicable when there is CCA used in the target cell regardless if CCA was used in the PCell prior to reestablishment |
| 6.2.3 and subsections | SA: RRC Connection Release with Redirection | Applicable when there is no CCA used in the target cell regardless if CCA was used in the PCell prior to redirection |
| 6.2.3A and subsections | RRC Connection Release with Redirection with CCA | Applicable when there is CCA used in the target cell regardless if CCA was used in the PCell prior to redirection |

Table 3.6.7.2-2: Requirements for UEs which support carrier aggregation of Scells operating with CCA, prior to the addition of an Scell operating with CCA

| 8.2.1.2.3 | Interruptions at SCell addition/release | Applicable to a CCA or non CCA SCell |
| --- | --- | --- |

### 3.6.7.3 Applicability of requirements after configuring a UE with a CCA serving cell

| 38.133 Clause | Title | Scenario A (Carrier aggregation between NR PCell without CCA and SCells (SCell may have both DL and UL, or DL-only.) | Scenario B : Dual connectivity between LTE PCell with FS1 or FS2 and NR PSCell and Scell(s) with CCA | Scenario C : Standalone operation with all serving frequencies using CCA |
| --- | --- | --- | --- | --- |
| 4.2.2.1 | UE measurement capability | Scenario applies in RRC connected | Scenario applies in RRC connected | Applicable (referred to from 4A.2.2.1) |
| 4.2.2.4 | Measurements of inter-frequency NR cells | Scenario applies in RRC connected | Scenario applies in RRC connected | The requirements apply for a target cell measurements on a carrier frequency with no CCA, regardless of whether the camping cell is on a carrier frequency with or without CCA. |
| 4.2.2.5 | Measurements of inter-RAT E-UTRAN cells | Scenario applies in RRC connected | Scenario applies in RRC connected | The requirements apply for the UE camping on a cell on a carrier frequency with no CCA |
| 4.2.2.6 | Maximum interruption in paging reception | Scenario applies in RRC connected | Scenario applies in RRC connected | The requirements apply when the cell for which SI is being received is on a carrier frequency with no CCA, regardless of whether the camping cell is using CCA or not |
| 4.2.2.7 | General requirements | Scenario applies in RRC connected | Scenario applies in RRC connected | The requirements apply for a target cell measurements on a carrier frequency with no CCA, regardless of whether the camping cell is on a carrier frequency with or without CCA. |
| 4.2A | Cell Re-selection when CCA is used | Scenario applies in RRC connected | Scenario applies in RRC connected | Applicable |
| 4.2A.2.1 | UE measurement capability (note: section is a reference to 4.2.2.1) | Scenario applies in RRC connected | Scenario applies in RRC connected | Applicable (referred to 4A.2.2.1) |
| 4.2A.2.2 | Measurement and evaluation when CCA is used on the serving cell | Scenario applies in RRC connected | Scenario applies in RRC connected | The requirements apply for the UE camping on a cell on a carrier frequency with CCA |
| 4.2A.2.3 | Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell | Scenario applies in RRC connected | Scenario applies in RRC connected | The requirements apply for a target cell measurements on a carrier frequency with CCA and the serving cell also has CCA |
| 4.2A.2.4 | Measurements of inter-frequency NR cells when CCA is used on the target cell | Scenario applies in RRC connected | Scenario applies in RRC connected | The requirements apply for a target cell measurements on a carrier frequency with CCA, regardless of whether the camping cell is on a carrier frequency with or without CCA. |
| 4.2A.2.5 | Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell | Scenario applies in RRC connected | Scenario applies in RRC connected | The requirements apply for the UE camping on a cell on a carrier frequency with CCA |
| 4.2A.2.6 | Maximum interruption in paging reception when CCA is used on the cell which the UE is reselecting towards | Scenario applies in RRC connected | Scenario applies in RRC connected | The requirements apply when the cell for which SI is being received is on a carrier frequency with CCA, regardless of whether the camping cell is using CCA or not |
| 4.2A.2.6 | General requirements | Scenario applies in RRC connected | Scenario applies in RRC connected | The requirements apply for a target cell measurements on a carrier frequency with CCA, regardless of whether the camping cell is on a carrier frequency with or without CCA. |
| 5 and subclauses | SA: RRC_INACTIVE state mobility | Scenario applies in RRC connected | Scenario applies in RRC connected | See corresponding subclause in section 4 |
|  | RRC_CONNECTED state mobility |  |  |  |
| 6.1 | Handover | Section contains requirements for legacy HO to non CCA cells | N/A | Applicable for PCell handover when there is no CCA used in target cell |
| 6.1.1 and subclauses | NR Handover | Section contains requirements for legacy HO to non CCA cells | N/A | Applicable for PCell handover when there is no CCA used in target cell |
| 6.1.2 and subclauses | NR Handover to other RATs | Section contains requirements for legacy HO to non CCA cells | N/A | The requirements are extended to include the case in which the serving cell is in a CCA frequency |
| 6.1A and subclauses | Handover when CCA is used at least in the target cell | N/A | N/A | Applicable for PCell handover when there is CCA used in target cell, regardless if CCA is used in the serving cell or not |
| 6.2 | RRC Connection Mobility Control |  |  | Applicable |
| 6.2.1 and subclauses | SA: RRC Re-establishment | Section contains requirements for legacy reestablishment to non CCA cells | N/A | Applicable when there is no CCA used in the target cell regardless if CCA was used in the PCell prior to reestablishment |
| 6.2.1A and subclauses | RRC Re-establishment with CCA | N/A | N/A | Applicable when there is CCA used in the target cell regardless if CCA was used in the PCell prior to reestablishment |
| 6.2.1.2.1 | UE Re-establishment delay requirement | Section contains requirements for legacy reestablishment to non CCA cells | N/A | Applicable when there is no CCA used in the target cell regardless if CCA was used in the PCell prior to reestablishment |
| 6.2A.1.2.1 | UE Re-establishment delay requirement | N/A | N/A | Applicable when there is CCA used in the target cell regardless if CCA was used in the PCell prior to reestablishment |
| 6.2.2A | Random access with CCA | N/A | Applicable to NR PSCell (with CCA) | Applicable to NR PCell (with CCA) |
| 6.2.2A.1 | Introduction | N/A | Applicable to NR PSCell (with CCA) | Applicable to NR PCell (with CCA) |
| 6.2 2.A.2 | Requirements | N/A | Applicable to NR PSCell (with CCA) | Applicable to NR PCell (with CCA) |
| 6.2 2.A.2.1 | Contention based random access | N/A | Applicable to NR PSCell (with CCA) | Applicable to NR PCell (with CCA) |
| 6.2 2.A.2.2 | Non-Contention based random access | N/A | Applicable to NR PSCell (with CCA) | Applicable to NR PCell (with CCA) |
| 6.2 2.A..2.3 | UE behaviour when configured with supplementary UL | N/A | Applicable to NR PSCell (with CCA) | Applicable to NR PCell (with CCA) |
| 6.2.3 and subclauses | SA: RRC Connection Release with Redirection | Section contains requirements for legacy reestablishment to non CCA cells | N/A | Applicable when there is no CCA used in the target cell regardless if CCA was used in the PCell prior to redirection |
| 6.2.3A and subclauses | RRC Connection Release with Redirection with CCA | N/A |  | Applicable when there is CCA used in the target cell regardless if CCA was used in the PCell prior to redirection |
| 7 Timing |  |  |  |  |
| 7.1 | UE transmit timing | Applicable to NR PCell (no CCA) and uplink Scell(s) (with CCA) | Applicable to NR PSCell (with CCA) | Applicable |
| 7.2 | UE timer accuracy | Applicable to NR PCell (no CCA) and uplink Scell(s) (with CCA) | Applicable to NR PSCell (with CCA) | Applicable |
| 7.3 | Timing advance | Applicable to NR PCell (no CCA) and uplink Scell(s) (with CCA) | Applicable to NR PSCell (with CCA) | Applicable |
| 7.4 | Cell phase synchronization accuracy | Applicable to NR PCell (no CCA) and uplink Scell(s) (with CCA) | Applicable to NR PSCell (with CCA) | Applicable |
| 7.5 | Maximum Transmission Timing Difference | Applicable to NR PCell (no CCA) and uplink Scell(s) (with CCA) | Applicable | Applicable |
| 7.5.1 | Introduction | Applicable to NR PCell (no CCA) and uplink Scell(s) (with CCA) | Applicable | Applicable |
| 7.5.2 | Minimum Requirements for inter-band EN-DC | N/A | Applicable | N/A |
| 7.5.4 | Minimum Requirements for NR Carrier Aggregation | Applicable to NR PCell (no CCA) and uplink Scell (with CCA) | N/A | Applicable |
| 7.5.6 | Minimum Requirements for inter-band NR DC | N/A | N/A | N/A |
| 7.6 | Maximum Receive Timing Difference | Applicable to NR PCell (no CCA) and Scell(s) (with CCA) | Applicable | Applicable to NR PCell ( CCA) and Scell(s) (with CCA) |
| 7.6.1 | Introduction | Applicable to NR PCell (no CCA) and Scell(s) (with CCA) | Applicable | Applicable |
| 7.6.2 | Minimum Requirements for inter-band EN-DC | N/A | Applicable | N/A |
| 7.6.4 | Minimum Requirements for NR Carrier Aggregation | Applicable to NR PCell (no CCA) and Scell(s) (with CCA) | Applicable between PSCell and Scell(s) | Applicable to NR PCell ( CCA) and Scell(s) (with CCA) |
| 7.6.6 | Minimum Requirements for inter-band NR DC | N/A | N/A | N/A |
| 7.7 | deriveSSB-IndexFromCell tolerance | Applicable to NR PCell (no CCA) and Scell(s) (with CCA) | Applicable to NR PSCell and Scell(s) (all with CCA) | Applicable to NR PCell ( CCA) and Scell(s) (with CCA) |
| 8.1 and subclauses | Radio Link Monitoring | Applicable to NR PCell (no CCA) | N/A | N/A |
| 8.1A and subclauses | 8.1A Radio Link Monitoring with CCA on target frequency | N/A | Applicable to PSCell | Applicable to PCell |
| 8.2.1.1 and subclauses | EN-DC interruption : Introduction | N/A | Applicable | N/A |
| 8.2.2 and subclauses | SA: Interruptions with Standalone NR Carrier Aggregation | Applicable to victim NR PCell (no CCA) and Scell(s) (with CCA) | N/A | Applicable to victim NR PCell (with CCA) and Scell(s) (with CCA) |
| 8.2.4 and subclauses | NR-NR DC: Interruptions | N/A | N/A | N/A |
| 8.3A and subclauses | 8.3A Activation and Deactivation Delay of SCell operating with CCA | Applicable to NR SCell (uses CCA) | Applicable to NR SCell (uses CCA) | Applicable to NR SCell (uses CCA) |
| 8.5 and subclauses | 8.5 Link recovery procedures | Applicable to NR PCell (no CCA) | Applicable to NR PCell (no CCA) | N/A |
| 8.5A and subclauses | Link Recovery Procedures when CCA is used on target frequency | Applicable to NR SCell (with CCA) | Applicable to NR SCell (with CCA) | Applicable to NR PCell (with CCA) and SCells (with CCA) |
| 8.6 and subclauses | Active BWP switch delay | Applicable to NR PCell (no CCA) and Scell(s) (with CCA) | Applicable to NR PCell (no CCA) and Scell(s) (with CCA) | Applicable to NR PCell (with CCA) and SCells (with CCA) |
| 8.9 and subclauses | NR-DC: PSCell Addition and Release Delay | N/A | N/A | N/A |
| 8.10 and subclauses | Active TCI state switching delay | Applicable to NR PCell (no CCA) | Applicable | N/A |
| 8.10A and subclauses | Active TCI state switching delay when CCA is used on target frequency | Applicable to NR SCell (with CCA) | Applicable | Applicable |
| 8.11 | NR PSCell Change | N/A | Applicable | N/A |
|  | Measurement Procedure |  |  |  |
| 9.1 and subclauses | General measurement requirement | Applicable | Applicable | Applicable |
| 9.2 and subclauses | NR intra-frequency measurements | Applicable to PCC frequency (no CCA) | N/A | N/A |
| 9.2A and subclauses | NR intra-frequency measurements when CCA is used | Applicable to SCC frequency/frequencies (with CCA) | Applicable to PSCell frequency and SCC (both with CCA) | Applicable to PCell frequency and SCC (both with CCA) |
| 9.3 and subclauses | NR inter-frequency measurements | Applicable to neighbour frequencies without CCA | Applicable to neighbour frequencies without CCA | Applicable to neighbour frequencies without CCA |
| 9.3A and subclauses | NR inter-frequency measurements when CCA is used | Applicable to neighbour frequencies with CCA | Applicable to neighbour frequencies with CCA | Applicable to neighbour frequencies with CCA |
| 9.4 and subclauses | Inter-RAT measurements | Applicable | Applicable | Applicable |
| 9.5 to 9.5.3.3 | L1-RSRP measurements for Reporting | Applicable to PCell (no CCA) and SCells (with CCA) | Applicable to PSCell and SCells | Applicable |
| 9.5.4 and subclauses | L1-RSRP measurement requirements | Applicable to PCell (no CCA) | N/A | N/A |
| 9.5.4A and subclauses | L1-RSRP measurement requirements (with CCA on serving cell) | Applicable to PCell (no CCA) | N/A | N/A |
| 9.5.5 and subclauses | Measurement restriction for CSI-RS and SSB for L1-RSRP measurement | Applicable to PCell (no CCA) and SCells (with CCA) | Applicable to PSCell and SCells | Applicable to PCell and SCells |
| 9.5.6 and subclauses | Scheduling availability of UE during L1-RSRP measurement | Applicable to PCell (no CCA) and SCells (with CCA) | Applicable to PSCell and SCells | Applicable to PCell and SCells |
| 10 entire clause | Measurement Performance requirements | Applicable to frequencies with and without CCA | Applicable to frequencies with and without CCA | Applicable to frequencies with and without CCA |