3GPP TSG-RAN WG4 Meeting #94-e	R4-2001410

# Online,  24 February 2020 - 6 March 2020

Title: 					TP:Updates to DAPS handover requirements for LTE

Source: 				Ericsson

Agenda item:	7.12.1.2

Document for:	Approval

This is a text proposal for 36.133 to update requirements for LTE DAPS handover

Reason for change

Tbd and errors in specification

Summary of change

- Specify the power imbalance between source cell and target cell shall be within 6 dB

- Remove editors note : “FFS if the bandwidth of target cell is larger than the bandwidth of source cell for intra-frequency DAPS handover.”

- Add 1ms interruption in Dhandover2 if target cell is larger Bw than source

- Correct an incorrect reference from 38.331 to 36.331

Consequences if not approved:

DAPS handover specification remains incomplete

## 5.7	E-UTRAN DAPS Handover

### 5.7.1	Introduction

The purpose of DAPS handover is to change the PCell to another cell.

### 5.7.2	Requirements

#### 5.7.2.1	E-UTRAN FDD – FDD

The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers.

##### 5.7.2.1.1	DAPS Handover delay

Procedure delays for the procedure that can command a DAPS handover are specified in TS 36.331 [2]. DAPS delay is comprised of Dhandover1 and Dhandover2.

When the UE receives a RRC message implying handover, the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover1 seconds from the end of the last TTI containing the RRC command when UE is configured with dual active protocol stack handover.

Dhandover1 = TRRC_procedure+ Tsearch + TIU + 20 ms

Where

TRRC_procedure is the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2].

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

A cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause 8.1.2.2.1 for intra-frequency handover and Clause 8.1.2.3.1 for inter-frequency handover.

After successful RACH procedure of the target cell, when the UE receives a [TBD] message implying source cell release command, the UE shall accomplish the release actions specified in TS 36.331 [2] within Dhandover2.

Where:

TRRC_procedure is the maximum RRC procedure delay to be defined in clause 11.2 in TS 36.331 [2].

##### 5.7.2.1.2	Interruption time

During Dhandover1 the UE is allowed an interruption of up to Tinterrupt1 on source cell:

-	Tinterrupt1 is 1 ms for intra-frequency DAPS handover when the bandwidth of target cell is no larger than the bandwidth of source cell,

Note:	The power imbalance between source cell and target cell shall be within 6 dB.

-	Tinterrupt1 is 5 ms for intra-band inter-frequency DAPS handover, where the source cell and target cell are synchronous,

-	Tinterrupt1 is 1 ms for synchronous inter-band inter-frequency DAPS handover, and 2 ms for asynchronous inter-band inter-frequency DAPS handover.

During Dhandover2 the UE is allowed an interruption of up to Tinterrupt2 on target cell:

-	Tinterrupt2 is 2 ms for intra-frequency DAPS handover when the bandwidth of target cell is smaller than the bandwidth of source cell,

Note:	The power imbalance between source cell and target cell shall be within 6 dB.

-	Tinterrupt2 is 1 ms for intra-frequency DAPS handover when the bandwidth of target cell is equal to, or larger than the bandwidth of source cell,

Note:	The power imbalance between source cell and target cell shall be within 6 dB.

-	Tinterrupt2 is 5 ms for intra-band inter-frequency DAPS handover, where the source cell and target cell are synchronous,

-	Tinterrupt2 is 1 ms for synchronous inter-band inter-frequency DAPS handover, and 2ms for asynchronous inter-band inter-frequency DAPS handover.

#### 5.7.2.2	E-UTRAN FDD – TDD

The requirements in this clause are applicable to DAPS handover from FDD to TDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.7.2.1 apply for this section.

#### 5.7.2.3	E-UTRAN TDD – FDD

The requirements in this clause are applicable to DAPS handover from TDD to FDD. The requirements in this clause shall apply to UE supporting FDD and TDD.

The requirements in clause 5.7.2.1 apply for this section.

#### 5.7.2.4	E-UTRAN TDD – TDD

The requirements in this clause are applicable to DAPS handover from TDD to TDD. The requirements in this clause shall apply to UE supporting TDD.

The requirements in clause 5.7.2.1 apply for this section.