3GPP TSG-RAN WG4 Meeting #94-e	R4-2001428

Online, 24th February – 6th March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0257 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-1: Time mask requirements for switching between 1Tx and 2Tx transmissions for inter-band UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No requirements are defined for UE Tx switching between case 1 and case 2 transmissions for inter-band UL CA |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE time mask requirements for switching and transient periods are defined based on the following agreed RAN4 documents: R4-1916083 LS on UE capability and RRC signalling for Tx switching R4-1916084 WF on Tx switching between case 1 and 2 and the agreement captured in the RAN4#93 chairman’s minutes: Power class declaration will NOT be changed between case 1 and case 2. Rel-16 power class singling will be followed for Tx switching between case 1 and case 2. Terminology for single-layer transmission and 2-layer UL-MIMO transmission follows TS38.211. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE Tx switching between case 1 and case 2 transmissions is not allowed by TS38.101-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.4, 6.3C, 6.3D.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR ... CR ... TS38.306, TS38.331 |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.3A.3	Transmit ON/OFF time mask for CA

#### 6.3A.3.1	Void

#### 6.3A.3.2	Void

#### 6.3A.3.3	Transmit ON/OFF time mask for inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the general output power ON/OFF time mask specified in clause 6.3.3.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in clause 6.3.3.1 shall only be applicable for each component carrier when all the component carriers are OFF.

#### 6.3A.3.4 Time mask for switching between single-layer and two-layer transmissions for inter-band UL CA

In addition to the requirements in clause 6.3A.3.3, the time mask requirements in this clause apply for the UE indicating support for the switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on two uplink carriers configured in different NR bands.

The time mask for switching between single-layer transmission with one antenna port on two uplink carriers configured in different NR bands and two-layer transmission with two antenna ports on one uplink carrier defines the switching period and transient period allowed between single-layer transmission and two-layer transmission on different NR bands. The RRC signalling TBD defines on which uplink carrier the switching period is located as shown in Figure 6.3A.3.4-1.  The switching period shall be less than TBD us and the transient period is less than 10 us.

The UE indicating support for the switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on two uplink carriers configured in different NR bands shall support both two-layer and single-layer transmissions on the uplink carrier with two-layer transmission. The UE indicating support for the switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on two uplink carriers configured in different NR bands shall be able to transmit simultaneously single-layer transmission on both of the two uplink carriers configured in different NR bands without any switching period or transient period as shown in Figure 6.3A.3.4-2.

The UE shall follow the BS scheduling and rank adaptation for switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers configured in different NR bands and for transmitting simultaneously with single-layer transmission with one antenna port on both of the two uplink carriers configured in different NR bands.

The UE power class shall be the same for the simultaneous single-layer transmissions with one antenna port on both of the uplink carriers configured on different NR bands and when switching between single-layer transmission with one antenna port and two-layer transmission with two antenna port between the two uplink carriers configured on different NR bands.

Figure 6.3A.3.4-1: Time mask for switching between single-layer transmissions with one antenna port on two uplink carriers and two-layer transmission with two ports on one of the two uplink carriers configured in different NR bands

Figure 6.3A.3.4-2: Simultaneous single-layer transmission with one antenna port on two uplink carriers configured in different NR bands without switching and transient periods

### 6.3A.4	Power control for CA

#### 6.3A.4.1	Void

#### 6.3A.4.2	Void

#### 6.3A.4.3	Power control for inter-band CA

No requirements unique to CA operation are defined.

## 6.3C	Output power dynamics for SUL

### 6.3C.1	Void

### 6.3C.2	Void

### 6.3C.3	Transmit ON/OFF time mask for SUL

#### 6.3C.3.1	Void

#### 6.3C.3.2	Void

#### 6.3C.3.3	Void

#### 6.3C.3.4 Time mask for switching between single-layer and two-layer for SUL

The time mask requirements in this clause apply for the UE indicating support for the switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on two uplink carriers configured in different NR bands including one SUL band.

The time mask for switching between single-layer transmission with one antenna port on two uplink carriers configured in NR band and NR SUL band and two-layer transmission with two antenna ports on NR uplink carrier defines the switching period and transient period allowed between single-layer transmission and two-layer transmission on different NR bands. The RRC signalling TBD defines on which uplink carrier the switching period is located as shown in Figure 6.3C.3.4-1.  The switching period shall be less than TBD us and the transient period is less than 10 us.

The UE indicating support for the switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on two uplink carriers configured in different NR bands shall support both two-layer and single-layer transmissions on the uplink carrier with two-layer transmission. The UE indicating support for the switching between single-layer transmission with one antenna port on SUL carrier and two-layer transmission with two antenna ports on NR uplink carrier shall support switching of single-layer with one antenna port between SUL and NR uplink crrier without any switching period and transient period as defined in Table 5.2C-1.

The UE shall follow the BS scheduling and rank adaptation for switching between single-layer transmission with one antenna port on SUL carrier and two-layer transmission with two antenna ports on NR uplink carrier and for switching between single-layer transmission with one antenna port on SUL carrier and single-layer transmission with one antenna port on NR uplink carrier.

The UE power class shall be the same for the single-layer transmissions with one antenna port on NR uplink carrier and SUL carrier and when switching between single-layer transmission with one antenna port and two-layer transmission with two antenna port between SUL carrier and NR uplink carrier.

Figure 6.3A.3.4-1: Time mask for switching between single-layer transmissions with one antenna port on SUL carrier and two-layer transmission with two antenna ports on NR uplink carrier

## Figure 6.3A.3.4-2: Switching between single-layer transmission with one antenna port on SUL carrier and single-layer transmission with one antenna port on NR uplink carrier without switching and transient periods

## 6.3D	Output power dynamics for UL MIMO

### 6.3D.1	Minimum output power for UL MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the minimum output power is defined as the sum of the mean power at each transmit connector in one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3.1-1.

If UE is configured for transmission on single-antenna port, the requirements in clause 6.3.1 apply

### 6.3D.2	Transmit OFF power for UL MIMO

The transmit OFF power is defined as the mean power at each transmit antenna connector in a duration of at least one sub-frame (1 ms) excluding any transient periods.

The transmit OFF power at each transmit antenna connector shall not exceed the values specified in Table 6.3.2-1.

### 6.3D.3	Transmit ON/OFF time mask for UL MIMO

For UE supporting UL MIMO, the ON/OFF time mask requirements in clause 6.3.3 apply at each transmit antenna connector. Additionally, for UE supporting switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on two uplink carriers configured in different NR bands without SUL band the time mask requirements in clause 6.3A.3.4 apply. Additionally, for UE supporting switching between single-layer transmission with one antenna port on SUL carrier and two-layer transmission with two antenna ports on NR uplink carrier the time mask requirements in clause 6.3A.3.4 apply.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the general ON/OFF time mask requirements specified in clause 6.3.3.1 apply to each transmit antenna connector. The requirements shall be met with the UL MIMO configurations described in clause 6.2D.1.

If UE is configured for transmission on single-antenna port, the requirements in clause 6.3.3 apply.

### 6.3D.4	Power control for UL MIMO

For UE supporting UL MIMO, the power control tolerance applies to the sum of output power at each transmit antenna connector.

The power control requirements specified in clause 6.3.4 apply to UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme. The requirements shall be met with UL MIMO configurations described in clause 6.2D.1.

If UE is configured for transmission on single-antenna port, the requirements in clause 6.3.4 apply