3GPP TSG-RAN WG4 Meeting #94-e	R4-2001429

Online, 24th February – 6th March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0205 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-3: Time mask requirements for switching between 1Tx and 2Tx transmissions for inter-band EN-DC without SUL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No requirements are defined for UE Tx switching between case 1 and case 2 transmissions for inter-band EN-DC without SUL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE time mask requirements for switching and transient periods are defined based on the following agreed RAN4 documents: R4-1916083 LS on UE capability and RRC signalling for Tx switching R4-1916084 WF on Tx switching between case 1 and 2 and the agreement captured in the RAN4#93 chairman’s minutes: Power class declaration will NOT be changed between case 1 and case 2. Rel-16 power class singling will be followed for Tx switching between case 1 and case 2. Terminology for single-layer transmission and 2-layer UL-MIMO transmission follows TS38.211. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE Tx switching between case 1 and case 2 transmissions is not allowed for inter-band EN-DC without SUL by TS38.101-3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3B.0, 6.3B.4, 6.3B.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR ... CR ... TS38.306, TS38.331 |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.3	Output power dynamics

Output power dynamics for EN-DC operations in FR1 and FR2 as specified in TS 38.101-1 [2] and TS 38.101-2 [3], respectively. E-UTRA as specified in TS 36.101 [4]. For intra-band contiguous EN-DC operation in FR1, minimum output power requirements specified in clause 6.3.1 of TS 38.101-1 [2] and clause 6.3.2 of TS 36.101 [4] shall only apply when the power of all NR and E-UTRA carriers are set to minimum value. Similarly, OFF power requirements specified in clause 6.3.2 of TS 38.101-1 [2] and clause 6.3.3 of TS 36.101 [4] shall only apply when the power of all NR and E-UTRA carriers are OFF. The OFF power condition in transmit ON/OFF time mask requirements specified in clause 6.3.3 of TS 38.101-1 [2] and clause 6.3.4 of TS 36.101 [4] is applicable only when all NR and E-UTRA carriers are OFF. If both E-UTRA and NR transition between ON and OFF states simultaneously, the longer transient time shall apply to both. If either E-UTRA or NR is OFF and the other carrier transitions from OFF to ON, then the transiet time associated with that carrier applies.

## 6.3A	Output power dynamics for CA

For inter-band NR CA between FR1 and FR2, output power dynamics as specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for FR1 and FR2 respectively.

6.3B	Output power dynamics for DC

### 6.3B.0	General

The E-UTRA and NR switching time mask defines the observation period between E-UTRA subframe and NR slot/mini-slot boundary. Both E-UTRA subframe and NR slot/mini-slot have ON power transmissions. The ON power is defined as the mean power over the symbol duration excluding any transient period. For E-UTRA subframe or NR slot/mini-slot having OFF power transmission, the general time mask for E-UTRA or NR shall apply.

For inter-band EN-DC, output power dynamics requirement for E-UTRA single carrier and CA operation specified in clauses 6.3 of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.3 of TS 38.101-1 [2] and clause 6.3, 6.3A and 6.3D of TS 38.101-2 [3] apply. For inter-band EN-DC without SUL and UE supporting switching between single-layer transmission with one antenna port on E-UTRA uplink carrier and two-layer transmission with two antenna ports NR uplink carrier the time mask requirements of clause 6.3B.4.1 apply.

### 6.3B.1	Output power dynamics for EN-DC with UL sharing from UE perspective

#### 6.3B.1.1	E-UTRA and NR switching time mask for TDM based UL sharing from UE perspective

The E-UTRA and NR switching time mask is applicable for non-simultaneous transmissions between E-UTRA and NR in TDM based UL sharing from the UE perspective in the same channel, which is shared by E-UTRA and NR.

For UEs reporting E-UTRA and NR switching time capability of type 1 with switching time < 0.5 us for TDM based UL sharing from UE perspective within FR1 time masks in Figure 6.3B.1.1-1 and Figure 6.3B.1.1-2 shall apply. For UEs reporting E-UTRA and NR switching time capability of type 2 with switching time < 20 us for TDM based UL sharing from UE perspective within FR1, time masks in Figure 6.3B.1.1-3 and Figure 6.3B.1.1-4 shall apply.

Figure 6.3B.1.1-1: E-UTRA to NR switching time mask for type 1 for TDM based UL sharing from UE perspective within FR1

Figure 6.3B.1.1-2: NR to E-UTRA switching time mask for type 1 for TDM based UL sharing from UE perspective within FR1

Figure 6.3B.1.1-3: E-UTRA to NR switching time mask for type 2 for TDM based UL sharing from UE perspective within FR1

Figure 6.3B.1.1-4: NR to E-UTRA switching time mask for type 2 for TDM based UL sharing from UE perspective within FR1

### 6.3B.1a	Output power dynamics for NE-DC with UL sharing from UE perspective

The E-UTRA and NR switching time mask is applicable for non-simultaneous transmissions between E-UTRA and NR in TDM based UL sharing from the UE perspective in the same channel, which is shared by E-UTRA and NR. Unless otherwise specified, the 6.3B.1.1 clauses for NE-DC are applicable.

### 6.3B.2	Output power dynamics for intra-band EN-DC without dual PA capability

For both non-contiguous intra-band EN-DC and contiguous EN-DC for DC_(n)41 and DC_(n)71 without dual PA capability, maximum UL switching time is defined as 120 us and DL reception interruption is allowed during UL switching. Time masks in Figure 6.3B.2-1 and Figure 6.3B.2-2 shall apply. Unless otherwise stated, for intra-band contiguous EN-DC of other band combinations the switching time in 6.3B.1.1 applies.

Figure 6.3B.2-1: E-UTRA to NR switching time mask for intra-band EN-DC without dual PA capabilitywhen single UL is allowed

Figure 6.3B.2-2: NR to E-UTRA switching time mask for intra-band EN-DC without dual PA capabilitywhen single UL is allowed

### 6.3B.3	Output power dynamics for intra-band EN-DC with dual PA capability

For both contiguous and non-contiguous intra-band EN-DC with dual PA capability, time masks in Figure 6.3B.3-1 and Figure 6.3B.3-2 shall apply.

Figure 6.3B.3-1: E-UTRA to NR switching time mask for intra-band EN-DC with dual PA capability

Figure 6.3B.3-2: NR to E-UTRA switching time mask for intra-band EN-DC with dual PA capability

### 6.3B.4	Output power dynamics for inter-band EN-DC without SUL

#### 6.3B.4.1	Time mask for switching between single-layer and two-layer transmissions for inter-band EN-DC without SUL

The time mask requirements in this clause apply for the UE indicating support of the switching between single-layer transmission with one antenna port on E-UTRA uplink carrier and two-layer transmission with two antenna ports on NR uplink carriers for inter-band EN-DC without SUL.

The time mask for switching between single-layer transmission with one antenna port on E-UTRA and NR uplink carriers configured in different bands and two-layer transmission with two antenna ports on NR uplink carrier defines the switching period and transient period allowed between single-layer transmission and two-layer transmissions on E-UTRA and NR bands. The switching period, which is located on NR uplink carrier, shall be less than TBD us as shown in Figure 6.3B.4.1-1. The transient period on NR uplink carrier shall be less than 10 us and the transient period on E-UTRA uplink carrier less than 20 us as shown in Figure 6.3B.4.1-1.

The UE indicating support for the switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on E-UTRA and NR uplink carriers configured in different bands shall support both two-layer and single-layer transmissions on the NR uplink carrier. The UE indicating support for the switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on E-UTRA and NR uplink carriers configured in different bands shall be able to transmit simultaneously single-layer transmission on E-UTRA and NR uplink carriers configured in NR bands without any switching period or transient period as shown in Figure 6.3B.4.1-2.

The UE shall follow the BS scheduling and rank adaptation for switching between single-layer transmission with one antenna port and two-layer transmission with two antenna ports on the E-UTRA and NR uplink carriers configured in different bands and transmitting simultaneously with single-layer transmission with one antenna port on E-UTRA and NR uplink carriers configured in different bands.

The UE power class shall be the same for the simultaneous single-layer transmissions with one antenna port on E-UTRA and NR uplink carriers configured on different bands and when switching between single-layer transmission with one antenna port and two-layer transmission with two antenna port between the E-UTRA and NR uplink carriers configured on different bands.

Figure 6.3B.4.1-1: Time mask for switching between single-layer transmissions with one antenna port on E-UTRA and NR uplink carriers and two-layer transmission with two ports on NR uplink carrier in inter-band EN-DC without SUL

## Figure 6.3B.4.1-2: Simultaneous single-layer transmission with one port on E-UTRA uplink carrier and NR uplink carrier without switching and transient periods in inter-band EN-DC

## 6.4	Void