3GPP TSG-RAN WG4 Meeting #94-e	R4-2001452

# Electronic Meeting, 24th Feb - 6th Mar, 2020

Title: 	Simulation results for LTE HST BS demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.13.4.1

Document for:	Discussion

Background

During RAN4#93 meeting, the following agreement is achieved on LTE Rel-16 BS demodulation.

Agreement Define requirements for maximum Doppler shift of 1944Hz and 1750Hz for both BS enhanced HST open space and tunnel scenarios. Define applicability rules based on BS declaration (i.e. BS manufacturer can choose one of the two test cases during conformance testing)

However, the simulation results provided by companies is not aligned. In this contribution, we resubmit our simulation results about the BS demodulation requirements for LTE Rel-16 HST.

Simulations

The following Table 3.1-1 shows our simulation results based on the simulation assumption in WF [1].

Table 3.1-1: Ideal Simulation results for LTE PUSCH under HST single-tap

| CBW [MHz] | CP | FRC | Antenna configuration | Propagation conditions | Fraction of maximum throughput | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 1944Hz | 1750Hz |
| 1.4 | Normal | A3-2 | 1x1 | HST Scenario 3 | 30% | -4.97 | -5.16 |
|  |  |  |  |  | 70% | -0.85 | -0.85 |
|  |  |  | 1x2 | HST Scenario 1 Low | 30% | -7.21 | -7.11 |
|  |  |  |  |  | 70% | -3.86 | -3.86 |
| 3 | Normal | A3-3 | 1x1 | HST Scenario 3 | 30% | -5.11 | -5.07 |
|  |  |  |  |  | 70% | -0.79 | -0.80 |
|  |  |  | 1x2 | HST Scenario 1 Low | 30% | -7.80 | -7.80 |
|  |  |  |  |  | 70% | -3.85 | -3.85 |
| 5 | Normal | A3-4 | 1x1 | HST Scenario 3 | 30% | -6.16 | -6.22 |
|  |  |  |  |  | 70% | -1.57 | -1.57 |
|  |  |  | 1x2 | HST Scenario 1 Low | 30% | -9.09 | -9.16 |
|  |  |  |  |  | 70% | -4.61 | -4.61 |
| 10 | Normal | A3-5 | 1x1 | HST Scenario 3 | 30% | -5.29 | -5.30 |
|  |  |  |  |  | 70% | -0.89 | -0.87 |
|  |  |  | 1x2 | HST Scenario 1 Low | 30% | -8.32 | -8.24 |
|  |  |  |  |  | 70% | -3.97 | -3.98 |
| 15 | Normal | A3-6 | 1x1 | HST Scenario 3 | 30% | -6.24 | -6.24 |
|  |  |  |  |  | 70% | -1.55 | -1.56 |
|  |  |  | 1x2 | HST Scenario 1 Low | 30% | -9.20 | -9.26 |
|  |  |  |  |  | 70% | -4.60 | -4.60 |
| 20 | Normal | A3-7 | 1x1 | HST Scenario 3 | 30% | -5.27 | -5.27 |
|  |  |  |  |  | 70% | -0.67 | -0.67 |
|  |  |  | 1x2 | HST Scenario 1 Low | 30% | -8.40 | -8.37 |
|  |  |  |  |  | 70% | -3.74 | -3.74 |

Conclusions

In this contribution, we resubmit our simulation results about the BS demodulation requirements for LTE Rel-16 HST for alignment.

Reference

- R4-1912777, Way forward on LTE HST BS demodulation requirements, RAN4#92Bis, NTT DOCOMO, INC