3GPP TSG-RAN WG4 Meeting #94-e	R4-2001456

# Electronic Meeting, 24th Feb - 6th Mar, 2020

Title: 	Discussion and simulation results on NR UE HST performance requirements under multi-path fading channel

Source: 	Huawei, HiSilicon

Agenda item:	8.17.2.1.4

Document for:	Discussion

Background

During RAN4#93 meeting, way forward [1] for NR Rel-16 UE demodulation was approved. In this contribution, we share our views about the UE demodulation requirements for NR Rel-16 HST under multi-path fading channel.

Discussion

MCS and rank

- MCS - Option 1: MCS4 - Option 2: MCS13 - Option 3: MCS17 - Note: MCS should be discussed together with rank assumption - Rank - Option 1: Rank = 1 - Option 2: Rank = 2 - Note: rank1 is mandatory, rank2 is mandatory with capability signalling

We provide our updated simulations as per the simulation assumptions in [1] for multi-path fading channel as following:

Table 3-1: Ideal Simulation results for NR PDSCH under multi-path fading channel

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | Rank | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 1 | 4 | -2.08 |
| 2 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 2 | 4 | 2.10 |
| 3 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 1 | 13 | 6.56 |
| 4 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 2 | 13 | N/A |
| 5 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 1 | 17 | 11.19 |
| 6 | 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 2 | 17 | N/A |
| 7 | 10MHz/15kHz | 600 | 2Tx 4Rx ULA Low | 1 | 4 | -5.61 |
| 8 | 10MHz/15kHz | 600 | 2Tx 4Rx ULA Low | 2 | 4 | -1.92 |
| 9 | 10MHz/15kHz | 600 | 2Tx 4Rx ULA Low | 1 | 13 | 2.89 |
| 10 | 10MHz/15kHz | 600 | 2Tx 4Rx ULA Low | 2 | 13 | 7.62 |
| 11 | 10MHz/15kHz | 600 | 2Tx 4Rx ULA Low | 1 | 17 | 7.00 |
| 12 | 10MHz/15kHz | 600 | 2Tx 4Rx ULA Low | 2 | 17 | 13.54 |
| 13 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 1 | 4 | -2.41 |
| 14 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 2 | 4 | 2.94 |
| 15 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 1 | 13 | 7.33 |
| 16 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 2 | 13 | N/A |
| 17 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 1 | 17 | 13.66 |
| 18 | 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 2 | 17 | N/A |
| 19 | 40MHz/30kHz | 1200 | 2Tx 4Rx ULA Low | 1 | 4 | -5.52 |
| 20 | 40MHz/30kHz | 1200 | 2Tx 4Rx ULA Low | 2 | 4 | -1.39 |
| 21 | 40MHz/30kHz | 1200 | 2Tx 4Rx ULA Low | 1 | 13 | 3.33 |
| 22 | 40MHz/30kHz | 1200 | 2Tx 4Rx ULA Low | 2 | 13 | 8.57 |
| 23 | 40MHz/30kHz | 1200 | 2Tx 4Rx ULA Low | 1 | 17 | 7.82 |
| 24 | 40MHz/30kHz | 1200 | 2Tx 4Rx ULA Low | 2 | 17 | 15.91 |

Figure 3-1: Ideal Simulation results for NR PDSCH under multi-path fading channel

As per simulation results, we can get the following observation and proposal:

Observation 1: For rank 1, it is not feasible for MCS 17 for 2Tx2Rx since it cannot achieve 70% maximum throughput. Observation 2: For rank 2,

- It is not feasible for MCS 13, 17 for 2Tx2Rx since it cannot achieve 70% maximum throughput

- It is not feasible for MCS 17 for 2Tx4Rx since it cannot achieve maximum throughput.

As per above observations, we can get that MCS 17 is not feasible for both rank 1 and rank 2. Considering rank 2 is mandatory with capability signalling, we think rank 1 is more suitable comparing to rank 2. For MCS 4 and MCS 13 for rank 1, we prefer MCS 13 as it can achieve higher throughput.

Proposal 1: Adopt MCS 13 and rank 1 for multi-path fading channel performance requirements.

Proposals

In this contribution, we discuss and provide our simulation results on NR UE HST performance requirements under multi-path fading channel model. Our observations and proposals are:

Observation 1: For rank 1, it is not feasible for MCS 17 for 2Tx2Rx since it cannot achieve 70% maximum throughput. Observation 2: For rank 2,

- It is not feasible for MCS 13, 17 for 2Tx2Rx since it cannot achieve 70% maximum throughput

- It is not feasible for MCS 17 for 2Tx4Rx since it cannot achieve maximum throughput.

Proposal 1: Adopt MCS 13 and rank 1 for multi-path fading channel performance requirements.

Reference

- R4-1915926, WF on UE demodulation for NR HST, RAN4#93, CMCC

- Annex Simulation assumption

- Simulation assumption for multi-path fading channel Parameter; Value. FDD 15KHz SCS; TDD 30KHz SCS. Antenna configuration; 2×2; 2×4. DMRS type; type 1. Number of DMRS symbols; DMRS 1+1+1. TDD pattern; 7D1S2U, S: 6D 4G 4U. MCS; MCS 4; MCS 13; MCS 17 based on 64QAM table. Propagation condition; TDL-C 300ns. TRS periodicity; 10ms, 2slot pattern. PDSCH mapping; Type A, Start symbol 2, Duration 12. Rank; Rank = 1 and 2. BW; 10MHz; 40MHz. Maximum Doppler shift; 600Hz; 1200Hz. Testing metric; SNR @70% of maximum throughput.