3GPP TSG-RAN WG4 Meeting #94-e	R4-2001458

# Electronic Meeting, 24th Feb - 6th Mar, 2020

Title: 	Discussion on NR HST PUSCH performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.17.2.2.1

Document for:	Discussion

Background

During RAN4#93 meeting, way forward [1] for NR Rel-16 BS demodulation was approved. In this contribution, we share our views about the BS demodulation requirements for NR Rel-16 HST.

Discussion

l0 for PUSCH mapping type A

- l0 for PUSCH mapping type A - l0 = 2 (For simulation alignment) - If no performance different between l0 = 2 and l0 = 3, define performance requirements based on l0 = 2 - l0 value for testing is based on BS declaration

For both l0=2 and 3, the minimum distance between two columns of adjacent DMRS sequences is 4 symbols, i.e. same capability of frequency tracking using baseline method. So there is almost no performance difference between l0=2 and 3 for PUSCH mapping type A in theory. Our simulation results in [2] also support the point. So it is reasonable to define one set of performance requirements for l0 = 2 and 3, and l0 value for testing is based on BS declaration.

Observation 1: There is almost no performance difference between l0=2 and 3 for PUSCH mapping type A.

Proposal 1: Define one set of performance requirements for l0 = 2 and 3.

Antenna configuration

- Antenna configuration: - Tunnel scenario: 1x2, FFS 1x1 - Test setup for conducted test with 1x1 needs further discussion - FFS on OTA test for 1x1 - Whether to introduce test for 1x1 needs to consider the testable SNR value

For antenna configuration in tunnel scenario, RAN 4 has defined 1x2 antenna configuration as it is more practical and popular deployment mode. Since there is only 'single' tap which is belongs to AWGN model, two receive antennas receive same signals, and it is expected that the performance of antenna configuration 1x2 is about 3dB better than that of antenna configuration 1x1. That is to say, performance of antenna configuration 1x1 can be inferred from that of antenna configuration 1x2. So it is enough to define antenna configuration 1x2 and no need to consider antenna configuration 1x1 for tunnel scenario.

Proposal 2: Do not consider antenna configuration 1x1 for tunnel scenario.

MCS

- MCS - 350km/h - Open space scenario: MCS 2 and MCS 16 - Tunnel scenario: MCS 2 and MCS 16 - 500km/h - Open space scenario: MCS 2 and MCS 16 - Tunnel scenario: MCS 2, FFS on MCS 16

At last meeting, some companies worry about poor performance for MCS 16 in tunnel scenario. However, as per our simulation results in [2], MCS 16 has a good performance in tunnel.

Observation 2: MCS 16 has a good performance in tunnel scenario for 500 km/h.

Proposal 3: Define MCS 16 in tunnel scenario for 500 km/h.

Proposals

In this contribution, we analyses the BS performance requirement for NR HST, and our conclusions and proposals are:

Observation 1: There is almost no performance difference between l0=2 and 3 for PUSCH mapping type A.

Observation 2: MCS 16 has a good performance in tunnel scenario for 500 km/h.

Proposal 1: Define one set of performance requirements for l0 = 2 and 3.

Proposal 2: Do not consider antenna configuration 1x1 for tunnel scenario.

Proposal 3: Define MCS 16 in tunnel scenario for 500 km/h.

Reference

- R4-1915886, Way forward on NR HST PUSCH demodulation requirements, RAN4#93, Huawei, HiSilicon

- R4-2001459, Simulation results on NR HST PUSCH performance requirements, RAN#94e, Huawei, HiSilicon