3GPP TSG-RAN WG4 Meeting #94-e	R4-2001459

# Electronic Meeting, 24th Feb - 6th Mar, 2020

Title: 	Simulation results on NR HST PUSCH performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.17.2.2.1

Document for:	Discussion

Background

During RAN4#93 meeting, updated simulation assumption for NR HST PUSCH was approved, and the simulation assumption is extracted as shown below:

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | v = 350km/h |  | v = 500km/h |  |
| Transform precoding | Disabled |  | Disabled |  |
| Number of Tx | 1 |  | 1 |  |
| Number of Rx | Tunnel: 2, FFS on 1 Open space: 2, 8 |  | Tunnel: 2, FFS on 1 Open space: 2, 8 |  |
| Number of layers | 1 |  | 1 |  |
| SCS | 15kHz, 30kHz |  | 15kHz, 30kHz |  |
| Reference signal | DMRS type 1 with 1+1+1 |  | DMRS type 1 with 1+1+1 |  |
| l0 | l0 = 2 (for simulation alignment) (Note 1) |  | l0 = 2 (for simulation alignment) (Note 1) |  |
| symbols length | 14 |  | 14 |  |
| start symbol index | 0 |  | 0 |  |
| Time domain resource allocation type | type A |  | type A |  |
| Frequency domain resource | Full applicable test bandwidth |  | Full applicable test bandwidth |  |
| MCS index | Tunnel: MCS 2 and MCS 16 Open space: MCS 2 and MCS 16 |  | Tunnel: MCS 2, FFS on MCS 16 Open space: MCS 2 and MCS 16 |  |
| Maximum Doppler shift | 15kHz SCS: 1340Hz | 30kHz SCS: 2334Hz | 15kHz SCS: 1740Hz | 30kHz SCS: 3334Hz |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |  | 15kHz SCS: 1.8GHz (Band n3) 30kHz SCS: 3.6GHz |  |
| Propagation condition | HST single-tap channel model: Tunnel: Ds=300m, Dmin=2m Open space: Ds=700m, Dmin=150m |  |  |  |
| SCS and BW | 15kHz: 10MHz; 30kHz: 40MHz |  |  |  |
| Timing offset | 0 |  | 0 |  |
| Frequency offset | 0 |  | 0 |  |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |  | Disabled |  |
| Number of HARQ transmissions | 4 |  | 4 |  |
| Testing metric | SNR @70% of maximum throughput |  | SNR @70% of maximum throughput |  |
| Note 1: Company is encouraged to evaluate the performance difference between l0 = 2 and l0 = 3 |  |  |  |  |

In this contribution, we provide our simulation results on NR HST PUSCH performance requirement for alignment.

Simulations

For 350km/h

Table 2.1-1: Ideal Simulation results for 350km/h for NR PUSCH

| Case Number | l0 position | Antenna configuration | CHBW/SCS | maximum Doppler shift(Hz) | Propagation condition | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 1x2 | 10MHz/15kHz | 1340 | Tunnel | 2 | -6.60 |
| 2 | 3 | 1x2 | 10MHz/15kHz | 1340 | Tunnel | 2 | -6.60 |
| 3 | 2 | 1x2 | 10MHz/15kHz | 1340 | Tunnel | 16 | 5.41 |
| 4 | 3 | 1x2 | 10MHz/15kHz | 1340 | Tunnel | 16 | 5.41 |
| 5 | 2 | 1x2 | 10MHz/15kHz | 1340 | Open space | 2 | -6.60 |
| 6 | 3 | 1x2 | 10MHz/15kHz | 1340 | Open space | 2 | -6.60 |
| 7 | 2 | 1x2 | 10MHz/15kHz | 1340 | Open space | 16 | 5.40 |
| 8 | 3 | 1x2 | 10MHz/15kHz | 1340 | Open space | 16 | 5.40 |
| 9 | 2 | 1x8 | 10MHz/15kHz | 1340 | Open space | 2 | -12.57 |
| 10 | 3 | 1x8 | 10MHz/15kHz | 1340 | Open space | 2 | -12.57 |
| 11 | 2 | 1x8 | 10MHz/15kHz | 1340 | Open space | 16 | 0.40 |
| 12 | 3 | 1x8 | 10MHz/15kHz | 1340 | Open space | 16 | 0.40 |
| 13 | 2 | 1x2 | 40MHz/30kHz | 2334 | Tunnel | 2 | -6.59 |
| 14 | 3 | 1x2 | 40MHz/30kHz | 2334 | Tunnel | 2 | -6.59 |
| 15 | 2 | 1x2 | 40MHz/30kHz | 2334 | Tunnel | 16 | 5.41 |
| 16 | 3 | 1x2 | 40MHz/30kHz | 2334 | Tunnel | 16 | 5.42 |
| 17 | 2 | 1x2 | 40MHz/30kHz | 2334 | Open space | 2 | -6.67 |
| 18 | 3 | 1x2 | 40MHz/30kHz | 2334 | Open space | 2 | -6.63 |
| 19 | 2 | 1x2 | 40MHz/30kHz | 2334 | Open space | 16 | 5.40 |
| 20 | 3 | 1x2 | 40MHz/30kHz | 2334 | Open space | 16 | 5.40 |
| 21 | 2 | 1x8 | 40MHz/30kHz | 2334 | Open space | 2 | -12.60 |
| 22 | 3 | 1x8 | 40MHz/30kHz | 2334 | Open space | 2 | -12.60 |
| 23 | 2 | 1x8 | 40MHz/30kHz | 2334 | Open space | 16 | 0.40 |
| 24 | 3 | 1x8 | 40MHz/30kHz | 2334 | Open space | 16 | 0.40 |

For 500km/h

Table 2.2-1: Ideal Simulation results for 500km/h for NR PUSCH

| Case Number | l0 position | Antenna configuration | CHBW/SCS | maximum Doppler shift(Hz) | Propagation condition | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 2 | 1x2 | 10MHz/15kHz | 1740 | Tunnel | 2 | -6.60 |
| 26 | 3 | 1x2 | 10MHz/15kHz | 1740 | Tunnel | 2 | -6.60 |
| 27 | 2 | 1x2 | 10MHz/15kHz | 1740 | Tunnel | 16 | 5.41 |
| 28 | 3 | 1x2 | 10MHz/15kHz | 1740 | Tunnel | 16 | 5.41 |
| 29 | 2 | 1x2 | 10MHz/15kHz | 1740 | Open space | 2 | -6.60 |
| 30 | 3 | 1x2 | 10MHz/15kHz | 1740 | Open space | 2 | -6.60 |
| 31 | 2 | 1x2 | 10MHz/15kHz | 1740 | Open space | 16 | 5.40 |
| 32 | 3 | 1x2 | 10MHz/15kHz | 1740 | Open space | 16 | 5.40 |
| 33 | 2 | 1x8 | 10MHz/15kHz | 1740 | Open space | 2 | -8.50 |
| 34 | 3 | 1x8 | 10MHz/15kHz | 1740 | Open space | 2 | -10.28 |
| 35 | 2 | 1x8 | 10MHz/15kHz | 1740 | Open space | 16 | 0.40 |
| 36 | 3 | 1x8 | 10MHz/15kHz | 1740 | Open space | 16 | 0.40 |
| 37 | 2 | 1x2 | 40MHz/30kHz | 3334 | Tunnel | 2 | -6.59 |
| 38 | 3 | 1x2 | 40MHz/30kHz | 3334 | Tunnel | 2 | -6.59 |
| 39 | 2 | 1x2 | 40MHz/30kHz | 3334 | Tunnel | 16 | 5.42 |
| 40 | 3 | 1x2 | 40MHz/30kHz | 3334 | Tunnel | 16 | 5.42 |
| 41 | 2 | 1x2 | 40MHz/30kHz | 3334 | Open space | 2 | -6.62 |
| 42 | 3 | 1x2 | 40MHz/30kHz | 3334 | Open space | 2 | -6.61 |
| 43 | 2 | 1x2 | 40MHz/30kHz | 3334 | Open space | 16 | 5.40 |
| 44 | 3 | 1x2 | 40MHz/30kHz | 3334 | Open space | 16 | 5.40 |
| 45 | 2 | 1x8 | 40MHz/30kHz | 3334 | Open space | 2 | -12.60 |
| 46 | 3 | 1x8 | 40MHz/30kHz | 3334 | Open space | 2 | -12.60 |
| 47 | 2 | 1x8 | 40MHz/30kHz | 3334 | Open space | 16 | 0.40 |
| 48 | 3 | 1x8 | 40MHz/30kHz | 3334 | Open space | 16 | 0.40 |

Conclusion

In this contribution, we give our simulation results on NR HST PUSCH performance requirement for alignment.

Reference

- R4-1915886, Way forward on NR HST PUSCH demodulation requirements, RAN4#93, Huawei, HiSilicon