3GPP TSG-RAN WG4 Meeting #94-e	R4-2001497

# Electronic Meeting, 24th Feb - 6th Mar, 2020

Title: 	Discussion and simulation results on NR UE HST performance requirements under SFN

Source: 	Huawei, HiSilicon

Agenda item:	8.17.2.1.2

Document for:	Discussion

Background

During RAN4#93 meeting, way forward [1] for NR Rel-16 UE demodulation was approved. In this contribution, we share our views about the UE demodulation requirements for NR Rel-16 HST under SFN channel model.

Discussion

Maximum Doppler shift

- For TDD 30 KHz SCS, 500km/h - Option 1: 1500Hz - Option 2: 1667Hz - For FDD 15 KHz SCS, 500km/h - Option 1: 712Hz - Option 2: 875Hz - Option 3: 851Hz - FFS on whether +-0.1ppm UE DL frequency error or lower value should be used when determine the maximum Doppler frequency - Further discuss on the estimation error methodology and other errors

Firstly, we discuss on UE DL frequency error. ±0.1ppm frequency error which is defined in TS 38.101-1[2] is shown below:

The UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms compared to the carrier frequency received from the NR Node B.

At the UE side, UE receives signals with carrier frequency fc from gNB and then UE estimates the Rx frequency fc' according to fc from several RRUs. Then UE calculates Tx frequency fc'' according to fc'. So we can get the following equation:

± 0.1ppm=fc''-fc

It can be understood that ±0.1ppm frequency error contains UE DL frequency error which we concern about and also, UE UL frequency error which is no influence in UE demodulation performance. However, neither of them is defined alone in specs. Considering the worst case, UE DL frequency error is ±0.1ppm. Note that UE only estimate one fc' simultaneously, the frequency error is +0.1ppm or -0.1ppm.

Observation 1: ±0.1ppm frequency error contains UE DL frequency error which we concern about and also, UE UL frequency error which is no influence in UE demodulation performance.

Observation 2: Considering the worst case, UE DL frequency error is 0.1ppm or -0.1ppm.

The following figure shows the Doppler shift and power trajectories for FDD for different RRHs with maximum Doppler shift 875Hz.

Figure 2.1-1: Doppler shift and power trajectories for FDD

For one path, frequency tracking compensation value should be near maximum Doppler shift (875Hz or -875Hz). For two path or more, frequency tracking compensation value should be in somewhere (blue line in Figure 2.1-1) between [-875Hz, 875Hz] based on power of different path.

Doppler spread is about 2 times maximum Doppler shift. Considering the most extreme cases, in above figure, the value of dotted lines which are with FTL error (±0.1ppm) is in range of about [-600Hz, 600Hz], and frequency shift for all paths are not greater than maximum frequency tracking capability. It can be calculatored by same method for TDD.

Observation 3: For SFN, maximum frequency tracking capability is not affected by FTL error no matter where UE is.

So we can get the following proposal:

Proposal 1: No need to consider ±0.1ppm UE DL frequency error and other errors.

For maximum Doppler shift, as per simulation results in section 3, less than 875Hz is all feasible for 15kHz SCS, and less than 1667Hz is all feasible for 15kHz SCS.

As per WF [4], NR BS HST performance requirement is on discussion and the agreement on maximum Doppler shift is shown below:

- Maximum Doppler shift - Single tap HST 500km/h - 15kHz SCS : 1740Hz - 30kHz SCS: 3334Hz

To align with BS, we should define maximum Doppler shift half of maximum Doppler shift defined for BS side for 500km/h, i.e. 870Hz for FDD 15 kHz and 1667 Hz for TDD 30 kHz, respectively.

Proposal 2: Adopt maximum Doppler shift 870Hz for FDD, 1667Hz for TDD.

MCS

- MCS (for Rank 2) - Option 1: MCS4 - Option 2: MCS13 - Option 3: MCS17 - MCS should be decided based on whether the maximum throughput can be achieved

As per simulation results in Section 3, we can get the following observation:

Observation 4: For MCS 17, all the cases are not feasible.

For other options which are feasible, it is reasonable to choose a bigger one, i.e. MCS 13 to achieve a higher throughput for SFN scenario.

Proposal 3: Adopt MCS 13 for SFN.

Simulations

- Updated simulation assumption for HST-SFN Parameter; Value. FDD 15KHz SCS; TDD 30KHz SCS. Antenna configuration; 2×2; 2×4. DMRS type; type 1. Number of DMRS symbols; DMRS 1+1+1. TDD pattern; 7D1S2U, S: 6D 4G 4U. MCS; MCS 4; MCS 13; MCS 17 based on 64QAM table. Propagation condition; HST-SFN. TRS periodicity; 10ms, 2slot pattern. PDSCH mapping; Type A, Start symbol 2, Duration 12. Ds and Dmin; Ds=700m, Dmin=150m. Rank; Rank = 2. BW; 10MHz; 40MHz. Maximum Doppler shift; - Option 1: 712Hz - Option 2: 875Hz - Option 3: 851Hz; - Option 1: 1500Hz - Option 2: 1667Hz. Testing metric; SNR @70% of maximum throughput.

We provide our updated simulations as per the simulation assumptions in [1] for HST SFN as following:

Table 3-1: Ideal Simulation results for NR PDSCH under HST-SFN

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | MCS | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- |
| 1 | 10MHz/15kHz | 712 | 2Tx 2Rx ULA Low | 4 | 0.62 |
| 2 | 10MHz/15kHz | 875 | 2Tx 2Rx ULA Low | 4 | 0.86 |
| 3 | 10MHz/15kHz | 851 | 2Tx 2Rx ULA Low | 4 | 0.82 |
| 4 | 10MHz/15kHz | 712 | 2Tx 2Rx ULA Low | 13 | 9.88 |
| 5 | 10MHz/15kHz | 875 | 2Tx 2Rx ULA Low | 13 | 10.41 |
| 6 | 10MHz/15kHz | 851 | 2Tx 2Rx ULA Low | 13 | 10.28 |
| 7 | 10MHz/15kHz | 712 | 2Tx 2Rx ULA Low | 17 | 15.6 |
| 8 | 10MHz/15kHz | 875 | 2Tx 2Rx ULA Low | 17 | N/A |
| 9 | 10MHz/15kHz | 851 | 2Tx 2Rx ULA Low | 17 | 19.07 |
| 10 | 10MHz/15kHz | 712 | 2Tx 4Rx ULA Low | 4 | -1.94 |
| 11 | 10MHz/15kHz | 875 | 2Tx 4Rx ULA Low | 4 | -1.72 |
| 12 | 10MHz/15kHz | 851 | 2Tx 4Rx ULA Low | 4 | -1.76 |
| 13 | 10MHz/15kHz | 712 | 2Tx 4Rx ULA Low | 13 | 7.57 |
| 14 | 10MHz/15kHz | 875 | 2Tx 4Rx ULA Low | 13 | 8 |
| 15 | 10MHz/15kHz | 851 | 2Tx 4Rx ULA Low | 13 | 7.86 |
| 16 | 10MHz/15kHz | 712 | 2Tx 4Rx ULA Low | 17 | 13.35 |
| 17 | 10MHz/15kHz | 875 | 2Tx 4Rx ULA Low | 17 | 17.68 |
| 18 | 10MHz/15kHz | 851 | 2Tx 4Rx ULA Low | 17 | 16 |
| 19 | 40MHz/30kHz | 1500 | 2Tx 2Rx ULA Low | 4 | 1.11 |
| 20 | 40MHz/30kHz | 1667 | 2Tx 2Rx ULA Low | 4 | 1.2 |
| 21 | 40MHz/30kHz | 1500 | 2Tx 2Rx ULA Low | 13 | 11.09 |
| 22 | 40MHz/30kHz | 1667 | 2Tx 2Rx ULA Low | 13 | 11.37 |
| 23 | 40MHz/30kHz | 1500 | 2Tx 2Rx ULA Low | 17 | 18.46 |
| 24 | 40MHz/30kHz | 1667 | 2Tx 2Rx ULA Low | 17 | N/A |
| 25 | 40MHz/30kHz | 1500 | 2Tx 4Rx ULA Low | 4 | -1 |
| 26 | 40MHz/30kHz | 1667 | 2Tx 4Rx ULA Low | 4 | -1.23 |
| 27 | 40MHz/30kHz | 1500 | 2Tx 4Rx ULA Low | 13 | 8.97 |
| 28 | 40MHz/30kHz | 1667 | 2Tx 4Rx ULA Low | 13 | 9.09 |
| 29 | 40MHz/30kHz | 1500 | 2Tx 4Rx ULA Low | 17 | 15.8 |
| 30 | 40MHz/30kHz | 1667 | 2Tx 4Rx ULA Low | 17 | N/A |

Figure 3-1: Ideal Simulation results for NR PDSCH under HST SFN

Proposals

In this contribution, we discuss and provide our simulation results on NR UE HST performance requirements under SFN channel model. Our observations and proposals are:

Observation 1: ±0.1ppm frequency error contains UE DL frequency error which we concern about and also, UE UL frequency error which is no influence in UE demodulation performance.

Observation 2: Considering the worst case, UE DL frequency error is 0.1ppm or -0.1ppm.

Observation 3: For SFN, maximum frequency tracking capability is not affected by FTL error no matter where UE is.

Observation 4: For MCS 17, all the cases are not feasible.

Proposal 1: No need to consider ±0.1ppm UE DL frequency error and other errors.

Proposal 2: Adopt maximum Doppler shift 870Hz for FDD, 1667Hz for TDD.

Proposal 3: Adopt MCS 13 for SFN.

Reference

- R4-1915926, WF on UE demodulation for NR HST, RAN4#93, CMCC

- TS 38.101-1, User Equipment (UE) radio transmission and reception part 1: Range 1 Standalone

- R4-1915119, Discussion and simulation results on NR UE HST performance requirements under SFN, RAN4#93, Huawei, HiSilicon

- R4-1915886, Way forward on NR HST PUSCH demodulation requirements, RAN4#93, Huawei, HiSilicon