3GPP TSG-RAN WG4 Meeting #94-e	R4-2001537

Online, 24th February – 6th March 2020

Source:	Ericsson, Samsung, Rogers

Title:	TP for TR 37.716-21-11: Including band combinations 7-66_n71

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.716-21-11 to add DC_7A-66A_n71A and DC_7A-66A-66A_n71A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_7-66_n71

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-66_n71 | CA_7-66 | n71 | No |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-66A_n71A | DC_7A_n71A DC_66A_n71A | CA_7A-66A | n71 |
| DC_7A-66A-66A_n71A | DC_7A_n71A DC_66A_n71A | CA_7A-66A-66A | n71 |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_7-66_n71, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n71 DC_7-66-66_n71 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n71 DC_7-66-66_n71 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.1 |

### 5.1.x.4	REFSENS requirements

Co-existence analysis from DC_66_n71 shows that there is no impact from DC_66_n71 UL to Band 7 DL.

Co-existence analysis from DC_7_n71 shows that there is no impact from DC_7_n71 UL to Band 66 DL.

---End of changes---

# Reference

[1]		RP-192815, “Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” Huawei, HiSilicon