3GPP RAN WG4 Meeting #94-e                                     R4-2001553

Online, 24th February – 6th March 2020

Agenda item:	7.11.3.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on the TA offset setting for NR and NB-IoT coexistence

Document for:	Discussion

- Introduction

In last meeting RAN4 93, the RRM measurement on the non-anchor carriers for R16 NB-IoT enhancements are discussed, and a way forward was agreed in [1]. Another issue was raised about the TA offset setting for NR and NB-IoT coexistence. We analyse the problem in this paper and give some options for further discussions.

- Discussion

The value of is defined in TS 38.133 [2] as shown in the table below.

Table I. The Value of

| Frequency range and band of cell used for uplink transmission | (Unit: TC) |
| --- | --- |
| FR1 FDD band without LTE-NR coexistence case or FR1 TDD band without LTE-NR coexistence case | 25600 (Note 1) |
| FR1 FDD band with LTE-NR coexistence case | 0 (Note 1) |
| FR1 TDD band with LTE-NR coexistence case | 39936 (Note 1) |
| FR2 | 13792 |
| Note 1: The UE identifies based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of is set as 25600 for FR1 band. In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to clause 4.2 in TS 38.213 [3] and the value 39936 of can also be provided for a FDD serving cell. Note 2: Void |  |

For some scenarios, the TA offset setting for the coexistence between NR and NB-IoT needs further investigation. For the cases when NR is deployed without LTE-NR coexistence, the is configured as 25600 Tc according to Table I. When deploy NB-IoT in these scenarios, the TA offset setting should unified. However, only 39936 Tc is supported for NB-IoT nodes.

Observation 1: For the scenarios where NR has already deployed without LTE-NR coexistence, the configured is inconsistent with supported value of NB-IoT.

Therefore, we present two options here to handle the issues:

Option 1: For the NR without LTE-NR coexistence case where the has been configured as 25600 Tc, reconfigure the current setting of the NR BS to 39936 Tc.

Option 2: For NB-IoT nodes, the configuration of 25600 Tc shall be supported.

Proposal 1:

Option 1: For the NR without LTE-NR coexistence case where the has been configured as 25600 Tc, reconfigure the current setting of the NR BS to 39936 Tc.

Option 2: For NB-IoT nodes, the configuration of 25600 Tc shall be supported.

For the option 1, the current NR BSs have to be shut off for reconfiguration, which will impact the performance of current networks. For the option 2, additional TA value should be supported, which also has impact on other working groups. We would like to further discuss this issue in the coming RAN4#94 meeting.

- Conclusions

Observation 1: For the scenarios where NR has already deployed without LTE-NR coexistence, the configured is inconsistent with supported value of NB-IoT.

Proposal 1:

Option 1: For the NR without LTE-NR coexistence case where the has been configured as 25600 Tc, reconfigure the current setting of the NR BS to 39936 Tc.

Option 2: For NB-IoT nodes, the configuration of 25600 Tc shall be supported.

References

[1] R4-1915890 Way forward on RRM requirements of R16 enhancement for NB-IoT, Huawei

[2] TS 38.133