3GPP RAN WG4 Meeting #94-e                                       R4-2001556

Online, 24th February – 6th March 2020

Agenda item:	8.1.4.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRC connection mobility control in NR-U

Document for:	Discussion

- Introduction

In last meeting RAN4 93, the RRM requirements and impact of NR-U are discussed, and the potential agreements and the remaining open issues about RRC connection mobility control are summarized in the WF [1].

In this paper, we discuss the remaining issues about RRC connection mobility control in NR-U based on the discussion in the last meeting.

- Discussion

2.1 RRC re-establishment

The RRC connection re-establishment procedure is described in TS 38.331 [2]. The UE initiates the re-establishment procedure when the one conditions in 5.3.7.2 [2] is met. As defined in TS 38.133 [3], In RRC_CONNECTED mode the UE shall be capable of sending RRCReestablishmentRequest message within Tre-establish_delay seconds from the moment it detects a loss in RRC connection. The total RRC connection delay (Tre-establish_delay) shall be less than:

Tre-establish_delay=TUE_re-establish_delay+TUL_grant

The UE re-establishment delay (TUE_re-establish_delay) requirement shall be less than:

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI_NR+TPRACH

Considering the impact of LBT failure in NR-U, the UE behaviors were discussed in the last meeting and three options are presented in [1] as follows:

- UE behavior: - Option 1: - UE behavior is based on K1,max, K2,i,max, and KSI,max (FFS K3,max) - KSI,max is the maximum time for SI reading - Option 2: - Use the existing RAN2 procedure upon the expiring of the T311 timer - Option 3: - UE behavior is based on K1,max, K2,i,max, and KSI,max (FFS K3,max) and timer T311, whichever comes first

As we mentioned in the last RAN4#93 meeting, there are two timers (T311 and T301) to restrict the RRC re-establishment process. The corresponding relationship between the timers and particular UE process in a successful RRC re-establishment process is illustrated in Figure 1.

Figure 1

It can be observed that the requirements of cell search (both intra-frequency and inter-frequency) and the time for acquiring system information are restricted by T311. Therefore, in NR-U scenarios, when the unavailable DMTC cycles exceeding the maximum values, UE should still have the chance for cell searching before T311 expires.

Observation 1: The requirements of cell search (both intra-frequency and inter-frequency) and the time for acquiring system information are restricted by T311

Proposal 1: UE behavior should follow existing T311 and no addition UE behaviors are needed for cell search and acquiring system information, which is the Option 2 in the WF.

With respect to the recovery mechanism for the consistent UL LBT failure from RAN2, the corresponding configurations will be released when the RRC re-establishment is initiated. Thus, the UE will continuously attempt to transmit PRACH until the expiration of T301 based on the following RAN2 agreements:

Agreements in RAN2 #105bis The PREAMBLE_TRANSMISSION_COUNTER is not increased if the preamble is not transmitted due to LBT failure MAC returns to the resource selection step if LBT fails for Msg1 transmission opportunity(ies)

Observation 2: UE will continuously attempt to transmit PRACH until the expiration of T301.

As for the PRACH process, when a suitable cell is found after the cell search process, the timer T301 will be triggered upon the transmission of RRCReestabilshmentRequest from RRC layer. Considering the LBT failure for the PRACH transmission, UE will go to RRC_IDLE mode upon expiration of T301 if the UE fails to transmit the PRACH due to UL LBT.

Proposal 2: UE behaviour for PRACH transmission should follow the existing T301,  which means and K3,max  is not needed.

2.2 RRC release with re-direction

In the RAN4 92bis meeting, the requirements of RRC release with redirection are discussed considering the impact of LBT failure. And the maximum unavailable DMTC cycles due to LBT during the cell search are discussed as:

Based on the discussion in the last RAN4 93 meeting, the UE behavior upon exceeding the limits are FFS. According to the cell search process triggered by the RRC release with redirection in [2], which is shown as:

“At reception of RRCRelease message to transition the UE to RRC_IDLE or RRC_INACTIVE, UE shall attempt to camp on a suitable cell according to redirectedCarrierInfo if included in the RRCRelease message. If the UE cannot find a suitable cell, the UE is allowed to camp on any suitable cell of the indicated RAT.”

Therefore, when the unavailable DMTC cycles during the cell search process exceeds the maximum values, it is reasonable for the UE to camp on any suitable cell of the indicated RAT.

Proposal 3: When the unavailable DMTC cycles during the cell search process exceeds the maximum values, the UE is allowed to camp on any suitable cell of the indicated RAT.

- Conclusions

Observation 1: The requirements of cell search (both intra-frequency and inter-frequency) and the time for acquiring system information are restricted by T311

Proposal 1: UE behavior should follow existing T311 and no addition UE behaviors are needed for cell search and acquiring system information, which is the Option 2 in the WF.

Observation 2: UE will continuously attempt to transmit PRACH until the expiration of T301.

Proposal 2: UE behaviour for PRACH transmission should follow the existing T301, which means and K3,max  is not needed.

Proposal 3: When the unavailable DMTC cycles during the cell search process exceeds the maximum values, the UE is allowed to camp on any suitable cell of the indicated RAT.

References

[1] R4-1915777 “WF on NR-U RRM Requirements”, Ericsson, Qualcomm Inc.

[2] TS 38.331