3GPP TSG-RAN WG4 Meeting #94-e	R4-2001579

Online, 24th February – 6th March, 2020

Title: 	Discussion on measurement restrictions for L1-SINR measurement

Source: 	Huawei, HiSilicon

Agenda item:	8.11.2.1

Document for:	Discussion

Introduction

In RAN4#93 meeting, the RRM requirements for L1-SINR measurements were discussed. For measurement restriction/scheduling availability of L1-SINR reporting measurements, the following agreements has been achieved in [1].

- For CSI-RS configured as CMR: - In FR2 it is that possible to configure CSI-RS with “repetition = on” - Measurement restriction is applied if this CSI-RS with “repetition = on” is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement. - CSI-RS configured as NZP-IMR: - For CSI-RS configured as NZP-IMR in FR2, FFS whether or not CSI-RS with “repetition = on” can be configured, i.e., beam refinement based on NZP-IMR. - Option-1: NZP-IMR only with “repetition = off”. - Option-2: NZP-IMR can be configured with “repetition = off” or “repetition = on”. - CSI-RS configured as ZP-IMR: - For CSI-RS configured as ZP-IMR in FR2, FFS whether or not CSI-RS with “repetition = on” can be configured, i.e., beam refinement based on ZP-IMR. - Option-1: ZP-IMR only with “repetition = off”. - Option-2: ZP-IMR can be configured with “repetition = off” or “repetition = on”.

In this contribution, we provide the discussion on measurement restrictions of L1-SINR reporting measurements

Discussion

In TS38.331, the field description for repetition configuration in NZP-CSI-RS-ResourceSet is defined as follows:

repetition Indicates whether repetition is on/off. If the field is set to off or if the field is absent, the UE may not assume that the NZP-CSI-RS resources within the resource set are transmitted with the same downlink spatial domain transmission filter and with same NrofPorts in every symbol (see TS 38.214 [19], clauses 5.2.2.3.1 and 5.1.6.1.2). Can only be configured for CSI-RS resource sets which are associated with CSI-ReportConfig with report of L1 RSRP or "no report".

It can be observed that only the CSI-RS resource sets for L1-RSRP reporting can be configured with repetition. A CSI-RS resource with repetition configuration will be used for L1-RSRP reporting.

Observation 1: In Rel-15, only the CSI-RS resource sets for L1-RSRP reporting can be configured with repetition.

In 38.331, the signalling of ZP-CSI-RS resource and ZP-CSI-RS resource set are defined as follows:

ZP-CSI-RS-Resource ::= SEQUENCE { zp-CSI-RS-ResourceId ZP-CSI-RS-ResourceId, resourceMapping CSI-RS-ResourceMapping, periodicityAndOffset CSI-ResourcePeriodicityAndOffset OPTIONAL, --Cond PeriodicOrSemiPersistent ... } ZP-CSI-RS-ResourceSet ::= SEQUENCE { zp-CSI-RS-ResourceSetId ZP-CSI-RS-ResourceSetId, zp-CSI-RS-ResourceIdList SEQUENCE (SIZE(1..maxNrofZP-CSI-RS-ResourcesPerSet)) OF ZP-CSI-RS-ResourceId, ... }

It can be observed that there is neither repetition configuration nor QCL configuration for a ZP-CSI-RS resource.

Observation 2: There is neither repetition configuration nor QCL configuration for a ZP-CSI-RS resource.

As we observed in above, a CSI-RS resource with repetition configuration will be used for L1-RSRP measurements. According to the current L1-RSRP measurements, UE needs to perform beam refinement on CSI-RS resource with “repetition=on” and not to perform beam refinement on CSI-RS resource with “repetition=off”.

Observation 3: When a CSI-RS resource with repetition configuration is configured as CMR or IMR for L1-SINR measurement, then the L1-SINR measurement occasions will be always overlapped with the L1-RSRP measurement occasions.

For CMR+IMR scenario, a CSI-RS resource with “repetition=off” is configured as CMR for L1-SINR measurements. If UE needs to perform beam refinement for the L1-SINR measurement, then UE could not guarantee both L1-RSRP measurement and L1-SINR measurement.

Proposal 1: For CMR+IMR scenario, RAN4 study when the measurement restrictions need to be applied between L1-SINR measurement and L1-RSRP measurements.

For CMR+IMR scenario, when SSB is configured as CMR for a L1-SINR measurement, UE need to perform beam refinement for this L1-SINR measurement. Hence, it is may not be suitable to configure the associated IMR as a CSI-RS resource with “repetition=off”. When CSI-RS resources a resource set with “repetition=on” are configured as CMR for a L1-SINR measurement, then it is better to configure the associated IMRs as CSI-RS resources in a resource set with “repetition=on”. RAN4 shall investigate the configuration restrictions between CMR and IMR.

Proposal 2: For CMR and IMR scenario, RAN4 study the configuration restrictions between CMR and IMR, where the.

At least the combinations of CMR and IMR listed in Table 1 need to be studied.

Table 1: Combination list of CMR and IMR

| CMR type | IMR type |
| --- | --- |
| SSB | ZP-IMR |
|  | CSI-RS resource with “repetition=on” |
|  | CSI-RS resource with “repetition=off” |
| CSI-RS resource with “repetition=on” | ZP-IMR |
|  | CSI-RS resource with “repetition=on” |
|  | CSI-RS resource with “repetition=off” |
| CSI-RS resource with “repetition=off” | ZP-IMR |
|  | CSI-RS resource with “repetition=on” |
|  | CSI-RS resource with “repetition=off” |

Conclusions

This contribution provides our analysis on measurement restrictions for L1-SINR measurements. The following are provided:

Observation 1: In Rel-15, only the CSI-RS resource sets for L1-RSRP reporting can be configured with repetition.

Observation 2: There is neither repetition configuration nor QCL configuration for a ZP-CSI-RS resource.

Observation 3: When a CSI-RS resource with repetition configuration is configured as CMR or IMR for L1-SINR measurement, then the L1-SINR measurement occasions will be always overlapped with the L1-RSRP measurement occasions.

Proposal 1: For CMR+IMR scenario, RAN4 study when the measurement restrictions need to be applied between L1-SINR measurement and L1-RSRP measurements.

Proposal 2: For CMR and IMR scenario, RAN4 study the configuration restrictions between CMR and IMR.

Reference

- R4-1915850, “Way forward on NR eMIMO RRM”, Samsung