3GPP TSG-RAN WG4 Meeting #94-e	R4-2001686

# Electronic Meeting, February 24 – March 6, 2020

Source:	Ericsson

Title:	EESS protection for NR BS operation in Band n257 and n258

Agenda item:	6.7.2

Document for:	Approval

# Background

At WRC-19 in Sharm el-Sheikh, a new allocation was identified for terrestrial IMT in the band 24.25 to 27.5 GHz. The new IMT allocation concerns 3GPP bands n257 and n258 for NR. In addition, WRC-19 established unwanted emission limits for protection of EESS in the band 23.6 to 24 GHz.

This contribution discusses the new limits on unwanted emissions and proposes how they can be implemented for NR BS. Related proposals for NR UE limits are given in [2] and [3].

# WRC-19 agreement on EESS protection limits

One of the outcomes of WRC-19 is an identification of a primary allocation for IMT in the band 24.75 GHz to 27.5 GHz applicable to all regions, as outlined in Resolution COM4/8 (WRC-19) of the Provisional Final acts [1]. The resolution also recognizes Resolution 750 (Rev.WRC-19) of the Provisional Final acts [1] that defines limits on unwanted emissions in the frequency band 23.6 to 24 GHz from IMT base stations and IMT mobile stations operating within the 24.25 to 27.5 GHz frequency band. The limits are summarized in Table 1, together with the table notes relevant for the case.

Table 1: Excerpt from Table 1-1 in Resolution 750 [1].

| EESS (passive) band | Active service band | Active service | Limits of unwanted emission power from active service stations in a specified bandwidth within the EESS (passive) band |
| --- | --- | --- | --- |
| 23.6-24.0 GHz | 24.25-27.5 GHz | Mobile | −33 dBWa in any 200 MHz of the EESS (passive) band for IMT base stations5 −29 dBWb in any 200 MHz of the EESS (passive) band for IMT mobile stations5 |
| Relevant Notes: 5 The unwanted emission power level is considered in terms of TRP. The TRP is to be understood here as the integral of the power transmitted from all antenna elements in different directions over the entire radiation sphere. a A limit of −39 dB(W/200 MHz) will apply to IMT base stations brought into use after 1 September 2027. This limit will not apply to IMT base stations which have been brought into use prior to this date. For those IMT base stations, the limit of −33 dB(W/200 MHz) will continue to apply after this date. b A limit of −35 dB(W/200 MHz) will apply to IMT mobile stations brought into use after 1 September 2027. This limit will not apply to IMT mobile stations which have been brought into use prior to this date. For those IMT mobile stations, the limit of −29 dB(W/200 MHz) will continue to apply after this date. |  |  |  |

It should be noted that that there is a “phasing” of the limits introduced in Table 1. A first set of “Phase 1” limits is defined in the table, while a set of more strict limits are defined for “Phase 2” in the notes for equipment brought into use after 1 September 2027.

Observation 1:	The limits of unwanted emissions agreed at WRC-19 are defined in two phases, with stricter limits applicable to IMT base stations brought into use after 1 September 2027.

With the limits adopted by WRC-19 and applicable to all regions, they should also be considered for 3GPP specifications. It is noted that the limits are defined as TRP using dBW, while 3GPP limits are normally defined using dBm.

Observation 2:	The limits of unwanted emissions agreed at WRC-19 are defined as TRP using dBW. The corresponding limits using dBm for IMT base stations would be -3 dBm/200 MHz and -9 dBm/200 MHz (TRP) respectively for Phase 1 and 2.

# Limits applicable to Band n257 and n258

The limits of unwanted emissions defined by WRC-19 in Table 1 would apply in FR2 to NR Bands n257 and n258, as shown in Table 2. Band n257 overlaps partly in the range 26.5-27.5 GHz with the “Active service band” defined in Table 1, while band n258 is identical to the Active service band in Table 1 and thus overlaps fully.

Table 2: Excerpt from TR 38.104 table 5.2-2 (NR operating bands in FR2)

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode |
| --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD |
| n258 | 24250 MHz – 27500 MHz | TDD |

The limits are defined for operation within the “Active service band” (24.25 GHz to 27.5 GHz) as defined by WRC-19 in Resolution 750 [1]. If the NR BS operating band only partly overlaps with that range, the limits should apply for operation in the overlapping range.

Proposal 1:	The limits of unwanted emissions in the range 23.6 GHz to 24 GHz should only apply for a BS where any part of the downlink transmission falls within the “Active service band” (24.25 GHz to 27.5 GHz) defined in Resolution 750 [1].

This is equivalent to the way the limits are proposed to apply for NR UEs in [2].

As noted in Observation 1, there is a “phasing” of the limits, with stricter limits applicable to IMT base stations brought into use after 1 September 2027. There are two ways to implement the phased approach in the NR BS specification:

- Only Phase 1 limits are implemented at this time. The Phase 2 limits would be implemented at a later point in time, but before 1 September 2027.

- Both Phase 1 and Phase 2 limits are implemented now. The phase 2 limits are stated as applicable for equipment brought into use after 1 September 2027.

It is noted that Option b) is more precise and also fully aligned with WRC-19 Resolution 750 [1]. There is also 3GPP precedence for such a solution, where a phased approach was used for Band 23/25 co-existence in 3GPP Rel-10.

Proposal 2:	Both Phase 1 and 2 limits from WRC-19 Resolution 750 [1] are implemented in NR BS specifications. The Phase 1 limits would apply for equipment brought into use until 1 September 2027, while Phase 2 limits would apply after that date.

# Proposal

The following was noted concerning the limits of unwanted emissions agreed at WRC-19 in Resolution 750 [1]:

Observation 1:	The limits of unwanted emissions agreed at WRC-19 are defined in two phases, with stricter limits applicable to IMT base stations brought into use after 1 September 2027.

Observation 2:	The limits of unwanted emissions agreed at WRC-19 are defined as TRP using dBW. The corresponding limits using dBm for IMT base stations would be -3 dBm/200 MHz and -9 dBm/200 MHz (TRP) respectively for Phase 1 and 2.

Based on the discussion above, the following is proposed:

Proposal 1:	The limits of unwanted emissions in the range 23.6 GHz to 24 GHz should only apply for a BS where any part of the downlink transmission falls within the “Active service band” (24.25 GHz to 27.5 GHz) defined in Resolution 750 [1].

Proposal 2:	Both Phase 1 and 2 limits from WRC-19 Resolution 750 [1] are implemented in NR BS specifications. The Phase 1 limits would apply for equipment brought into use until 1 September 2027, while Phase 2 limits would apply after that date.

.

# References

- World Radiocommunication Conference 2019 (WRC-19),  Provisional Final Acts.

- R4-2000230, "EESS protection from n257" (NTT DOCOMO, INC.).

- R4-2000409, "On 3GPP band n258 and WRC-19 EESS unwanted emission limits" (T-Mobile USA, AT&T, U.S. Cellular).