3GPP TSG-RAN WG4 Meeting #94-e                 	R4-2001687

Online, 24th Feb 2020 - 6th Mar 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-16 HST BS demodulation PUSCH and UL TA simulation results

Agenda item:	8.17.2.2

Document for:	Discussion

Introduction

In the RAN4#93 meeting, most of the remaining Rel-16 HST BS PUSCH and UL TA demodulation performance requirements were agreed upon. Agreements and the remaining steps were captured in WF [1].

Main changes from the previous simulation alignment were, the reduction of the centre frequency to 1.8GHz for PUSCH 15kHz 500kph, the resulting change in max Doppler shift, the introduction of 1T2R and 1T1R [FFS], UL TA parameter scaling was re-introduced.

In this contribution, we provide the results of our simulation campaign taking into account the above changes.
Additional simulation results, including full TPUT vs. SNR curves, can be found in our earlier contributions [2] and [3].

PUSCH Simulation results

Parameters

Unless otherwise stated in the following tables, the simulation setup follows the HST PUSCH baseline as captured in [1]:

Table 1: PUSCH HST baseline configuration

| Parameter | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | v = 350km/h |  | v = 500km/h |  |  |
| Transform precoding | Disabled |  | Disabled |  |  |
| Number of Tx | 1 |  | 1 |  |  |
| Number of Rx | Tunnel: 2, FFS on 1 Open space: 2, 8 |  | Tunnel: 2, FFS on 1 Open space: 2, 8 |  |  |
| Number of layers | 1 |  | 1 |  |  |
| SCS | 15kHz, 30kHz |  | 15kHz, 30kHz |  |  |
| Reference signal | DMRS type 1 with 1+1+1 |  | DMRS type 1 with 1+1+1 |  |  |
| l0 | l0 = 2 (for simulation alignment) (Note 1) |  | l0 = 2 (for simulation alignment) (Note 1) |  |  |
| symbols length | 14 |  | 14 |  |  |
| start symbol index | 0 |  | 0 |  |  |
| Time domain resource allocation type | type A |  | type A |  |  |
| Frequency domain resource | Full applicable test bandwidth |  | Full applicable test bandwidth |  |  |
| MCS index | Tunnel: MCS 2 and MCS 16 Open space: MCS 2 and MCS 16 |  | Tunnel: MCS 2, FFS on MCS 16 Open space: MCS 2 and MCS 16 |  |  |
| Maximum Doppler shift | 15kHz SCS: 1340Hz | 30kHz SCS: 2334Hz | 15kHz SCS: 1740Hz |  | 30kHz SCS: 3334Hz |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |  | 15kHz SCS: 1.8GHz (Band n3) 30kHz SCS: 3.6GHz |  |  |
| Propagation condition | HST single-tap channel model: Tunnel: Ds=300m, Dmin=2m Open space: Ds=700m, Dmin=150m |  |  |  |  |
| SCS and BW | 15kHz: 10MHz; 30kHz: 40MHz |  |  |  |  |
| Timing offset | 0 |  |  | 0 |  |
| Frequency offset | 0 |  |  | 0 |  |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |  |  | Disabled |  |
| Number of HARQ transmissions | 4 |  |  | 4 |  |
| Testing metric | SNR @70% of maximum throughput |  |  | SNR @70% of maximum throughput |  |
| Note 1: Company is encouraged to evaluate the performance difference between l0 = 2 and l0 = 3 |  |  |  |  |  |

Results 350 kph

Table 2: PUSCH, 350kph, CP-OFDM, AWGN, Type A TDRA: simulation setup and results summary

| Propagation condition | Carrier frequency (GHz) | SCS | CBW | Maximum Doppler shift | Tx/Rx | DMRS | l0 | MCS | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Ideal, l0=2 | Ideal, l0=3 |
| Tunnel | 2.1 | 15KHz | 10MHz | 1340Hz | 1T1R | 1+1+1 | 2 | 2 |  |  |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 |  |  |
|  |  |  |  |  | 1T2R | 1+1+1 | 2 | 2 | -5,93 | -5,94 |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 | 5,75 | 5,75 |
|  | 3.6 | 30KHz | 40MHz | 2334Hz | 1T1R | 1+1+1 | 2 | 2 |  |  |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 |  |  |
|  |  |  |  |  | 1T2R | 1+1+1 | 2 | 2 | -5,97 | -5,97 |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 | 5,74 | 5,75 |
| Open space | 2.1 | 15KHz | 10MHz | 1340Hz | 1T2R | 1+1+1 | 2 | 2 | -6,00 | -6,00 |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 | 5,73 | 5,73 |
|  |  |  |  |  | 1T8R | 1+1+1 | 2 | 2 |  |  |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 |  |  |
|  | 3.6 | 30KHz | 40MHz | 2334Hz | 1T2R | 1+1+1 | 2 | 2 | -6,01 | -6,02 |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 | 5,72 | 5,72 |
|  |  |  |  |  | 1T8R | 1+1+1 | 2 | 2 |  |  |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 |  |  |

  - Results 500 kph

Table 2: PUSCH, 500kph, CP-OFDM, AWGN, Type A TDRA: simulation setup and results summary

| Propagation condition | Carrier frequency (GHz) | SCS | CBW | Maximum Doppler shift | Tx/Rx | DMRS | l0 | MCS | Ideal SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tunnel | 1.8 | 15KHz | 10MHz | 1740Hz | 1T1R | 1+1+1 | 2 | 2 |  |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 |  |
|  |  |  |  |  | 1T2R | 1+1+1 | 2 | 2 | -5,80 |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 | 6,46 |
|  | 3.6 | 30KHz | 40MHz | 3334Hz | 1T1R | 1+1+1 | 2 | 2 |  |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 |  |
|  |  |  |  |  | 1T2R | 1+1+1 | 2 | 2 | -5,91 |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 | 5,81 |
| Open space | 1.8 | 15KHz | 10MHz | 1740Hz | 1T2R | 1+1+1 | 2 | 2 | -5,97 |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 | 5,75 |
|  |  |  |  |  | 1T8R | 1+1+1 | 2 | 2 |  |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 |  |
|  | 3.6 | 30KHz | 40MHz | 3334Hz | 1T2R | 1+1+1 | 2 | 2 | -5,98 |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 | 5,74 |
|  |  |  |  |  | 1T8R | 1+1+1 | 2 | 2 |  |
|  |  |  |  |  |  | 1+1+1 | 2 | 16 |  |

PUSCH UL TA Simulation results

Parameters

Unless otherwise stated in the following tables, the simulation setup follows the HST PUSCH UL TA baseline as captured in [1].

Table 1: PUSCH HST UL TA baseline configuration

| Parameter | Value |
| --- | --- |
| TDD configuration | - 15 kHz SCS: 3D1S1U, S=10D:2G:2U - 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| PUSCH transmission | - FDD - Slots #0, #2, #4, #6, and #8 in radio frames - TDD - 15 kHz SCS: slots #4 and #9 in radio frames - 30 kHz SCS: slots #8, #9, #18 and #19 in radio frames |
| SRS transmission (optional) | - FDD - Slot #1 in radio frames - TDD - The last symbol in the special slot [FFS] - 15kHz SCS: last symbol in slot #3 in radio frames - 30kHz SCS: last symbol in slot #7 in radio frames |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| SCS | 15kHz and 30kHz |
| Reference signal | DMRS type 1 with 1+1+1 and l0=2 |
| PUSCH TDRA length | Not discussed. |
| PUSCH TDRA start symbol index | Not discussed. |
| PUSCH TDRA type | Type A and type B |
| MCS index | 16 |
| Frequency domain resource allocation for PUSCH | - 10 MHz CBW / 15 kHz SCS: 25 continuously allocated RBs for each UE - Starting PRB index - Moving UE: 0 - Stationary UE: 25 - 40MHz CBW / 30 KHz SCS: 50 RBs continuously allocated for each UE - Starting PRB index - Moving UE: 0 - Stationary UE: 50 |
| Frequency domain resource allocation for SRS | - 10 MHz CBW / 15 kHz SCS: 40 contiguously allocated RBs starting from RPB index 0 - 40 MHz CBW / 30 KHz SCS: 80 contiguously allocated RBs starting from PRB index 0 |
| Maximum Doppler shift | [0] |
| Carrier frequency | FR1 |
| Propagation condition | See Scenario |
| SCS and BW | 10MHz / 15kHz 40MHz / 30kHz |
| Timing offset | Scenario Y |
| Number of HARQ transmissions | Not discussed |
| Testing metric | SNR@70% of maximum throughput for the moving UE |

Table 1: PUSCH HST UL TA baseline scenarios

| Parameter | Scenario X | Scenario Y | Scenario Z |
| --- | --- | --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: TDLC300-400 | Stationary UE: AWGN Moving UE: AWGN | Stationary UE: AWGN Moving UE: AWGN |
| UE speed | 120 km/h | 350 km/h | 500 km/h |
| CP length | Normal |  |  |
| A |  | 15kHz: 10 µs 30kHz: 5 µs |  |
| Δω |  | 15kHz: 0.13 s-1 30kHz: 0.26 s-1 |  |

Results

Table 2: PUSCH UL TA, 350kph, CP-OFDM, 1T2R, Scenario Y: simulation results summary

| PUSCH transmission | SRS transmission | CBW | SCS | DMRS | l0 | MCS | A | Δω | PUSCH mapping type | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Ideal, 70% TPUT | Ideal, 95% TPUT |
| FDD | Enabled | 10MHz | 15KHz | 1+1+1 | 2 | 16 | 10us | 0.13s-1 | Type A | 5.74 | 6 |
|  |  | 10MHz | 15KHz | 1+1+1 | 2 | 16 | 10us | 0.13s-1 | Type B | 5.74 | 6 |
|  | Disabled | 10MHz | 15KHz | 1+1+1 | 2 | 16 | 10us | 0.13s-1 | Type A | 5.74 | 6 |
|  |  | 10MHz | 15KHz | 1+1+1 | 2 | 16 | 10us | 0.13s-1 | Type B | 5.74 | 6 |
| TDD | Enabled | 10MHz | 15KHz | 1+1+1 | 2 | 16 | 10us | 0.13s-1 | Type A | 5.71 | 6.08 |
|  |  | 10MHz | 15KHz | 1+1+1 | 2 | 16 | 10us | 0.13s-1 | Type B | 5.71 | 6.08 |
|  | Disabled | 10MHz | 15KHz | 1+1+1 | 2 | 16 | 10us | 0.13s-1 | Type A | 5.71 | 6.08 |
|  |  | 10MHz | 15KHz | 1+1+1 | 2 | 16 | 10us | 0.13s-1 | Type B | 5.71 | 6.08 |
| FDD | Enabled | 40MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.26s-1 | Type A | 5.69 | 6.01 |
|  |  | 40MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.26s-1 | Type B | 5.7 | 6.03 |
|  | Disabled | 40MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.26s-1 | Type A | 5.7 | 6.03 |
|  |  | 40MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.26s-1 | Type B | 5.69 | 6.01 |
| TDD | Enabled | 40MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.26s-1 | Type A | 5.67 | 5.99 |
|  |  | 40MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.26s-1 | Type B | 5.67 | 5.99 |
|  | Disabled | 40MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.26s-1 | Type A | 5.67 | 5.99 |
|  |  | 40MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.26s-1 | Type B | 5.67 | 6 |

Conclusion

In this contribution we provided the results of our simulation campaign on PUSCH and UL TA baseline simulations for NR_HST Rel-16 BS demodulation performance requirements.

These results are discussed in detail in our companion discussion paper.

- References

- R4-1915886, Way forward on NR HST PUSCH demodulation requirements, Huawei, HiSilicon, RAN4#93.

- R4-1913400, NR Rel-16 HST BS demodulation PUSCH simulation results, Nokia, Nokia Shanghai Bell, RAN4#93.

- R4-1911192, NR Rel-16 HST BS demodulation PUSCH simulation results, Nokia, Nokia Shanghai Bell, RAN4#92-Bis.