3GPP TSG-RAN WG4 Meeting #94                 	R4-2001692

Athens, Greece, February 24th – 28th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-16 performance requirement enhancement BS demodulation simulation results and general discussion

Agenda item:	8.18.2

Document for:	Discussion

Introduction

In RAN4#93, most of the open issues concerning the 30% TPUT test point were agreed upon. Only two remaining issues are captured in the adhoc meeting minutes [1]:

Agreements: Antenna for FR1: 1T2R Use MCS 16 instead of MCS 2 for full PRB allocation. Waveform: only CP-OFDM for MCS 16 DM-RS configuration - FR2: - Option 1: 1+1 - Option 2: 1+0 - Option 3: 1+1 and 1+0 PT-RS for FR2: - Option 1: with and without PT-RS - Option 2: without PT-RS - Option 3: with PT-RS Channel Model: - FR1: TDLC300-100 (MCS 16) - FR2:TDLA30-300 (MCS 16) Aim to finalize all the test requirements in the next meeting.

We consider all questions on FR2 PUSCH 2T2R MCS 12 testing to be agreed upon, hence we provided the corresponding CRs in this meeting, as per the split of responsibilities agreed upon by email before RAN4#93:

Email discussion: RE: [NR Rel-16 Demod] CR work split for NR demod enhancement WI CR Responsibility. UE demodulation and CSI (38.101-4/307); CA normal demodulation for NR CA, EN-DC, NE-DC, NR-DC; FR1; Applicability; Intel. 2Rx requirements; CMCC. 4Rx requirements; China Telecom,Huawei. FRC; Intel. FR2; Applicability; Intel. 2Rx requirements; Huawei,Qualcomm. FRC; Intel. CA CQI for NR CA, EN-DC, NE-DC, NR-DC; To be added later. Tx ports larger than 8 and up to 32; Applicability; Huawei. Requirements for type I single-panel codebook; Ericsson, Huawei. Requirements for type II codebook; Samsung. FRC; Samsung. TDD LTE-NR co-existence; Applicability and requirements; Intel. FRC; Intel. CA power imbalance; To be added later. Release independence; China Telecom. BS demodulation (38.104/141); PUSCH at 30% throughput; Applicability and requirements; CATT, Ericsson. FR2 PUSCH 2T2R 16QAM; Requirements; ZTE, Nokia. FRC; ZTE.

In this contribution we deliver our simulation results and discuss the remaining open points (PT-RS and DM-RS configuration) for 30% TPUT test points.

Discussion on 30% TPUT test point open issues

DM-RS configuration

In the simulation delivery section, we present our evaluations for configuring FR2 DM-RS as 1+0 and 1+1, i.e., addPos={pos0, pos1}.
We observe the following:

The difference between FR2 DM-RS addPos={pos0, pos1} is a slight SNR degradation of less than 1dB for the 30%TPUT test point, with addPos=pos0. The absolute TPUT of DM-RS 1+0 is higher than DM-RS 1+1.

All three options from the last meeting are acceptable.

PT-RS configuration

In the simulation delivery section, we present our evaluations for configuring PT-RS on and off in FR2 MCS16.
We observe the following:

The difference between FR2 MCS16 PT-RS on and off is a slight SNR degradation of up to 0.5dB for the 30%TPUT test point. DM-RS configuration 1+0 and 1+1 are similarly impacted.

Hence, we propose

RAN4 to only define requirements with FR2 PT-RS deactivated.

Simulation delivery 30% TPUT

Unless stated otherwise in the following tables, the simulation setup follows the standard PUSCH setup, as captured in [2]. This is a revision and extension of the results, provided by Nokia in [3].

Table 1: Simulation summary 30%TPUT SINR.

| PUSCH 1T2R |  |  |  |  |  | SNR (dB) @ |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal 70% TP | Ideal 30% TP |
| CP-OFDM | FR1 Mappying type A | 15kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.79 | 0.55 |
|  |  | 30kHz, 40MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.58 | 0.49 |
|  | FR1 Mappying type B | 15kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.86 | 0.58 |
|  |  | 30kHz, 40MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.59 | 0.48 |
|  | FR2 with PTRS | 60kHz, 100MHz | MCS16 | TDLA30-300 | DMRS 1 | 8.84 | 0.68 |
|  |  |  |  |  | DMRS 1+1 | 8.16 | 0.7 |
|  |  | 120kHz, 100MHz | MCS16 | TDLA30-300 | DMRS 1 | 7.77 | 0.82 |
|  |  |  |  |  | DMRS 1+1 | 7.67 | 0.76 |
|  | FR2 without PTRS | 60kHz, 100MHz | MCS16 | TDLA30-300 | DMRS 1 | 8.43 | 0.59 |
|  |  |  |  |  | DMRS 1+1 | 7.67 | 0.4 |
|  |  | 120kHz, 100MHz | MCS16 | TDLA30-300 | DMRS 1 | 7.24 | 0.47 |
|  |  |  |  |  | DMRS 1+1 | 7.1 | 0.42 |
| CP-OFDM | FR1 Mappying type A | 15kHz, 5MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.81 | 0.59 |
|  |  | 15kHz, 20MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.79 | 0.45 |
|  |  | 30kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.7 | 0.63 |
|  |  | 30kHz, 20MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.66 | 0.53 |
|  |  | 30kHz, 100MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.58 | 0.49 |
|  | FR1 Mappying type B | 15kHz, 5MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.88 | 0.61 |
|  |  | 15kHz, 20MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.87 | 0.51 |
|  |  | 30kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.72 | 0.62 |
|  |  | 30kHz, 20MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.7 | 0.51 |
|  |  | 30kHz, 100MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 7.58 | 0.49 |
|  | FR2 with PTRS | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 8.59 | 0.92 |
|  |  |  |  |  | DMRS 1+1 | 8.14 | 0.81 |
|  |  | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 7.8 | 1.14 |
|  |  |  |  |  | DMRS 1+1 | 7.48 | 0.96 |
|  |  | 120kHz, 200MHz | MCS16 | TDLA30-300 | DMRS 1 | 7.99 | 0.83 |
|  |  |  |  |  | DMRS 1+1 | 7.76 | 0.72 |
|  | FR2 without PTRS | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 8.34 | 0.58 |
|  |  |  |  |  | DMRS 1+1 | 7.63 | 0.46 |
|  |  | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 7.19 | 0.62 |
|  |  |  |  |  | DMRS 1+1 | 6.87 | 0.47 |
|  |  | 120kHz, 200MHz | MCS16 | TDLA30-300 | DMRS 1 | 7.45 | 0.53 |
|  |  |  |  |  | DMRS 1+1 | 7.21 | 0.43 |

Conclusion

In this contribution we have delivered our simulation results and discussed the remaining open issues (FR2 PT-RS and DM-RS configuration) for 30% TPUT test points.
We have made the following observations and proposals:

DM-RS configuration

- The difference between FR2 DM-RS addPos={pos0, pos1} is a slight SNR degradation of less than 1dB for the 30%TPUT test point, with addPos=pos0. The absolute TPUT of DM-RS 1+0 is higher than DM-RS 1+1.

PT-RS configuration

the 30%TPUT test point. DM-RS configuration 1+0 and 1+1 are similarly impacted.

- RAN4 to only define requirements with FR2 PT-RS deactivated.

- References

- R4-1915804, Ad Hoc minutes for NR Rel-15 and Rel-16 BS demodulation, China Telecom, Huawei, HiSilicon, RAN4#93.

- R4-1907241, Way forward on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#91.

- R4-1911196, NR Rel-16 performance requirement enhancement BS demodulation simulation results, Nokia, Nokia Shanghai Bell, RAN4#92-Bis.