3GPP TSG-RAN WG4 Meeting #94-e                 	R4-2001695

Online, 24th Feb 2020 - 6th Mar 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	On NR Rel-16 high reliability BS demodulation test feasibility and methodology

Agenda item:	8.9.1.1

Document for:	Discussion

Introduction

In RAN4#93 good progress was made w.r.t. deciding the high reliability demodulation minim performance requirement test feasibility and finding possible test/configuration scenarios [1], [2].

Arguably the main advance was to decide on two, potentially practical, testing methods to be used for evaluation of the feasibility of high reliability KPI tests. In the following, these two approaches will be called method 1 (exhaustive, M1) and method 2 (perfect channel, M2):

[2]:

- Introduce PUSCH low BLER high confidence requirement - If feasible, define [1] test case to verify 10^-5 BLER - Target BLER: 10^-5 - Target test confidence level: 99.999% - Propagation conditions: Static channel - MCS: MCS5 from MCS Table 2 for PUSCH - Duplex mode: Both TDD and FDD - FFS TDD patterns - SCS: - TDD: 30KHz - FFS 15kHz - FDD: 15KHz - FFS 30KHz - Test method: refer to R4-1915866 (ad hoc minutes for NR URLLC test feasibility) - Method 1: Consider aggregation 1 or 2, but no HARQ for non-boosted SNR - Method 2: No aggregation or HARQ for boosted SNR. - FFS whether to use method 1 or method 2 for testing (as described below). Adjustment of the baseline parameters for the long test after simulations is not precluded.

[1]:

Baseline parameters for first test target BLER 10^-5 and CL 99.999%: No aggregation or HARQ for boosted SNR (method 2). Consider aggregation 1, 2 but not HARQ for non boosted SNR (method 1) Static channel MCS: MCS5 from table 3 for PDSCH, MCS5 from table 2 for PUSCH Consdier both TDD and FDD slot patterns. TDD pattern FFS. SCS: For BS. TDD: 30kHz, [FFS 15kHz] FDD: 15kHz, [FFS 30kHz]. For UE: 30kHz for TDD, FDD: 15kHz, [FFS 30kHz]. […] Baseline parameters for methodology 1: N (i.e. pass/fail criterion reported after observing N errors): [1, 10, 100] M (Bad device factor): 1.5 Baseline parameters for methodology 2: Targeted “real” BLER: [zero, 10^-7, 10^-6, 0,5*10^-5] M (Bad device factor): 1.5

In this contribution we analyse the proposed two methods in terms of test feasibility and meaningfulness and propose which method is suitable under expected practical test circumstances. We also contribute simulation results for both methods.

High reliability test feasibility

Method 1 - Exhaustive

Method 1 (M1) does not aim to substantially modify the standard early termination methodology for statistical testing. Major gains in reducing testing time come from reducing the transmission time per transport block, by reducing PUSCH aggregation to max. n2 and disabling HARQ.
Furthermore M1 was envisioned to reduce the required per test step confidence levels and risk levels (clearly pass and dearly fail), and therefore sample numbers/test time, by using fewer decision coordinates. The impact of reducing decision coordinates, on expected testing time is not immediately obvious and will be a main focus of this analysis.

As a side note, the bandwidth of the PUSCH frequency domain allocation can also be reduced as the static channel does not require much frequency diversity to offset possible systematic deep fading subcarriers. More relevant configuration parameters, repeated from the introduction, are

- Target BLER: 10^-5 - Target test confidence level: 99.999% - Propagation conditions: Static channel - MCS: MCS5 from MCS Table 2 for PUSCH - Duplex mode: Both TDD and FDD - SCS: TDD: 30KHz - Test method: refer to R4-1915866 (ad hoc minutes for NR URLLC test feasibility) - Method 1: Consider aggregation 1 or 2, but no HARQ for non-boosted SNR

and

Baseline parameters for first test target BLER 10^-5 and CL 99.999%: Consider aggregation 1, 2 but not HARQ for non boosted SNR (method 1) Static channel MCS: MCS5 from table 3 for PDSCH, MCS5 from table 2 for PUSCH Consdier both TDD and FDD slot patterns. TDD pattern FFS. SCS: For BS. TDD: 30kHz, [FFS 15kHz] FDD: 15kHz, [FFS 30kHz]. Baseline parameters for methodology 1: N (i.e. pass/fail criterion reported after observing N errors): [1, 10, 100] M (Bad device factor): 1.5.

Since M2 is so close to the standard methodology, we can refer to our previous contributions [3] for the justification of our way to calculate min, max, and expected testing times, with the above configurations and targets.

Applying the known testing time evaluation methods, leads us to the early termination methodology parametrization and decision coordinate observations, summarized in Table 1 and the corresponding remarks.
The main outcomes are the newly found per test step decision risks (which dominate the testing time) for the reduced set of decision coordinates [1, 10, 100, netarget], which improved much less than one could have thought (see comparison with full coordinate range in Table1). We introduced the worst-case error number in the decision coordinates, since otherwise most tests cannot be conclusively decided. Which leads us to:
The observed average number of error events to be required before a decision is taken (neobs) is one of the most interesting outcomes, since it indicates that reduction of decision coordinates is not a successful strategy for reducing testing time for marginal DUTs.

Table 1: Early termination methodology parametrization and observations for decision coordinates at error events [1, 10, 100, netarget] and assuming marginal and non-marginal DUTs.

| BLERtarget | BLERreal | CLtest | dearly fail | clearly pass | netarget | neobs | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.01 | 0.01 | 1-BLERtarget | 8e-3 | 1-1e-3 | 182 | 182.0 |  |
| 1e-3 | 1e-3 | 1-BLERtarget | 8e-4 | 1-1e-4 | 286 | 286.0 |  |
| 1e-4 | 1e-4 | 1-BLERtarget | 8e-5 | 1-5e-6 | 406 | 406.0 | False pass of bad DUT adjusted |
| 1e-5 | 1e-5 | 1-BLERtarget | 8e-6 | 1-5e-7 | 513 | 513.0 | Extrapolated |
| 1e-5 | 1e-5 | 0.99999 | 8e-7 | 1-2.5e-7 | 607 |  | Full decision coordinate range [1:netarget] for comparison. |
| 1e-4 | 0 | 1-BLERtarget | 8e-5 | 1-5e-6 | 406 | 1.0 |  |
| 1e-4 | BLERtarget/10 | 1-BLERtarget | 8e-5 | 1-5e-6 | 406 | 10.0 |  |
| 1e-4 | BLERtarget/10 | 1-BLERtarget | 8e-5 | 1-5e-6 | 406 | 36.5 |  |

Remarks:

- The per test step confidence levels and risk levels (clearly pass and dearly fail) are verified by the statistical analysis outlined in [4, Section G.7.10].

- netarget is the number of error events at the intersect point of the early pass/fail curves based on the per step decision risk (and using M). It represents the point, where the test methodology is able to decide all the test population. See visualisation in Figure 1.

- neobs is the observed average error event decision coordinate, at which pass/fail decisions are taken.

- The decision coordinates were changed from the M1 agreements to include netarget, so that all the test population can be decided. Without this addition, barely any marginal DUTs would be decided for pass/fail.

Figure 1: Decision thresholds in reduced decision coordinate framework for BLER=1e-5 with CLtest = 1-1e-5.

We make the following summarizing observations:

Marginal DUTs are almost always decided in the “bin” covering the last decision coordinate. Hence, netarget dominates the number of required error samples, and virtually no early termination is observed.

Due to fewer decision coordinates, and hence less risk accumulation, the netarget values are reduced, when compared to the unmodified methodology, however the gain is surprisingly low.
For BLER=1e-5 with CLtest = 1-1e-5 we extrapolate a required number of error observations of 513, which is a reduction of 15%, when compared to the unmodified test methodology (“607”, see [3, Table 6]). This value also approximately holds for the number of samples, i.e., testing time, that is evaluated later.

To translate the observations from Table 2, into an expected testing time, we take look at the full set of decision coordinates for BLER=1e-5 with CLtest = 1-1e-5, i.e., the tuple (ne, nsp, nsf) [5]:

Table 2: Full of the decision coordinates for BLER=1e-5 with CLtest = 1-1e-5 in marginal DUTs

| BLERtarget | CLtest | dearly fail | clearly pass | ne | Minimum number of samples for early pass decision (nsp) | Minimum number of samples for early fail decision (nsf) |
| --- | --- | --- | --- | --- | --- | --- |
| 1e-5 | 1-BLERtarget | 8e-6 | 1-5e-7 | 1 | 967’244 | 1 |
| 1e-5 | 1-BLERtarget | 8e-6 | 1-5e-7 | 10 | 2’243’230 | 162’135 |
| 1e-5 | 1-BLERtarget | 8e-6 | 1-5e-7 | 100 | 10’451’159 | 6’257’272 |
| 1e-5 | 1-BLERtarget | 8e-6 | 1-5e-7 | 513 | 42’102’204 | 42’108’702 |
|  |  |  |  | One decision coordinate tuple |  |  |

Using the above value of nobs and the system configuration (PUSCH, 30kHz SCS, aggregation factor n2, TDD pattern 7D1S2U with S=6D:4G:4U), we get the expected testing time for marginal DUTs:

The expected testing time using M1 for the scenario (PUSCH, marginal DUTs, 30kHz SCS, aggregation factor n2, TDD pattern 7D1S2U) is 42.1e6 x 0.5ms x 2 x 10/2 = 60 hours.

Method 2 - Perfect channel

In method 2 (M2), the impact of the channel (and thus equalization implementation performance) is removed from the test by boosting the SNR, until the static channel becomes error free (“perfect” or “constant identity”). 
In other words, M2 tests if there is an error floor in the performance due to implementation errors, but it does not check the performance of said implementation. This change in testing goal can be exploited to reduce testing time.

For the case of BLERreal = 0, M2 substantially modifies the standard early termination methodology for statistical testing. In this case it transforms the testing method to a single shot method (see [3, section 3.2] for single shot vs. early termination details). Single shot statistical tests do not suffer from risk accumulation. Furthermore, even the “no error/ne=0 case” terminates earlier with full statistical power, as one does not allow for continuation of the test after error event observation (no “rolling the dice until the result is right”).

In all other cases, the standard early termination framework testing time evaluation methods still apply. 
However, for a perfect channel the true BLER of the DUT can be expected to be much lower than the design target (since the detrimental channel effect is removed, which was included in the DUT design), and thus the DUT testing can reasonably expected to terminate with very few error events, i.e., early.

BLERreal = 0

In this case it is enough to run the DUT up to a given number of TBs and decide pass, given that no error is observed. If an error is observed at any point, the DUT fails.

Following the single shot formula for a perfect DUT [3, section 3.2]:

ns=-ln1-CLtestBLERtarget

One observes that the test for BLERtarget=1e-5 and CLtest = 1-1e-5, terminates after 1’151’293 samples (i.e., transport blocks).

The expected testing time using M2 for the scenario (PUSCH, perfect DUTs, 30kHz SCS, aggregation factor n1, TDD pattern 7D1S2U) is 1.15e6 x 0.5ms x 1 x 10/2 = 1.6 hours.

    - BLERreal not 0

In this case the already known framework of analyzing the early termination with decision at every error event method can be applied [3]. Due to computational constraints we run the simulations with BLERtarget=1e-4 and assume that the neobs is similar to the 1e-5 case, which was seen to be reasonable in the analysis of M1:

Table 3: Early termination methodology parametrization and observations for decision coordinates at error events [1 : netarget] and assuming non-marginal DUTs.

| BLERtarget | BLERreal | CLtest | dearly fail | clearly pass | netarget | neobs | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1e-4 | BLERtarget/10 | 1-BLERtarget | 8e-5 | 1-5e-6 | 406 | 5.9 |  |
| 1e-4 | BLERtarget*10 | 1-BLERtarget | 8e-5 | 1-5e-6 | 406 | 9.1 |  |

With the required samples being

Table 4: Full of the decision coordinates for BLER=1e-5 with CLtest = 1-1e-5 in marginal DUTs (2nd set)

| BLERtarget | CLtest | dearly fail | clearly pass | ne | Minimum number of samples for early pass decision (nsp) | Minimum number of samples for early fail decision (nsf) |
| --- | --- | --- | --- | --- | --- | --- |
| 1e-5 | 1-BLERtarget | 8e-6 | 1-5e-7 | 6 | 1’750’886 | 45’148 |
| 1e-5 | 1-BLERtarget | 8e-6 | 1-5e-7 | 9 | 2’124’999 | 127’820 |

The expected testing time using M2 for the scenario (PUSCH, BLERtarget/10 good DUTs, 30kHz SCS, aggregation factor n1, TDD pattern 7D1S2U) is 1.75e6 x 0.5ms x 1 x 10/2 = 2.5 hours.

Comparison of M1 and M2

When comparing M1 and M2, we need to take into account that both methods will experience different testing scenarios.

Our underlying assumption is that the DUT was designed to show the BLERtarget performance in real deployment. Hence, M1 has to deal with a testing scenario where the tested DUTs are marginal DUTs. 
M2 has an advantage, since the removal of channel uncertainty improves the BLER performance in the test environment, as the DUT was designed for a harsher environment.

Thus, it is a reasonable assumption that M2 will generally terminate with less observed error events, i.e., faster, than M1 and within a practical timeframe of less than 3 hours.

M1 has to deal with a test scenario, where the tested DUTs are marginal DUTs. M2 has an advantage, as the removal of channel uncertainty improves the BLER performance in the test environment, due to back-off from design targets. Thus, it is a reasonable assumption that M2 will terminate with less observed error events, i.e., faster, than M1 and within a practical timeframe of less than 3 hours.

Furthermore, the anticipated 60 hours of testing with M1 are not practically feasible in our opinion.

RAN4 to adapt method 2 or declare high reliability KPI not testable.

BS demodulation high reliability simulations

In this section we will quickly state the results of our simulation campaigns. The results will be references in the preceding chapters.

When obtaining the results, we based ourselves on the following configuration. If a parameter is not specifically stated given in the following subsections, please consult this table for the default value.

Table 8: PUSCH high reliability testing simulation setup

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| System | Mod/Demod branches | 1T2R |
|  | fc | 4 GHz |
|  | SCS | {15, 30} kHz |
|  | BW | 25MHz / 65 PRB |
|  | Uplink-downlink allocation for TDD | {Uplink slots only; 7D1S2U, S=6D:4G:4U} |
|  | Channel model | AWGN (static) with Low Correlation Matrix |
|  | Cyclic Prefix | Normal |
| High reliability features | pusch-AggregationFactor | {n1, n2} |
|  | mcs-Table | 2 (38.214 table 5.1.3.1-2) 3 (lowSE, 38.214 table 5.1.3.1-3) |
|  | MCS | {5, 6} |
|  | maxRank | 1 |
| HARQ | Maximum number of HARQ transmissions | HARQ OFF |
|  | RV sequence | OFF |
|  | Code block group based PUSCH retransmission | Disabled |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | B |
|  | Start symbol | 0 |
|  | Allocation length | 5 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Others | Channel coding: Limited buffer rate matching | Disabled |
| Performance metric |  | SNR@ 5-9s BLER with 5-9s CLtest |

Sim: Method 1 - Exhaustive

Method 1: Consider aggregation n1 and n2, but no HARQ for non-boosted SNR.
The SINR (dB) for the target BLER of 1e-5 are -6.8 dB and -8.9 dB, respective for first and second transmission of the aggregated transmission.

Figure 1:PUSCH high reliability testing simulation of standard scenario with Method 1; BLER vs SINR, Aggregation factor = n1 and n2.

Remark: The simulation of the above curves for M1 took 1829.4 (parallelized) CPU days. The number of error samples (~10 at 1e-5) does not suffice for CLtest=99.999%.

Sim: Method 2 - Perfect channel

Method 2: No aggregation, nor HARQ for boosted SNR.
The simulation was run for SINR of 30 dB with 1e8 transport blocks transmitted and we did not observe any error event.

Remark: The simulation of the M2 took 1135.8 (parallelized) CPU days. The statics of this result are above the required CLtest=99.999% of BLER lower than 1e-5.
Following the single shot formula for a perfect DUT, with a value of 1e8 transport blocks/samples:

ns=-ln1-CLBLER

This gives CL of the simulation implementation having an error floor of BLER <= 1e-5, of “1” within the accuracy of Matlab.

Conclusions

In this contribution we analysed the two proposed methods for testing high reliability (5-9s) requirements in terms of test feasibility and meaningfulness and proposed to either choose M2 or do not specify requirements with 5-9s BLER KPI, under the expected practical test circumstances. We also contributed simulation results based on both methods. 
We have made the following proposals and observations:

Method 1 - Exhaustive

- Marginal DUTs are almost always decided in the “bin” covering the last decision coordinate. Hence, netarget dominates the number of required error samples, and virtually no early termination is observed.

Due to fewer decision coordinates, and hence less risk accumulation, the netarget values are reduced, when compared to the unmodified methodology, however the gain is surprisingly low.
For BLER=1e-5 with CLtest = 1-1e-5 we extrapolate a required number of error observations of 513, which is a reduction of 15%, when compared to the unmodified test methodology (“607”, see [3, Table 6]). This value also approximately holds for the number of samples, i.e., testing time, that is evaluated later.

The expected testing time using M1 for the scenario (PUSCH, marginal DUTs, 30kHz SCS, aggregation factor n2, TDD pattern 7D1S2U) is 42.1e6 x 0.5ms x 2 x 10/2 = 60 hours.

Method 2 - Perfect channel

The expected testing time using M2 for the scenario (PUSCH, perfect DUTs, 30kHz SCS, aggregation factor n1, TDD pattern 7D1S2U) is 1.15e6 x 0.5ms x 1 x 10/2 = 1.6 hours.

The expected testing time using M2 for the scenario (PUSCH, BLERtarget/10 good DUTs, 30kHz SCS, aggregation factor n1, TDD pattern 7D1S2U) is 1.75e6 x 0.5ms x 1 x 10/2 = 2.5 hours.

Comparison of M1 and M2

M1 has to deal with a test scenario, where the tested DUTs are marginal DUTs. M2 has an advantage, as the removal of channel uncertainty improves the BLER performance in the test environment, due to back-off from design targets. Thus, it is a reasonable assumption that M2 will terminate with less observed error events, i.e., faster, than M1 and within a practical timeframe of less than 3 hours.

RAN4 to adapt method 2 or declare high reliability KPI not testable.

- References

- R4-1915866, Ad-hoc minutes for NR URLLC test feasibility, Ericsson, RAN4#93.

- R4-1915913, Way forward on NR URLLC demodulation and CSI requirements, Huawei, RAN4#93.

- R4-1913406, On NR Rel-16 high reliability BS demodulation requirements and test feasibility, Nokia, Nokia Shanghai Bell, RAN4#93.

- TS 36.521-1, V16.2.1, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance testing.

- R4-1911198, On NR Rel-16 high reliability BS demodulation requirements and test feasibility, Nokia, Nokia Shanghai Bell, RAN4#92-Bis.