3GPP TSG-RAN WG4 #94-e	R4-2001711

Electronic meeting, 24 February-6 March 2020

Agenda Item:	8.1.4.13

Source:	Ericsson

Title:	On the timing reference cell adaptation under DL LBT failure in reference cell

Document for:	Approval

# 1	Introduction

In DL, the BS may experience CCA failure in a carrier and not able to transmit, thus the UE will not be able to see the carrier anymore. In case of PCell or PScell, since these cells are used as timing reference for UL transmissions, if these carriers are available anymore due to CCA failure, the SCells need to assign timing reference for the forthcoming UL transmissions.

In this contribution, we describe the above issue for different scenarios and provide our proposal on how to solve this issue.

# 2	Discussion

As we have discussed earlier in [2], scenario B and C will experience the issue that, the DL reference cell may be lost due to LBT failure. In RAN4#93 meeting, following way forward was agreed:

WF on Timing reference cell as discussed in RAN4#93 [1]:

- Scenario B and C: if the current reference cell (i.e. PCell in Scenairo C or PSCell in Scenario B) is unavailable to UE after certain number of DL SSB detection attempts, then the UE can take any activated SCell with SSB available at the UE within this CG as the new reference cell

- FFS: The UE applies one shot timing adjustment

Since the PScell in scenario B and Pcell in scenario C will be on same TAG with other UL carriers (since they are all intra-band NR-U UL carriers), there is no need to apply any one shot adjustment. Thus we agree that,

Proposal: There is no need to apply any one shot adjustment when timing reference cell need to be changed due to LBT failure.

So, the situation will be:

- For scenario B, NR-U carriers will be in STAG. If no SSB of a reference cell on a carrier frequency belonging to the STAG, which is subject to CCA, is available at the UE for more than 160 ms then the UE shall use any of the activated SCell in STAG as a new reference cell provided at that least one SSB is available at the UE in the new reference cell.

- For scenario C, all NR-U carriers will be in PTAG. If no SSB of a reference cell on a carrier frequency belonging to the PTAG, which is subject to CCA, is available at the UE for more than 160 ms then the UE shall use any of the activated SCell in PTAG as a new reference cell provided at that least one SSB is available at the UE in the new reference cell.

Based on the above understanding, we propose to agree on the draft CR as submitted in [3]. The CR reflects the above sentences.

# 3	Summary

We propose to agree on the proposal in this paper and subsequently agree on the related CR in [3]

# References

- R4-1915777, WF on NR-U RRM Requirements (all agreements in RAN4#93), Ericsson, Qualcomm Inc.

- R4-1914617, Further discussions on timing reference cell adaptation under DL LBT failure in reference cell, Ericsson

- R4-2001710, ”Draft CR on UE transmit timing accuracy and timing reference cell under DL LBT failure”, Ericsson.