3GPP TSG-RAN WG4 Meeting #94-e                                                   R4-2001727

Electronic Meeting, February 24-March 06, 2020

Source:	Ericsson, Nokia, Nokia Shanghai Bell

Title:	Update to NR-U FRC definition

Agenda item:	8.1.3.2

Document for:	Approval

# 1.	Introduction

During RAN4#93, interested companies defined the new NR-U BS RX simulation assumptions needed for input towards new FRC(s) to be defined.  The FRC parameters will differ for NR-U compared to NR due to interleaving aspects for NR-U compared to that of NR.

# 2.	Discussion

During deeper inspection of the FRC parameters agreed in [1] some discoveries of inaccuracies arose.  The following are a list of FRCs for the support of 60 kHz SCS:

- FRC-A1-12

- FRC-A1-15

- FRC-A1-18

- FRC-A1-20

- FRC-A1-22

- FRC-A2-9

- FRC-A2-12

- FRC-A2-15

- FRC-A2-17

- FRC-A2-19

As there is no support for interlace in TS 38.211, it would not be necessary to have specific NR-U FRCs for 60 kHz SCS.

Proposal 1: No support for interlacing for NR-U for 60 kHz SCS.  Remove above listed FRCs.

This would mean that additional receiver requirements which consider interlacing would not be needed.  However, how this would affect the BS RF receiver requirements would still need to be discussed.  One approach is that no specific NR-U receiver requirements would be applied and for 60 kHz.

It can also be observed that the number of resource block interlaces are fixed values dependent upon subcarrier spacing.  In which case, the current formulation of interlacing does not align with TS 38.211.

TS 38.211 Release 16 exert:

#### 4.4.4.6 Interlaced resource blocks Multiple interlaces of resource blocks are defined where interlace m∈0,1,…,M-1 consists of common resource blocks m,M+m, 2M+m, 3M+m, …, with M being the number of interlaces given by Table 4.4.4.6-1. The relation between the interlaced resource block nIRB,mμ∈0,1,… in bandwidth part i and interlace m and the common resource block nCRBμ is given by nCRBμ=MnIRB,mμ+NBWP,istart,μ+m-NBWP,istart,μ mod M where NBWP,istart,μ is the common resource block where bandwidth part starts relative to common resource block 0. When there is no risk for confusion the index μ may be dropped. Table 4.4.4.6-1: The number of resource block interlaces. μ; M. 0; 10. 1; 5.

The following is an example of these updates.

Proposal 2: Align FRC interlacing design according to TS 38.211

The following two tables have the following changes added upon the existing agreed tables.  These tables in the Annex help to determine the parameters for simulations to generate SNR operating point of 95% throughput.

The current proposed values are to illustrate the changes to FRC parameters.  Currently due to the nature of the interlace for large bandwidths this leaves some interlace parameters overlapping in nature.  A few options exist to address this issue.  First, to leave the overlapping interlacing as a parameters.  Second, to have separate interlace configurations in the notes that is bandwidth dependent.  Third, ensure non-overlapping interlacing parameters.

Table 1. FRC parameters for NR-U reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation, in-channel selectivity [based on WF R4-1916162]

| Reference channel | G-FR1-A1-10 | G-FR1-A1-11 |  | G-FR1-A1-13 | G-FR1-A1-14 |  | G-FR1-A1-16 | G-FR1-A1-17 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 |  | 15 | 30 |  | 15 | 30 |  |
| Allocated resource blocks | 10 | 10 |  | 10 | 10 |  | 20 | 10 |  |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 |  | 12 | 12 |  | 12 | 12 |  |
| Modulation | QPSK | QPSK |  | QPSK | QPSK |  | QPSK | QPSK |  |
| Code rate (Note 2) | 1/3 | 1/3 |  | 1/3 | 1/3 |  | 1/3 | 1/3 |  |
| Payload size (bits) | 888 | 888 |  | 888 | 888 |  | 1736 | 888 |  |
| Transport block CRC (bits) | 16 | 16 |  | 16 | 16 |  | 16 | 16 |  |
| Code block CRC size (bits) | - | - |  | - | - |  | - | - |  |
| Number of code blocks - C | 1 | 1 |  | 1 | 1 |  | 1 | 1 |  |
| Code block size including CRC (bits) (Note 3) | 904 | 904 |  | 904 | 904 |  | 1752 | 904 |  |
| Total number of bits per slot | 2880 | 2880 |  | 2880 | 2880 |  | 5760 | 2880 |  |
| Total symbols per slot | 1440 | 1440 |  | 1440 | 1440 |  | 2880 | 1440 |  |
| NOTE 1: UL-DMRS-config-type = 1 with UL-DMRS-max-len = 1, UL-DMRS-add-pos = 1 with = 2, = 11 as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity and in-channel selectivity NOTE 3: Code block size including CRC (bits) equals to in sub-clause 5.2.2 of TS 38.212 [15]. NOTE 4: For reference channel A1-10, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N, N+10, N+20, N+30, N+40 where N={0,1,2,3,4,5,6}. NOTE 5: For reference channel A1-11, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N, N+5, N+10, N+15 where N={0,1,2,3,4,5}. NOTE 7: For reference channel A1-13, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N, N+10,N+20,..N+90 where N={0,1,2,3,...,15}. NOTE 8: For reference channel A1-14, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N,N+5,N+10,..,N+45 where N={0,1,2,3,4,5}. NOTE 10: For reference channel A1-16, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N,N+10,N+20,...,N+200 where N={0,1,2,3,4,...,25}. NOTE 11: For reference channel A1-17, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N, N+5, N+10, ...,N+100 where N={0,1,2,3,4,...,15}. |  |  |  |  |  |  |  |  |  |

Table 2: FRC parameters for NR-U reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation, in-channel selectivity [based on WF R4-1916162]

| Reference channel | G-FR1-A1-19 |  | G-FR1-A1-21 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 30 |  | 30 |  |  |  |  |  |  |
| Allocated resource blocks | 20 |  | 20 |  |  |  |  |  |  |
| CP-OFDM Symbols per slot (Note 1) | 12 |  | 12 |  |  |  |  |  |  |
| Modulation | QPSK |  | QPSK |  |  |  |  |  |  |
| Code rate (Note 2) | 1/3 |  | 1/3 |  |  |  |  |  |  |
| Payload size (bits) | 1736 |  | 1736 |  |  |  |  |  |  |
| Transport block CRC (bits) | 16 |  | 16 |  |  |  |  |  |  |
| Code block CRC size (bits) | - |  | - |  |  |  |  |  |  |
| Number of code blocks - C | 1 |  | 1 |  |  |  |  |  |  |
| Code block size including CRC (bits) (Note 3) | 1752 |  | 1752 |  |  |  |  |  |  |
| Total number of bits per slot | 5760 |  | 5760 |  |  |  |  |  |  |
| Total symbols per slot | 2880 |  | 2880 |  |  |  |  |  |  |
| NOTE 1: UL-DMRS-config-type = 1 with UL-DMRS-max-len = 1, UL-DMRS-add-pos = 1 with = 2, = 11 as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity and in-channel selectivity NOTE 3: Code block size including CRC (bits) equals to in sub-clause 5.2.2 of TS 38.212 [15]. NOTE 4: For reference channel A1-19, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N,N+5,N+10,...,N+155 where N={0,1,2,...,6}. NOTE 6: For reference channel A1-21, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N,N+5,N+10,...,N+205 where N={0,1,2,3,4,...,11}. |  |  |  |  |  |  |  |  |  |

Table 3: FRC parameters for NR-U dynamic range [based on WF R4-1916162]

| Reference channel | G-FR1-A2-7 | G-FR1-A2-8 |  | G-FR1-A2-10 | G-FR1-A2-11 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 |  | 15 | 30 |  |
| Allocated resource blocks | 10 | 10 |  | 10 | 10 |  |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 |  | 12 | 12 |  |
| Modulation | 16QAM | 16QAM |  | 16QAM | 16QAM |  |
| Code rate (Note 2) | 2/3 | 2/3 |  | 2/3 | 2/3 |  |
| Payload size (bits) | 3752 | 3752 |  | 3752 | 3752 |  |
| Transport block CRC (bits) | 16 | 16 |  | 16 | 16 |  |
| Code block CRC size (bits) | - | - |  | - | - |  |
| Number of code blocks - C | 1 | 1 |  | 1 | 1 |  |
| Code block size including CRC (bits) (Note 3) | 3768 | 3768 |  | 3768 | 3768 |  |
| Total number of bits per slot | 5760 | 5760 |  | 5760 | 5760 |  |
| Total symbols per slot | 1440 | 1440 |  | 1440 | 1440 |  |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0= 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: MCS index 16 and target coding rate = 658/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. NOTE 4: For reference channel A2-7, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N,N+10,..N+40 where N={0,1,2,3,4,5}. NOTE 5: For reference channel A2-8, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N,N+5, N+10, N+15 where N={0,1,2,3,4,5, 6, 7, 8}. NOTE 7: For reference channel A2-10, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N,N+10,N+20,..N+90 where N={0,1,2,3,...,15}. NOTE 8: For reference channel A2-11, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N,N+5,N+10,..,N+45 where N={0,1,2,3,4,5}. |  |  |  |  |  |  |

Table 4: FRC parameters for NR-U dynamic range [based on WF R4-1916162]

| Reference channel | G-FR1-A2-13 | G-FR1-A2-14 |  | G-FR1-A2-16 |  | G-FR1-A2-18 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 |  | 30 |  | 30 |  |
| Allocated resource blocks | 20 | 10 |  | 20 |  | 20 |  |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 |  | 12 |  | 12 |  |
| Modulation | 16QAM | 16QAM |  | 16QAM |  | 16QAM |  |
| Code rate (Note 2) | 2/3 | 2/3 |  | 2/3 |  | 2/3 |  |
| Payload size (bits) | 7424 | 3752 |  | 7424 |  | 7424 |  |
| Transport block CRC (bits) | 24 | 16 |  | 24 |  | 24 |  |
| Code block CRC size (bits) | - | - |  | - |  | - |  |
| Number of code blocks - C | 1 | 1 |  | 1 |  | 1 |  |
| Code block size including CRC (bits) (Note 3) | 7448 | 3768 |  | 7448 |  | 7448 |  |
| Total number of bits per slot | 11520 | 5760 |  | 11520 |  | 11520 |  |
| Total symbols per slot | 2880 | 1440 |  | 2880 |  | 2880 |  |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos1 with l0= 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: MCS index 16 and target coding rate = 658/1024 are adopted to calculate payload size. NOTE 3: Code block size including CRC (bits) equals to K' in sub-clause 5.2.2 of TS 38.212 [15]. NOTE 4: For reference channel A2-13, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N, N+10, N+20,...,N+200 where N={0,1,2,3,4,...,15}. NOTE 5: For reference channel A2-14, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N, N+10, ...,, N+90 where N={0,1,2,3,4,...,15}. NOTE 7: For reference channel A2-16, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N, N+5, N+10,..., N+12, N={0,1,2,...,9} NOTE 9: For reference channel A2-18, the allocated RB’s are uniformly spaced over the channel bandwidth at RB index N, ,N+5, N+10, N+20,..., N+205 where N={0,1,2,3,4,...,11}. |  |  |  |  |  |  |  |

# 3.	Conclusion

In this contribution, the following proposals have been presented:

Proposal 1: No support for interlacing for NR-U for 60 kHz SCS.  Remove above listed FRCs.

Proposal 2: Align FRC interlacing design according to TS 38.211

# 4.	References

[1] 		R4-1916162, “WF on NR-U BS RF RX FRC”, ZTE