3GPP TSG-RAN WG4 Meeting #94-e	R4-2001731

Electronic Meeting, February 24- March 6, 2020

Agenda Item:	8.1.1.5

Source:	Futurewei

Title:	Sync raster design for NR-U

Document for:	Approval

# Introduction

In RAN4#93, a draft CR listing the synchronization raster points for 38.104 was endorsed [1]. It was expected that an accompanying CR for 38.101-1 would be provided for RAN4#94. One open issue was the type of “SS block pattern” to be listed in the table. This document discusses the “SS block pattern” and then provides a TP for 38.101-1 that captures the appropriate value.

# Discussion

In 38.104 and 38.101-1, Table 5.4.3.3-1 has a column identifying the “SS block pattern”. This pattern is used in 38.213 subclause 4.1 as summarized below.

Table 1. Summary of SS block pattern in 38.213

| SCS, kHz | Pattern | Formula | Channel access | Condition |
| --- | --- | --- | --- | --- |
| 15 | Case A | 2,8+14⋅n | Without shared spectrum | n=0,1, &fc≤3GHz0,1,2,3, &fc>3GHz |
|  |  |  | With shared spectrum | n=0,1,2,3,4 |
| 30 | Case B | 4,8,16,20+28⋅n |  | n=0, &fc≤3GHz0,1, &fc>3GHz |
| 30 | Case C | 2,8+14⋅n | Without shared spectrum and paired spectrum | n=0,1, &fc≤3GHz0,1,2,3, &fc>3GHz |
|  |  |  | Without shared spectrum and unpaired spectrum | n=0,1, &fc≤2.4GHz0,1,2,3, &fc>2.4GHz |
|  |  |  | With shared spectrum | n=0,…,9 |

From the summary in Table 1 and the design parameters for the sync raster in [1], for 30 kHz SCS, the SS block pattern that should be used is “Case C” because “Case C” is the only 30 kHz SCS that supports shared spectrum channel access. Until further confirmation, square brackets can be used around the value.

Note due to the e-meeting format, a formal CR based on

Proposal 1: a 38.104 CR can be based on the endorsed CR (R4-1915982) and the addition of “[Case C]” for the “SS block pattern” column in band n46.

An accompanying TP for 38.101-1 capturing the sync raster for 30 kHz SCS is presented below.

Proposal 2: a 38.101-1 CR for the sync raster for 30 kHz SCS can be based on the TP below.

One observation is that RAN1 has specified a 15 kHz SCS SSB pattern, as indicated in Table 1. However, in RAN4, no sync raster for 15 kHz SCS was agreed for standalone operation based on chairman notes in RAN4#93.

# Conclusions

The type of SS block pattern is discussed in this contribution.

Proposal 1: a 38.104 CR can be based on the endorsed CR (R4-1915982) and the addition of “[Case C]” for the “SS block pattern” column in band n46.

An accompanying TP for 38.101-1 capturing the sync raster for 30 kHz SCS is presented below.

Proposal 2: a 38.101-1 CR for the sync raster for 30 kHz SCS can be based on the TP below.

# References

R4-1915982, “Draft CR for Sync raster design for NR-U in 38.104”, Futurewei, RAN4#93, Nov. 18-22, 2019.

# Appendix for 38.101-1

#### 5.4.3.3	Synchronization raster entries for each operating band

The synchronization raster for each band is give in Table 5.4.3.3-1. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 5.4.3.3-1.

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15 kHz | Case A | 5279 – <1> – 5419 |
| n2 | 15 kHz | Case A | 4829 – <1> – 4969 |
| n3 | 15 kHz | Case A | 4517 – <1> – 4693 |
| n5 | 15 kHz | Case A | 2177 – <1> – 2230 |
|  | 30 kHz | Case B | 2183 – <1> – 2224 |
| n7 | 15 kHz | Case A | 6554 – <1> – 6718 |
| n8 | 15 kHz | Case A | 2318 – <1> – 2395 |
| n12 | 15 kHz | Case A | 1828 – <1> – 1858 |
| n14 | 15 kHz | Case A | 1901 – <1> – 1915 |
| n18 | 15 kHz | Case A | 2156 – <1> – 2182 |
| n20 | 15 kHz | Case A | 1982 – <1> – 2047 |
| n25 | 15 kHz | Case A | 4829 – <1> – 4981 |
| n28 | 15 kHz | Case A | 1901 – <1> – 2002 |
| n29 | 15 kHz | Case A | 1798 – <1> – 1813 |
| n30 | 15 kHz | Case A | 5879 – <1> – 5893 |
| n34 | 15 kHz | Case A | 5030 – <1> – 5056 |
| n38 | 15 kHz | Case A | 6431 – <1> – 6544 |
| n39 | 15 kHz | Case A | 4706 – <1> – 4795 |
| n40 | 15 kHz | Case A | 5756 – <1> – 5995 |
| n41 | 15 kHz | Case A | 6246 – <3> – 6717 |
|  | 30 kHz | Case C | 6252 – <3> – 6714 |
| n462 | 30 kHz | [Case C] | 8993 - <1> - 9530 |
| n48 | 30 kHz | Case C | 7884 – <1> – 7982 |
| n50 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n51 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n65 | 15 kHz | Case A | 5279 – <1> – 5494 |
| n66 | 15 kHz | Case A | 5279 – <1> – 5494 |
|  | 30 kHz | Case B | 5285 – <1> – 5488 |
| n70 | 15 kHz | Case A | 4993 – <1> – 5044 |
| n71 | 15 kHz | Case A | 1547 – <1> – 1624 |
| n74 | 15 kHz | Case A | 3692 – <1> – 3790 |
| n75 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n76 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n77 | 30 kHz | Case C | 7711 – <1> – 8329 |
| n78 | 30 kHz | Case C | 7711 – <1> – 8051 |
| n79 | 30 kHz | Case C | 8480 – <16> – 8880 |
| n90 | 15 kHz | Case A | 6246 – <1> – 6717 |
|  | 30 kHz | Case C | 6252 – <1> – 6714 |
| n91 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n92 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n93 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n94 | 15 kHz | Case A | 3584 – <1> – 3787 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [8] NOTE 2: The following GSCN are allowed for operation in band n46: {GSCN = 8996, 9010, 9024, 9038, 9051, 9065, 9079, 9093, 9107, 9121, 9218, 9232, 9246, 9260, 9274, 9288, 9301, 9315, 9329, 9343, 9357, 9371, 9385, 9402, 9416, 9430, 9444, 9458, 9472, 9485, 9499, 9513}. |  |  |  |