3GPP TSG RAN WG4 Meeting #94-e	R4-2001848

Electronic Meeting, February 24-March 06, 2020

Agenda Item:	8.1.4.8

Source: 	Ericsson

Title:	Analysis of BWP switching requirement due to consistent UL failure

Document for:	Discussion

# Introduction

In RAN4#93 meeting the WF on RRM requirements for NR-U was approved [1]. In [2] it was proposed to specify the BWP switching requirements triggered due to consistent UL LBT failure. There was no agreement but the following was captured in the WF on the RRM [1]:

UL BWP switching triggered by consistent UL LBT failure

FFS whether to introduce RRM requirements for UL BWP switching triggered by consistent UL LBT failure

In this paper we provide analysis BWP switching requirements triggered due to consistent UL LBT failure.

# Background: UL BWP switching due to consistent UL LBT failure

According to the latest version of the RAN2 running stage-2 CR to TS 38.300 the following has been captured regarding the UE behaviour upon reaching consistent uplink LBT failures. As noted in the highlighted (yellow shaded) text that under consistent uplink LBT failures on SpCell (i.e. PCell or PSCell) the UE switched to another UL BWP i.e. performs UL BWP switching on another RB sets and sends RACH to report the failure. Furthermore, it is up to select the UL BWP if multiple UL BWPs are configured. RAN2 is defining the UE behaviour in the stage-3 MAC specification, TS 38.321.

When the UE detects consistent uplink LBT failures, it takes actions as specified in TS 38.321 [6]. The detection is per BWP and based on all uplink transmissions within this BWP. When consistent uplink LBT failures are detected on SCell(s), the UE reports this to the corresponding gNB (MN for MCG, SN for SCG) via MAC CE on a different serving cell other than the SCell (s) where the failures were detected. If no resources are available to transmit the MAC CE, a Scheduling Request (SR) can be transmitted by the UE. When consistent uplink LBT failures are detected on SpCell, the UE switches to another UL BWP with configured RACH resources on that cell, initiates RACH, and reports the failure via MAC CE. When multiple UL BWPs are available for switching, it is up to the UE implementation which one to select. For PSCell, if consistent uplink LBT failures are detected on all the UL BWPs with configured RACH resources, the UE declares SCG RLF and reports the failure to the MN via SCGFailureInformation. For PCell, if the uplink LBT failures are detected on all the UL BWP(s) with configured RACH resources, the UE declares RLF.

In the latest version of the RAN2 running CR to TS 38.331 the procedure to configure the UE with paraneters comprising an UL LBT failure detection timer and a maximum number of UL LBT failure count, which trigger consistent uplink LBT failures is being specified, as shown below:

#### –	LBT-FailureRecoveryConfig

The IE LBT-FailureRecoveryConfig-r16 is used to configure the parameters used for detection of consistent uplink LBT failures for operation with shared spectrum channel access, as specified in TS 38.321 [3].

LBT-FailureRecoveryConfig information element

LBT-FailureRecoveryConfig-r16 ::=            SEQUENCE {

lbt-FailureInstanceMaxCount                         ENUMERATED {ffsValue}      OPTIONAL,   -- Need

lbt-FailureDetectionTimer                           ENUMERATED {ffsValue}      OPTIONAL,   -- Need

}

LBT-FailureRecoveryConfig field descriptions

lbt-FailureDetectionTimert Timer for consistent uplink LBT failure detection (see TS 38.321 [3]).

lbt-FailureInstanceMaxCount This field determines after how many consistent uplink LBT failure events the UE triggers uplink LBT failure recovery (see TS 38.321 [3]).

# UL BWP switching requirements

Based on the RAN2 agreements as described in section 2, it is evident that upon consistent UL LBT failures on SpCell the UE shall trigger the UL BWP switching if configured with RACH resources on that cell. Therefore, RAN4 needs to specify the UL BWP switching requirements when consistent UL LBT failures on SpCell. The UL BWP switching is analogous to DCI-based or timer-based UL BWP switching. Therefore the existing UL BWP switching delay and interruption defined for DCI- or timer-based UL BWP switching in section 8.6.2 of TS 38.133 can be reused. Also the interruption requirements defined for DCI- or timer-based UL BWP switching in sections 8.2.2.2.5 for CA and 8.2.4.2.5 for NR-DC of TS 38.133 can be reused.

- Summary

In this paper we have analysed the impact of triggering the BWP switching due to consistent UL LBT failure. We have proposed that BWP switching requirements triggered due to consistent UL LBT failure UE are specified to ensure the UE does BWP switching within the specified delay.

- Observation # 1: Upon consistent UL LBT failures on SpCell the UE triggers the UL BWP switching if configured with RACH resources on that cell.

- Proposal # 1: Specify UL BWP switching delay and interruption requirements upon detecting consistent UL LBT failures on SpCell provided that the UE is configured with RACH resources on that cell.

- Proposal # 2: The existing requirements defined for DCI-based or timer-based UL BWP switching are reused for defining the UL BWP switching requirements upon detecting consistent UL LBT failures on SpCell.

CRs to introduce the corresponding requirements is provided in [3-4].

# References

- R4-1915777, WF on NR-U RRM Requirements, Ericsson.

- R4-1914874, Discussion on Active BWP switching requirements in NR-U, Huawei, HiSilicon.

- R4-2001849, BWP switching requirement due to consistent UL failure in 38.133, Ericsson.

- R4-2001850, Interruption due to BWP switching at consistent UL failure in 36.133, Ericsson.