# 3GPP TSG-RAN WG4 Meeting #94-e                                                       R4-2001869

# Electronic Meeting, February 24-March 06, 2020

Source:		Ericsson

Title:			IAB MT IBE requirement including carrier leakage and IQ image

Agenda item:		8.5.4.2.1

Document for:	Approval

# Introduction

For IAB MT backhaul connection to IAB parent, there could be a scenario where the UE uplink traffic and IAB MT uplink traffic could be FDM:ed at IAB parent. This then motivate the discussion of the IBE requirement on IAB MT.

In this paper, we present our view on IBE requirement.

# Discussion

The In-band emission is to specify the noise level at the non-allocated RB for a UE when BS aggregate multiple UE signal, the interference from UE:es on the non-allocated noise should be controlled and not result in the demod issue for the allocated RB from other UE. The IBE requirement has below components:

- General mask

- IQ image

- Carrier leakage

## General mask

The general mask consists of the 3 components, namely

- The general mask which guards the generic level where maximum # of UE aggregate at the BS receiver but still 64 QAM can be demoded. The formula is -25 -10.log10NRBLCRB,

- The noise floor related to the modulation scheme and tightening for the remote non-allocated RB with some change slope, the formula is 20.log10EVM- 5.|∆RB-1LCRB

- The absolute level at the low output power, the formula is -55.1dBm-PRB

As the 64QAM is highest modulation scheme required for IAB MT, it still make sense to reuse the item 1 and 2 above. For the item 3, as it relates to the emission noise floor when relative req at item 1 and 2 goes too low, for IAB MT, the new value needs to be discussed. As the chapter 2.3 discussion indicates that only one IAB MT transmit power to be verified, we recommend removing the item 3 in generic mask.

Proposal-1: Reuse the item 1 and 2 for relative level of generic mask.

Proposal-2: the absolute level could to be removed.

## Carrier leakage

As the IAB MT and IAB DU may be configured with different # of carriers or the different bandwidth of one carrier. This will result in the different operating Bandwidth for IAB MT and IAB DU. However, the basic assumption for maximum power of the MT and DU for the shared transceiver architecture is that they have the same maximum power and MT and DU operate in TDM mode. This will mean that if there are any # carrier configured differently for IAB MT and IAB DU, there will be PSD difference on IAB MT carrier and IAB DU carrier.

Observation #1: There will be x dB PSD difference between IAB MT and DU carrier for shared transceiver if IAB MT and DU were configured with different # of carrier.

Most likely for the IAB node, the bandwidth configured for IAB MT would be greater than the channel bandwidth of IAB DU as IAB MT will be responsible for backhauling of the IAB DU traffic and maybe also the for the traffic of other IAB child node in a chain.  In Figure 1 case#1 shows LO leakage power when IAB MT and DU operate in TDM mode on a shared transceiver, to simplify the illustration, the IAB DU carrier and IAB MT carrier are showed in FDM way in Figure 1, in reality the IAB DU can IAB MT can be configured as the same carrier (in-band operation). IAB MT power could be further power controlled to Y dB down relative to the maximum transmit PSD but the LO leakage level will not be changing while the IAB MT transmit power decrease due to the LO leakage power will be relative to delivered maximum transmitted power of the PA and as such, the LO leakage for IAB MT will be fixed irrespective what is IAB MT transmitted power. Case#2 in Figure also shows another example where LO leakage will be relative to the maximum PSD of the IAB MT transmitted power.

Observation #2: IAB MT LO leakage power only relative to the maximum transmitted PSD of IAB DU/MT.

Figure 1: LO leakage for the IAB MT and DU for a shared transceiver

As LO leakage only relative to the maximum configured PSD power level for either IAB DU or IAB MT, the LO leakage will relate to both IAB MT and IAB DU. From test and requirement perspective, if IAB MT is configured with maximum declared power, such configuration should cover the both case#1 and case#2 in Figure 1.

Proposal-3: LO leakage power should be specified relative to IAB MT output power when IAB MT is configured with the maximum declared power.

As IAB MT antenna array size will be greater compared to a normal UE, the correlation level of the LO leakage signal from different transceiver branch of one antenna element need to be discussed. Unlike the wanted signal which will be beamformed with a gain of the 10log#antenna element in a predefined direction, the LO leakage signal may not be beamformed in the same way as the wanted signal. If LO leakage signals are fully uncorrelated, then LO signal level will be (10log#antenna element) dB less than wanted signal only because of the beamforming gain. In the worst case, if LO leakage signal happens to be fully correlated and also beam formed in the same direction of the wanted signal, the LO leakage relative level to wanted signal then will be the same measured at TAB or measured at RIB, so the starting point of LO leakage level at TAB is ok to reuse for RIB level. The additional suppression of the LO leakage due to uncorrelated signal additional could be implementation specific.

Proposal-4: the additional suppression due to the uncorrelated LO signal can be implementation specific and so the correlation characteristic of LO signal discussion can be skipped.

## IQ image

The IQ image level could benefit from the additional suppression due to beamforming on top of the RF mixer IQ image rejection. As IQ image frequency position would be known when RB is scheduled by IAB parent, it can be calibrated in a way that not beamformed as the wanted signal. So additional suppression could be achieved but again this is implementation specific so reusing the IQ image requirement from UE seems ok.

## Proposal on the IBE requirement on IAB MT

As the note 1 in 38.101-2 indicate a greater of PRB -25 dB and sum of (general, IQ image, carrier leakage) for IBE requirement, so basically the generic mask will have limited impact on IBE performance for high PRB. From the above discussion, the LO leakage is fixed level relative to the maximum configured PSD power on IAB DU or IAB MT, the IBE requirement is correct when IAB MT set its maximum declared power as the same as IAB DU, this is corresponding to the long distance backhaul where the IAB MT is set to its maximum transmission power. When IAB is deployed near the IAB parent, due to its limited dynamic range and also higher SNR expected at IAB parent for higher modulation, better IBE requirement would not help to aggregate UE traffic with IAB MT traffic, in such a case, the test on other IAB MT transmit power would not be needed. As such, we think IBE requirement is only relevant to IAB MT for maximum transmitted power and no need to test on other IAB MT transmitted power.

The proposed requirement is as below:

#### x.y.z	In-band emissions for IAB MT

##### x.y.z.1	General

The in-band emission is defined as the average across 12 sub-carriers and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the IAB MT UE output power in a non–allocated RB to the IAB MT UE output power in an allocated RB.

The basic in-band emissions measurement interval is identical to that of the EVM test.

The requirement is verified with the test metric of In-band emission (Link=TX beam peak direction, Meas=Link angle). This requirement is verified with the maximum declared IAB MT power configured.

##### x.y.z.2	In-band emissions for IAB MT

The relative in-band emission shall not exceed the values specified in Table x.y.z.2-1 for IAB MT.

Table x.y.z.2-1: Requirements for in-band emissions for IAB MT

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | max-25 -10.log10NRBLCRB, 20.log10EVM- 5.¦∆RB-1LCRB, -55.1dBm-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 | Maximum declared Output power > 27 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | Output power ≤ 27 dBm |  |
| Carrier leakage | dBc | -25 | Output power > 17 dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | 4 dBm ≤ Output power ≤ 17 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of (PRB - 25 dB) and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if NRB is odd, or in the two RBs immediately adjacent to the DC frequency if NRB is even but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see Figure 5.3.3-1). NOTE 7: NRB is the Transmission Bandwidth Configuration (see Figure 5.3.3-1). NOTE 8: EVM s the limit for the modulation format used in the allocated RBs. NOTE 9: RB is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. RB= 1 or RB= -1 for the first adjacent RB outside of the allocated bandwidth). NOTE 10: PRB is the transmitted power per allocated RB, measured in dBm. NOTE 11: All powers are EIRP in beam peak direction. |  |  |  |  |

# Conclusions

In this contribution, IBE requirement together with carrier leakage and IQ image is investigated with below observations and proposals:

Proposal-1: Reuse the item 1 and 2 for relative level of generic mask.

Proposal-2: the absolute level could to be removed.

Observation #1: There will be x dB PSD difference between IAB MT and DU carrier for shared transceiver if IAB MT and DU were configured with different # of carrier.

Observation #2: IAB MT LO leakage power only relative to the maximum transmitted PSD of IAB DU/MT.

Proposal-3: LO leakage power should be specified relative to IAB MT output power when IAB MT is configured with the maximum declared power.

Proposal-4: the additional suppression due to the uncorrelated LO signal can be implementation specific and so the correlation characteristic of LO signal discussion can be skipped.

# References

- R4-1916161, IAB Ad Hoc meeting minutes, Qualcomm Incorporated