3GPP TSG RAN WG4 Meeting #94-e	R4-2001882

Electronic Meeting, February 24-March 06, 2020

Agenda Item:	8.5.4.2.1

Source: 	Ericsson

Title:	TP to TR 38.xxx: OTA unwanted emissions

Document for:	Approval

- Introduction

After RAN4#92bis the IAB TR [1] is email approved. Section 2 provides text proposal on 9 OTA unwanted emissions for the IAB TR 38.xxx (Integrated access and backhaul radio transmission and reception).

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

## 9.7	OTA unwanted emissions

Detailed structure of the subclause is TBD.

#### 9.7.1 ACLR for IAB MT of IAB type 2-O

ETSI harmonized standard [3] set the ACLR requirement on BS, the IAB MT can operate on downlink time slot to transmit, then it is not possible to specify a different ACLR than BS ACLR to comply with ETSI spec [3], or in the other words, specifying a different number than BS ACLR will forbid the IAB MT operation on downlink time slot. This will significantly reduce the IAB deployment flexibility. As such, it is proposed the same ACLR as IAB DU for IAB MT.

#### 9.7.2 Out of band unwanted emission for IAB MT of IAB type 2-O

As OBUE is power relevant and IAB MT declare its transmission power as the same as IAB DU, hence the same OBUE requirement of the IAB DU could be reused on IAB MT. As IAB is fixed node as the same as BS, the OBUE requirement for IAB MT also is regional related.

With the same OBUE on IAB MT and IAB DU, it means the Operation Band Unwanted Emission Masks for NR BS is reused based on the same principles as in 38.817-2, applicable to all IAB classes:

-	A fixed mask, same as FCC limit applies to the highest IAB power levels.

-	Variable masks scaled with power level apply to IAB with power level below certain thresholds:

-	One mask for 24.25 – 33.4 GHz, based on ACLR = 28 dBc.

-	One mask for 37 – 52.6 GHz, based on ACLR = 26 dBc.

-	Masks are limited to an absolute lowest emission limit of -20 dBm as in the WP5D LS response [10]

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-1913332, “Updated IAB TR”, Samsung, RAN4#93