3GPP TSG RAN WG4 Meeting #94-e	R4-2001885

Electronic Meeting, February 24-March 06, 2020

Agenda Item:	8.5.4.2.2

Source: 	Ericsson

Title:	TP to TR 38.xxx: OTA In-band blocking

Document for:	Approval

- Introduction

After RAN4#92bis the IAB TR [1] is email approved. Section 2 provides text proposal on 10 OTA In-band blocking for the IAB TR 38.xxx (Integrated access and backhaul radio transmission and reception).

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

#### 10.5.3 In-band blocking for IAB MT of IAB type 2-O

Traditionally the in-band blocker is set according to the 99.99% probability of interferer cdf curve. In[6], it is recommended that between 99% and 99.9% could be considered due to the NR OFDMA scheme does not suffer greatly as for WCDMA. Also, the fact of the blocking only happens with the simultaneous receiving a weak wanted signal and strong interfere decide that for IAB node deployment, the in-band blocking could happen more likely at the cell edge where weak wanted signal and strong interfere could be received simultaneously.

Figure 10.5.3-1 to Figure 10.5.3-2 is the simulation results on blocker conducted level based on layout2. For FR2, in Figure 2, it can be observed that the blocker level is -67.5 dBm @99% point at RIB assuming a 3dBi antenna element gain of IAB MT for 40m physical separation distance. Also, in Figure 10.5.3-2, it can be observed that the blocker level is -75dBm @99% point at RIB assuming a 3dBi antenna element gain of IAB MT for 50m physical separation distance.

For in-band blocking requirement of LA BS OTA FR2, EISREFSENS_50M is an integer value in the range -96 to -119 dBm, then the blocker level is the EISREFSENS_50M + 3 + 33 = -119 + 3+ 33= -83 dBm, so the blocker level is between -60dBm and -83 dBm depending different antenna gain of BS. For antenna gain of 24 dBi, this could be corresponding to the blocking level of -74 dBm [6].

For in-band blocking requirement of MR BS OTA FR2, EISREFSENS_50M is an integer value in the range -91 to -114 dBm, then the blocker level is the EISREFSENS_50M + 3 + 33 = -114 + 3+ 33= -78 dBm, so the blocker level is between -55dBm and -78 dBm depending different antenna gain of BS. For antenna gain of 24 dBi, this could be corresponding to the blocking level of -74 dBm [6].

Though AAS architecture may be different for different product and so the different antenna size (gain) before LNA, the received blocked level (conducted) will be fixed for one transceiver design. This means that increasing antenna gain before analog LNA will reduce the tolerated blocker level. On the other hand, to have the same blocker level as the conducted level, the antenna subarray size will be minimal and so is the same as one antenna element. This will drive the cost and size of antenna panel. Therefore, how to choose the AAS design need to be tailored to the different deployment scenario to benefit the performance and cost.

The blocker level at EIRP level according to system simulation will be different for different separation distance, to have a similar blocking performance with current BS spec, the separation distance should be 50m for FR2.

Figure 10.5.3-1: incoming leakage power for FR2 with min 40m/50m/60m distance to IAB donor

Figure 10.5.3-2: Zoom in 99% percentile point of Figure 1

#### 10.5.4 in-band blocking for IAB MT of IAB type 1-O

Figure 10.5.3-3 to Figure 10.5.3-4 is our simulation on blocker conducted level based on layout2. For FR1, in Figure 10.5.3-4 the blocker level is -55dBm @99% point at RIB assuming a 3dBi antenna element gain of IAB MT.

For FR1, the in-band blocking requirement is – 43- ΔOTAREFSENS, where ΔOTAREFSENS is related to the beam pattern of the AAS array architecture, for example, it could be in a range of 5dB to 16 dB for 64 antenna elements depending how subarray is defined. So the blocker level is within -48 dBm to -59 dBm depending AAS architecture.

To have a similar blocking performance with current BS spec, the separation distance should be 40m for FR1.

Figure 10.5.3-3: incoming leakage power for FR1 with min 40m distance to IAB donor

Figure 10.5.3-4: Zoom in 99% percentile point for Figure 4

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-1913332, “Updated IAB TR”, Samsung, RAN4#93