3GPP TSG RAN WG4 Meeting #94-e	R4-2001894

Electronic Meeting, February 24-March 06, 2020

Agenda Item:	8.5.4.2.1

Source: 	Ericsson

Title:	TP to TS 38.174: OTA output power dynamics

Document for:	Approval

- Introduction

After RAN4#92bis the IAB TS [1] is email approved. Section 2 provides text proposal on 9 OTA output power dynamics for the IAB TS 38.174 (Integrated access and backhaul radio transmission and reception).

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

## 9.4	OTA output power dynamics

### 9.4.1	General

The requirements in subclause 9.4 apply during the transmitter ON period. Transmit signal quality (as specified in subclause 9.6) shall be maintained for the output power dynamics requirements.

The OTA output power requirements are directional requirements and apply to the beam peak directions over the OTA peak directions set.

### 9.4.2	OTA RE power control dynamic range

#### 9.4.2.1	General

The OTA RE power control dynamic range is the difference between the power of an RE and the average RE power for a IAB DU at maximum output power (Pmax,c,EIRP) for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

#### 9.4.2.2	Minimum requirement for IAB DU of IAB type 1-O

The OTA RE power control dynamic range is specified the same as the conducted RE power control dynamic range requirement for IAB DU of IAB type 1-H in table 6.3.2.2-1.

### 9.4.3	OTA total power dynamic range

#### 9.4.3.1 General

The OTA total power dynamic range for IAB DU is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

The OTA power dynamic range for IAB MT is the difference between declared IAB MT maximum power and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE 1:	The upper limit of the OTA total power dynamic range for IAB DU is the IAB DU maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all RBs. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbol carries PDSCH and not contain RS or SSB.

The upper limit of the OTA total power dynamic range for IAB MT is the IAB MT maximum carrier EIRP (Pmt,max,c,EIRP) when transmitting on all RBs. The lower limit of the OTA total power dynamic range is the average EIRP for all RB transmission in the same direction using the same beam. The OFDM symbol carries PUSCH.

### 9.4.3.2 Minimum requirement for IAB DU of IAB type 1-O

OTA total power dynamic range minimum requirement for BS type 1-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified for the conducted requirement for BS type 1-C and BS type 1-H in table 6.3.3.2-1.

#### 9.4.3.3	Minimum requirement for IAB DU of IAB type 2-O

OTA total power dynamic range minimum requirement for BS type 2-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified in table 9.4.3.3-1.

Table 9.4.3.3-1: Minimum requirement for BS type 2-O total power dynamic range

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | OTA total power dynamic range (dB) |  |  |  |
| 60 | 18.1 | 21.2 | 24.2 | N.A |
| 120 | 15.0 | 18.1 | 21.2 | 24.2 |

#### 9.4.3.4 Minimum requirement for IAB-MT of IAB type 2-O

OTA power dynamic range minimum requirement for IAB when IAB is subject to power control from another IAB node is specified such as for each NR carrier it shall be larger than or equal to the levels specified in table 9.4.3.4-1.

Table 9.4.3.4-1: Minimum requirement for IAB MT of IAB Type 2-O power dynamic range

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | OTA power dynamic range (dB) |  |  |  |
| 60 | [5] | [5] | [5] | N.A |
| 120 | [5] | [5] | [5] | [5] |

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-1916000, “Updated IAB TS”, Qualcommn Inc.