3GPP TSG-RAN WG4 Meeting #94-e	R4-2001974

Online, 24th February – 6th March 2020

Source:	Ericsson, US Cellular

Title:	TP for TR 37.716-21-11: Including band combinations 2-48_n12

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.716-21-11 to add DC_2A-48A_n12A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_2-48_n12

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-48_n12 | CA_2-48 | n12 | No |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-48A_n12A | DC_2A_n12A DC_48A_n12A | CA_2A-48A | n12 |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_2-48_n12, the TIB,c and RIB,c values are reused from LTE combination CA_2-13-48, and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n12 | 2 | 0.6 |
|  | 48 | 0.3 |
|  | n12 | 0.8 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-48_n12 | 2 | 0.2 |
|  | 48 | 0.5 |
|  | n12 | 0 |

### 5.1.x.4	REFSENS requirements

Co-existence analysis from DC_2_n12 shows that there is no impact from DC_2_n12 UL to Band 48 DL.

Co-existence analysis from DC_48_n12 shows that there is no impact from DC_48_n12 UL to Band 2 DL.

---End of changes---

# Reference

[1]		RP-192815, “Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” Huawei, HiSilicon