3GPP TSG-RAN WG4 Meeting #94	R4-2002121

E-meeting, 24 Feb. – 6 Mar., 2020

Agenda item:	8.11.2.2

Source: 	Qualcomm Incorporated

Title: 	RRM requirements for SCell BFD, CBD and BFR

Document for:	Discussion

# Introduction

During the last meeting [1], RAN4 made several agreements regarding SCell BFD and BFR:

Agreements - RS within a deactivated SCC is implicitly configured as the BFD-RS for another activated SCell: - Option-1: UE is not required to perform BFD on RS within a deactivated SCC which is implicitly configured as the BFD-RS for another activated SCell - Option-2: FFS - RS within an activated SCC is implicitly configured as the BFD-RS for another deactivated SCell: - Option-1: UE is not required to perform BFD on RS within an activated SCC which is implicitly configured as the BFD-RS for another deactivated SCell - Option-2: FFS - When more than 2 BFD-RSs are transmitted on a CC for current SCell and (implicitly configured for) other SCell - Option-1: UE is allowed to only perform BFD on BFD-RSs for current SCell. - Option-2: FFS Agreements - RRM core requirement impact by BFD on SCell - The evaluation periods for SSB/CSI-RS based BFD on SCell are same as those of SSB/CSI-RS based BFD on PCell/PSCell. Detailed requirement captured in Section 8.5.3/4 shall be reused for SCell BFD. - FFS whether to introduce a sharing factor for BFD and CBD measurement period - RRM core requirement impact by CBD on SCell - Option-1: The evaluation periods for SSB/CSI-RS based CBD on SCell are same as those of SSB/CSI-RS based CBD on PCell/PSCell. Detailed requirement captured in Section 8.5.5/6 shall be reused for SCell CBD. - FFS whether to introduce a sharing factor for BFD and CBD measurement period - Step-1 for BFRQ on SCell with DL only: UE reports beam failure event through a dedicated SR like PUCCH resource - Option-1: RAN4 requirement is defined for step 1 of BFR, UE reports beam failure event through a dedicated SR like PUCCH resource, in SCells with DL only. - Option-2: No RRM core requirement impact by SCell BFRQ mechanism - Step-2 for BFRQ on SCell with DL only: UE conveys new beam information and failed CC index(es) via MAC-CE - Option-1: FFS Defining requirement for step 2 - Option-2: No RRM core requirement impact by SCell BFRQ mechanism - UE Capability of Number of SCells for BFR: - The maximum number of SCell for which UE performs BFR is a UE capability - For BFD and CBD requirements in FR2, up to N serving cell is assumed to perform BFR procedure for one FR2 band: - Option-1: N=1 - Other options FFS

We discuss these aspects in this meeting.

# BFD in Deactivated SCells

We don’t think that UE shouldbe required to perform BFD on RS within a deactivated SCC which is implicitly configured as the BFD-RS for another activated SCell. Similarly, UE is not required to perform BFD on RS within an activated SCC which is implicitly configured as the BFD-RS for another deactivated Cell.

Besides, when more than 2 BFD-RSs are transmitted on a CC for current SCell and implicity configured for SCell, UE is required to only perform BFD on BFD-RSs for current SCell.

Proposal 1: UE is not required to perform BFD on RS within a deactivated SCC which is implicitly configured as the BFD-RS for another activated SCell

Proposal 2: UE is not required to perform BFD on RS within an activated SCC which is implicitly configured as the BFD-RS for another deactivated SCell.

Proposal 3: When more than 2 BFD-RSs are transmitted on a CC for current SCell and implicity configured for SCell, UE is required to only perform BFD on BFD-RSs for current SCell.

# UE capability regarding the number of SCells for BFR

The reference signals transmitted across different SCells in intra-band FR2 should be QCLed w.r.t. type D. Hence, it should be sufficient for the UE to perform BFD, CBD and BFR for only one SCell within one band.

Proposal 4: For BFD and CBD requirements in FR2, only one serving cell is assumed to perform BFR procedure for one FR2 band

# RRM Core Requirements for BFD and CBD in SCells

During the last meeting [1], RAN4 discussed if a sharing factor needs to be introduced in the evaluation periods of SCell BFD and CBD. If proposal 4 gets agreed, then UE only needs to perform BFD/CBD for up to one SCell in FR2. In FR2, we currently have only two different bands: 28 GHz and 39 GHz, UE will need to perform BFD/CBD for at most two SCells in FR2. Hence, a sharing factor does not need to be introduced for BFD/CBD in FR2 if proposal 4 gets agreed.

However, there might be multiple bands in FR1. The evaluation periods for BFD and CBD need to be scaled according to the number of SCells so that UE does not have to simultaneously perform BFD/CBD across many SCells.

Observation 1: If proposal 4 gets agreed, then UE only needs to perform BFD/CBD for up to one SCell in FR2.

Proposal 5: RAN4 supports introducing a sharing factor in the BFD and CBD evaluation period for SCells in FR1.

- The sharing factor is proportional to the number of SCells for which UE is performing BFD/CBD.

- Note: This proposal assumes that UE is performing BFR for only one serving cell in each FR2 band.

# RRM Core Requirements for BFR in SCells with DL only

In Rel-15, RAN4 has defined core requirements for BFD and CBD. It has not defined core requirements for BFR transmission. However, it has defined performance tests for BFR transmission because BFR is the mechanism that verifies whether UE is performing BFD and CBD properly or not.

In Rel-16, 3GPP has defined conveying SCell BFR through PUCCH of PSCell. Previous meeting’s agreement shows that there are two steps associated with it. In the first step, UE reports beam failure event that has occurred in SCells with DL only through a dedicated SR like PUCCH resource. In the second step, UE conveys new beam information and failed CC indices via MAC-CE.

The first step is, in theory, like PRACH based BFR transmission in Rel-15. RAN4 performance requirements only tests UE’s ability to transmit Msg1 to convey BFR in Rel-15. It does not test UE’s ability to communicate subsequent RACH messages with gNB. Following the same logic, Rel-16 can just define RRM core requirements for step 1 of SCell-BFRQ, i.e., where UE reports beam failure event through a dedicated SR like PUCCH resource.

Proposal 6: RAN4 defines requirement for step 1 of BFR, where UE reports beam failure event through a dedicated SR like PUCCH resources, in SCells with DL only.

- No RRM core requirement is defined for UE conveying new beam information and failed CC indices via MAC-CE.

## 5.1 Details of SCell BFR requirements

RAN1 has introduced following texts in 38.213 regarding conveying BFR event through PUCCH.

Text from 38.213 - A UE can be provided, by schedulingRequestIDForBFR, a configuration for PUCCH transmission with a link recovery request (LRR) as described in Clause 9.2.4. - The UE can transmit in a first PUSCH at least one MAC CE providing one index for at least one corresponding SCell with radio link quality worse than Qout,LR, an index qnew for a periodic CSI-RS configuration or for a SS/PBCH block provided by higher layers, as described in [11, TS 38.321], if any, for a corresponding SCell. - Section 9.2.4: A UE can be configured by schedulingRequestIDForBFR a configuration for LRR in a PUCCH transmission using either PUCCH format 0 or PUCCH format 1. The UE can be configured, by schedulingRequestPriority in SchedulingRequestResourceConfig, a priority index 0 or a priority index 1 for the SR.

The above text shows that UE should transmit BFR request through the first available PUCCH resources, that is configured as schedulingRequestIDForBFR, after BFR occurs. Hence, RAN4 can define a requirement to check if the UE transmits within one period of PUCCH resource, after UE has detected beam failure and found a new beam.

Proposal 7: After detecting beam failure in an Scell and determining that the L1-RSRP of one candidate beam in SCell is greater than the configured threshold, UE is required to transmit scheduling request in the PSCell within a period T

  - Where T is equal to the periodicity of PUCCH that has been configured with schedulingRequestForBFR.

# Conclusion

Observation 1: If proposal 4 gets agreed, then UE only needs to perform BFD/CBD for up to one SCell in FR2.

Proposal 1: UE is not required to perform BFD on RS within a deactivated SCC which is implicitly configured as the BFD-RS for another activated SCell

Proposal 2: UE is not required to perform BFD on RS within an activated SCC which is implicitly configured as the BFD-RS for another deactivated SCell.

Proposal 3: When more than 2 BFD-RSs are transmitted on a CC for current SCell and implicity configured for SCell, UE is required to only perform BFD on BFD-RSs for current SCell.

Proposal 4: For BFD and CBD  requirements in FR2, only one serving cell is assumed to perform BFR procedure for one FR2 band

Proposal 5: RAN4 supports introducing a sharing factor in the BFD and CBD evaluation period for SCells in FR1.

- The sharing factor is proportional to the number of SCells for which UE is performing BFD/CBD.

- Note: This proposal assumes that UE is performing BFR for only one serving cell in each FR2 band.

Proposal 6: RAN4 defines requirement for step 1 of BFR, where UE reports beam failure event through a dedicated SR like PUCCH resources, in SCells with DL only.

- No RRM core requirement is defined for UE conveying new beam information and failed CC indices via MAC-CE.

Proposal 7: After detecting beam failure in an Scell and determining that the L1-RSRP of one candidate beam in SCell is greater than the configured threshold, UE is required to transmit scheduling request in the PSCell within a period T

  - Where T is equal to the periodicity of PUCCH that has been configured with schedulingRequestForBFR.

# Reference

- Chairman’s notes, 3GPP TSG RAN WG4 Meeting #93.