3GPP TSG-RAN WG4 Meeting #94-e                                    R4-2002494

Electronic meeting, 24th Feb– 6th Mar, 2020

Agenda Item:   8.5.4

Source:        Samsung

Title:           WF on IAB-MT class definition

Document for:  Approval

# WF on IAB-MT class

- 2 IAB- MT classes will be defined in Rel-16 as a starting point

  - Wide area IAB MT

  - [medium range or Local area] IAB MT

Note: More classes in the future are not precluded.

- IAB-MT classification criteria

  - Many of the candidate options have been discussed in 1st round as below

  - Option 1: Distance from donor node (minimum/maximum)

  - Option 2: Power dynamic range

  - Option 3: Max Tx power(TRP or EIRP)

  - Option 4: Min distance to other operator

  - Option 5: planned or unplanned

  - It’s believed that the deployment scenario should be considered to derive criteria of IAB-MT classification

  - It seems distance from donor node would be easiest and most straight forward parameter as starting point for next step discussion

    - It is encouraged companies to provide proposal on distance to differentiate the scenario

  - Other factor on top of distance is not precluded if it can identify the scenario more accurately together with distance

- IAB-MT classification dependent RF requirement

  - At least it is agreed to discuss max Tx power requirements for the each IAB-MT class

  - Max Tx power is declared by vendor

  - Max Tx power is different for each IAB MT class

  - Wide area IAB

    - No max Tx power for FR2

    - FFS on Max Tx power for FR1

  - [Medium range or local area] IAB

    - FFS max Tx power for FR2

    - FFS on Max Tx power for FR1

  - FFS on other RF requirements

  - Note: ACLR &dynamic range, ACS, REFSENS and details on max TX power will be elaborated and discussed in separate WFs

# Reference

[1] R4-2002373, Email discussion summary for RAN4#94e_#83_NR_IAB_RF_Tx

[2] R4-2001868, IAB class definition

[3] R4-2001283, On IAB-MT classes and dynamic range

[4] R4-2001709, [IAB] Discussion on power classes

[5] R4-2001436, IAB-Node transmitter requirements for FR2

[6] R4-2000276, Radiated transmit power and OTA output power for IAB-MT