3GPP TSG RAN WG4 Meeting #94-e	R4-2002598

Online, 24th February – 6th March 2020

Agenda item:	9.3.2

Source:	SoftBank Corp.

Title:	TP for TR 37.716-11-11: DC_11_n3

Document for:	Approval

1	Introduction

2B EN-DC of DC_11_n3 was approved in RAN#86[1]. This TP is to capture the basic aspects for the EN-DC.

2	Band combination and Configurations

The band combination and configurations to be supported are listed in Table 2-1 and 2-2.

Table 2-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_11_n3 | 11 | n3 | No |

Table 2-2: Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_11A_n3A | DC_11A_n3A | 11A | n3A |

3.  Maximum Output Power

The maximum output power to be supported is in Table 3-1.

Table 3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_11A_n3A | 23 | +2/-3 |

4.  Spurious emission band UE co-existence for DC

The requirements of spurious emission band UE co-existence for band 11 and n3 are captured from 36.101 and 38.101-1, respectively.

Table 4-1: Requirements of spurious emission band UE co-existence for band 11

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 11 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. |  |  |  |  |  |  |  |

Table 4-2: Requirements of spurious emission band UE co-existence for band n3

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n3, n80 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76. NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 22, 42, 52, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 13: This requirement applies for 5, 10, 15 and 20 MHz NR channel bandwidth allocated within 1744.9 MHz and 1784.9 MHz. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

Then proposal to be captured in the TR is as below:

Table 4-3: Protection table for EN-DC

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_11_n3 | E-UTRA Band 1, 28, 34, 65 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 42 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 13 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. |  |  |  |  |  |  |  |

5. Delta Tib/Rib

From delta Tib/Rib of LTE CA_3-11, the Delta Tib/Rib proposals for DC_11_n3 are as follows.

Table 5-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_11_n3 | 11 | 0.8 |
|  | n3 | 0.9 |

Table 5-2: ΔRIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_11_n3 | 11 | 0.3 |
|  | n3 | 0.5 |

6.  MSD analysis for DC

The frequencies of harmonics and IMD products are captured in Table 6-1.

Table 6-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427.9 | 1447.9 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2855.8 | 2895.8 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4283.7 | 4343.7 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 5711.6 | 5791.6 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 7139.5 | 7239.5 | 8550 | 8925 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 8567.4 | 8687.4 | 10260 | 10710 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 9995.3 | 10135.3 | 11970 | 12495 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 357.1 | 262.1 | 3137.9 | 3232.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1070.8 | 1185.8 | 1972.1 | 2142.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4565.8 | 4680.8 | 4847.9 | 5017.9 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2498.7 | 2633.7 | 3682.1 | 3927.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 714.2 | 524.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5993.7 | 6128.7 | 6557.9 | 6802.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6275.8 | 6465.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5712.1 | 5392.1 | 4081.6 | 3926.6 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2499.2 | 2234.2 | 923.7 | 713.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8267.9 | 8587.9 | 7421.6 | 7576.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7985.8 | 8250.8 | 7703.7 | 7913.7 |

Based on Table 5-1, it can be seen that

- 2nd order harmonic may fall into Rx frequencies of band 22, 42, 48, 49, n48, n77 and n78.

- 3rd order harmonic may fall into Rx frequencies of band 46.

- 3rd order IMD products may fall into Rx frequencies of bands 1, 2, 4, 10, 25, 34, 36, 65, 66, and 70.

- 4th order IMD products may fall into Rx frequencies of bands 7, 38, 41, 43, 44, 48, 49, 69, 71, n77, n78, and n90.

- 5th order IMD products may fall into Rx frequencies of bands 5, 6, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 30, 40, 41, 44, 46, 53, 67, 68, 85, n77 and n90.

Because of no harmonic and IMD products falling into the own Rx Band, there is no MSD issue for this DC configuration.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 5-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6-2: Band 11 and Band n3 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 5th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 5th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 3rd Harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

7.  Conclusion

This paper proposes study results of DC_11_n3.

TP is shown in Annex.

8.  Reference

[1]  RP-192742  Revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band)

Annex.   Proposed text for TR37.716-11-11:

[Unchanged Parts Skipped]

## 6.1.x			DC_11_n3

6.1.x.1		Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_11_n3 | 11 | n3 | No |

### 6.1.x.2		Configuration for DC

Table 6.1.x.2-1: Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_11A_n3A | DC_11A_n3A | 11A | n3A |

### 6.1.x.3		Co-existence studies

Table 6.1.x.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_11A_n3A | 23 | +2/-3 |

### 6.1.x. 4		Spurious emission band UE co-existence for DC

For DC_11_n3, the TIB,c and RIB,c values for UEs not supporting simultaneous Rx/Tx are given in the tables below.

Table 6.1.x.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_11_n3 | E-UTRA Band 1, 28, 34, 65 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  |  | E-UTRA band 3 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 5 |  |
|  |  | E-UTRA Band 11, 18, 19, 21 |  | FDL_low |  | - |  | FDL_high |  | -50 |  | 1 |  | 13 |  |
|  | E-UTRA Band 42 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | - |  | 1915.7 |  | -41 |  | 0.3 |  | 3, 13 |  |
|  |  | Frequency range |  | 2545 |  | - |  | 2575 |  | -50 |  | 1 |  |  |  |
|  |  | Frequency range |  | 2595 |  | - |  | 2645 |  | -50 |  | 1 |  |  |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. |  |  |  |  |  |  |  |

### 6.1.x.5		MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.x.5-1

Table 6.1.x. 5-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1427.9 | 1447.9 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 2855.8 | 2895.8 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 4283.7 | 4343.7 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 5711.6 | 5791.6 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 7139.5 | 7239.5 | 8550 | 8925 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 8567.4 | 8687.4 | 10260 | 10710 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 9995.3 | 10135.3 | 11970 | 12495 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 357.1 | 262.1 | 3137.9 | 3232.9 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1070.8 | 1185.8 | 1972.1 | 2142.1 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4565.8 | 4680.8 | 4847.9 | 5017.9 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2498.7 | 2633.7 | 3682.1 | 3927.1 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 714.2 | 524.2 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5993.7 | 6128.7 | 6557.9 | 6802.9 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6275.8 | 6465.8 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5712.1 | 5392.1 | 4081.6 | 3926.6 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2499.2 | 2234.2 | 923.7 | 713.7 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8267.9 | 8587.9 | 7421.6 | 7576.6 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7985.8 | 8250.8 | 7703.7 | 7913.7 |

Based on Table 6.1.x. 5-1, it can be seen that

- 2nd order harmonic may fall into Rx frequencies of band 22, 42, 48, 49, n48, n77 and n78.

- 3rd order harmonic may fall into Rx frequencies of band 46.

- 3rd order IMD products may fall into Rx frequencies of bands 1, 2, 4, 10, 25, 34, 36, 65, 66, and 70.

- 4th order IMD products may fall into Rx frequencies of bands 7, 38, 41, 43, 44, 48, 49, 69, 71, n77, n78, and n90.

- 5th order IMD products may fall into Rx frequencies of bands 5, 6, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 29, 30, 40, 41, 44, 46, 53, 67, 68, 85, n77 and n90.

Because of no harmonic and IMD products falling into the own Rx Band, there is no MSD issue for this DC configuration.

When 2UL inter-band EN-DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.x.5-2 lists up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.x.5-2: Band 11 and Band n3 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | No |  |  |
| Galileo | 1559 | - | 1591 | No |  |  |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | No |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | 5th IMD |
|  | 2400 | - | 2494 | Yes | Asia | 5th IMD |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | 3rd Harmonic |
|  | 5150 | - | 5350 | Yes | Europe | 3rd Harmonic |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | Yes | Asia | 3rd Harmonic |
| 45GHz Unlicensed Bands | 42300 | - | 47000 | No | China |  |
|  | 47200 | - | 48400 | No | China |  |
| 60GHz Unlicensed Bands | 57000 | - | 66000 | No | Europe |  |
|  | 57050 | - | 64000 | No | USA Canada |  |
|  | 57000 | - | 64000 | No | South Korea |  |
|  | 59000 | - | 66000 | No | Japan |  |
|  | 59000 | - | 64000 | No | China |  |
|  | 59400 | - | 62900 | No | Australia |  |

6.1.x.6	∆TIB and ∆RIB values

For DC_11_n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.x.1.6-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_11_n3 | 11 | 0.8 |
|  | n3 | 0.9 |

Table 6.x.1.6-2: ΔRIB,c

| Inter-band EN-DC configuration | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_11_n3 | 11 | 0.3 |
|  | n3 | 0.5 |

[Unchanged Parts Skipped]