3GPP TSG-WG RAN4 Meeting #94-e	R4-2002633

# Online, 24th February – 6th March, 2020

Source: 	Nokia, Nokia Shanghai Bell, [AT&T]

Title: 					TP to TR 36.716-03-01: CA_7-26-66

Agenda Item: 			7.3.3

Document for:	Approval

# Introduction

This TP introduces the following LTE inter-band CA for 3 bands DL with 1 band UL for the CA bands CA_7-26-66.

- CA_7A-26A-66A_BCS0

**************************** Start of changes ********************************************

## 5.X	CA_7-26-66

### 5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Supported E-UTRA bandwidths per CA configuration for 3-band DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_7A-26A-66A | 7 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  | 26 |  | Yes | Yes | Yes | Yes |  |  |  |
|  | 66 |  | Yes | Yes | Yes | Yes | Yes |  |  |

### 5.X.2	∆TIB and ∆RIB values

The ∆TIB and ∆RIB values of CA_7-26-66 is proposed to be based on the diplexer combining CA_7-66 quadplexer and band 26 chain.

Table 5.X.2-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7-26-66 | 7 | 0.5 |
|  | 26 | 0.3 |
|  | 66 | 0.5 |

Table 5.X.2-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_7-26-66 | 2 | 0 |
|  | 26 | 0 |
|  | 66 | 0 |

### 5.X.3	REFSENS requirements

There is no REFSENS exception for CA_7A-26A-66A.

**************************** End of changes ********************************************