3GPP TSG-RAN WG4 Meeting #94-e	R4-2002671

Online, 24th February – 6th March 2020

Source:	Ericsson, BT plc

Title:	TP for TR 38.716-03-02: CA_n1-n7-n28

Agenda item:			9.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.716-03-02 to include CA_n1A-n7A-n28A as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

### 5.1.x	CA_n1A-n7A-n28A

#### 5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n7-n28 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n28 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n7A-n28A | CA_n1A-n7A CA_n1A- n28A CA_n7A-n28A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  |  |  |  | n28 |  | 15 |  |  | Yes |  | Yes |  | Yes |  | Yes2 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 30 |  |  |  |  | Yes |  | Yes |  | Yes2 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. NOTE 2: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.1.x.3	UE co-existence studies

The harmonic issue for Band n1+Band n7 + Band n28 has already been addressed in TR38.716-03-01, where there are no harmonic interference and harmonic mixing problem for CA_n1A_n7A-n28A.

Based on co-existence studies of Band n1 + Band n7, Band n1 + Band n28 and Band n7 + Band n28 captured in TR 38.716-02-00, own Rx impact of the IMD5 is the followings:

- 2nd order IMD generated by dual uplink of Band n1 + Band n28 may fall into part of own band n7.

- 5th order IMD generated by dual uplink of Band n1 + Band n7 may fall into part of own band n28.

#### 5.1.x.4	∆TIB and ∆RIB values

For three simultaneous DLs and one UL of Band n1, n7 and n28, the TIB,c and RIB,c  values are shown in table 5.1.x.4-1 and  table 5.1.x.4-2, respectively. Values are derived from DC_1-7_n28.

Table 5.1.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n7-n28 | n1 | 0.5 |
|  | n7 | 0.6 |
|  | n28 | 0.6 |

Table 5.1.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n7-n28 | n1 | 0 |
|  | n7 | 0 |
|  | n28 | 0.2 |

#### 5.1.x.5	REFSENS requirements

CA_n1-n7-n28 need to have the same MSD requirements as DC_1-7_n28. Below are the updates needed in Table 7.3A.5-2 of TS 38.101-1.

Table 5.1.x.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1A-n7A-n28A | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n7 | 2533 | 10 | 50 | 2653 | 30.0 | FDD | IMD2 |
|  | n28 | 718 | 5 | 25 | 773 | N/A | FDD | N/A |
|  | n1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |
|  | n28 | 730 | 10 | 50 | 785 | 4.5 | FDD | IMD5 |

---End of changes---

# Reference

[1] 		RP-192617, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL”, ZTE