3GPP TSG-RAN WG4 Meeting #94-e	R4-2002875

Electronic Meeting, Feb.24th – Mar.6th 2020

Agenda item:			6.3

Source:	Moderator (ZTE Corporation)

Title:	Email discussion summary for RAN4#94e_#2_NR_NewRAT_SysParameters

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion and provide some guidelines for email discussion if necessary.

This email thread is assigned to discuss issues related to Rel-15 system parameters maintenance.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round:

  - Collect understanding and views of each company on the listed open issues

    - If there is an unanimous understanding on one issue after collection, the issue will be removed from the open issues.

  - Discussion aiming to aligned understanding

- 2nd round:

  - Tentative observations from the first round discussion

  - Suggestion on WF

# Topic #1: Add 30 kHz SCS for n40 SSB

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2000193 | Huawei | Proposal: to address the misalignment between SSB SCS configuration and data SCS configuration when 30KHz data SCS is configured for Band n40, the following alternative solutions are proposed: • Alt#1: Add 30KHz SSB SCS in the synchronization raster configuration for n40 from Rel-15 • Alt#2: Add 30KHz SSB SCS in the synchronization raster configuration for n40 from Rel-16, and check the feasibility of Rel-16 network supporting both 15KHz SSB SCS and 30KHz SSB SCS. • Alt#3: Define a new band with the same frequency range as n40 and reusing the same RAN4 requirements and add 30KHz SSB SCS for it. -------------------------------------------------------------- Observation 1: Enabling 30KHz SCS for n40 SSB can improve the spectrum efficiency by saving the guard bands between SSB with 15KHz SCS and data with 30KHz SCS when 30KHz SCS is used. Observation 2: The UE capability of simultaneous reception of SSB and data with different numerologies is optional, and without supporting that capability the longer measurement time, the more frequent interruptions on the service and more scheduling restrictions will be expected. Observation 3: To support the simultaneous reception of SSB and data with different numerologies will result in the increasing complexity for UE, BS and test equipment, which would delay the deployment for n40. Observation 4: Adding 30KHz SCS for n40 SSB could lead to the tolerable additional delay for initial access, which is comparable to the initial access delay for the bands which support both 15KHz and 30KHz SCS SSB. Observation 5: if there was or was going to be no n40 device in the market, it could be feasible to add 30KHz SCS for n40 SSB from Rel-15. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

The main motivation of the issue comes from the intention of avoiding a configuration of different SCSs between SSB and data, i.e., to align the SCS between SSB and data. This sub-topic is to align understanding within the group on the necessity of adding 30kHz SCS for n40 SSB, which is fundamental to decide whether or not to introduce the new SCS for n40 SSB into specs.

Notes:

- The below issue 1-1 and 1-2 are for non-CA case, and 1-3 s for CA case, and please keep in minds that these issues are generic, not only to n40.

- Issue 1-4 is n40 specific.

- If adding 30k SCS for n40 SSB, it is obvious that the searching time will be increased during initial access, thus this is not listed.

### Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1: Is additional specs change required to address the case where SSB and data have different SCSs?

- Views

  - View 1: Yes, it can be at least 4 PRBs on each side of SSB

  - View 2: No, up to implementation according to current specs “inter-numerology guard band within the carrier is implementation dependent”

### Sub-topic 1-2

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2: In addition to potential guard-band, what would be other consequences if SSB and data is configured with different SCSs? And is any standardization efforts/correction required?

- Views

  - View 1: Long interruption and downlink throughput loss, and increased implementation complexity for BS, UE and test equipment, so the specs shall ensure that the same SCS of both SSB and data be supported for all supported SCSs in each band.

  - View 2: Implementation dependent only, and already addressed in the current specs via different capability signalling. No more specs change required.

### Sub-topic 1-3

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-3: Does supportedSubCarrierSpacingDL indicate both SSB and data in NR CA? For example, in an inter-band CA configuration, if CC#1 is configured with a 15k SCS SSB and 30k SCS data, and CC#2 is configured with a 30k SCS SSB and 30k SCS data, is the configuration supported by the current specs without the capability of simultaneous reception with different numerologies in CA?

- Views

- View 1: both SSB and data, and the example is not supported if the capability of simultaneous reception with different numerologies in CA.

- View 2: only refer to data, and the example is supported because the data SCS of both CCs are the same (30k).

### Sub-topic 1-4

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-4: Is there any NBC issue for Rel-15 if adding 30k SCS for n40 SSB? And what could be the solution to solve if yes?

- Views

- View 1: No, add 30k SCS for n40 SSB from Rel-15

- View 2: Yes, add 30k SCS for n40 SSB from Rel-16

- View 3: Yes, define a mirror band supporting 30k SCS SSB from Rel-16

- View 4: Yes, no changes needed at present

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| CMCC | Sub topic 1-1 (Issue 1-1): We think “View 2” is better. In the case that SSB and data have different SCSs, the guard band can leave to implementation. Sub topic 1-2 (Issue 1-2): It is not clear whether there is any standardization efforts required if SSB and data is configured with different SCSs. Our understanding is that the current spec can support mix numerology. But from practical deployment perspective, it is better to use the same SCS for SSB and data in order to get better performance. Sub topic 1-3 (Issue 1-3): Our understanding is that supportedSubCarrierSpacingDL only indicate data for CA. But there is another UE capability simultaneousRxDataSSB-DiffNumerology, which indicates whether UE supports concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology. This capability will have impact on the performance if 15K SCS SSB and 30KHz SCS data are deployed. Sub topic 1-4 (Issue 1-4): Based on our knowledge, there is no UE supporting n40 so far. We prefer to add 30KHz SSB SCS in Rel-15 (Veiw 1). |
| CATT | Sub topic 1-1 In our view, the guard band between SSB and data with different SCS is an implementation issue. Sub topic 1-2: From implementation complexity and performance point of view, we prefer to support same numerology between SSB and data as much as possible. Sub topic 1-3: Same view as CMCC. Sub topic 1-4: If there is no Rel-15 UE developed for Band n40. We prefer to add 30kHz SSB from Rel-15. |
| ZTE | Sub topic 1-1 We have similar view as CMCC and CATT, additional changes for spec are no need. It is an implementation issue. Sub topic 1-2: In my understanding, mix numerology is supported in current spec and it is band agnostic. Usually the introduction of new SSB SCS is pending on the operator. Similar view as CMCC, it is better to use the same SCS for SSB and data in order to get better performance. Sub topic 1-4: If there are similar situation for the other bands in future. How to treat? |
| Qualcomm | Sub topic 1-1 (Issue 1-1): We agree with view 2. Standardizing a guard band for different numerologies was discussed in Rel.15 time frame and agreement was to leave it to implementation. We do not think anything has changed since then such that this is now needed. Sub topic 1-2 (Issue 1-2): We agree with view 2, nothing is needed from a specification point of view. This scenario is supported Sub topic 1-2: (Issue 1-3) supportedSubCarrierSpacingDL is for data SCS, support of 15kHz and 30kHz SCS for SSB should be mandatory. Whether there is a need for any additional capabilities or IOdT bits should be further discussed. simultaneousRxDataSSB-DiffNumerology indicates if UE supports concurrent measurements and data reception with different numerology. This capability would be needed to avoid interruptions caused by measurement gaps configured for intra-frequency measurements. Sub topic 1-4(Issue 1-4): There will be backwards compatibility issues if 30kHz SSB SCS is added from Rel.15. Addition only from Rel.16 could also cause issues in the field. It is not clear why a new band would be needed since the specifications support the deployment scenario needed. |
| Vivo | We have a few questions about Topic 1: The misalignment between SSB SCS configuration and data SCS configuration, is it a general issue for all NR operating bands with only 15kHz SSB SCS or a specific issue only existing in band n40? If this is a general issue, why only add 30kHz SSB SCS for band n40? How about other bands with the same issue? [Huawei]: the other band with the similar issue is n50. For the other re-farming bands, there would be no issue. Sub topic 1-1: From our perspective, “View 2” matches the current spec. Sub topic 1-2:’View 1’ restricts too much for each band ensuring the same SCS between SSB and data. We prefer to ‘View 2’ Sub topic 1-3: For the basic NR CA operation, only data channels across CCs have SCS restrictions. So we support ‘view2’. Sub topic 1-4: We share the same view with CATT and CMCC. |
| Huawei | Sub topic 1-1: The guard band is needed because that the sub-carriers of SSB with 15KHz SCS is not orthogonal to the sub-carriers of data with 30KHz SCS, which causes the interferences between SSB with 15KHz SCS and data with 30KHz SCS. And in the practical network, the PSD on SSB and data would be also different depending on the beam forming used for them to fulfill the need of coverage. Because the different companies will utilize the different PSD difference between SSB and data, it is difficult to specify the guard band in-between SSB and data. According to our implementation, 4PRB seems a reasonable value for some case. We have no proposal to define guard band, as it is specified that inter-numerology guard band within the carrier is implementation dependent. We do not propose it. Besides, 4RPB guard band does not contradict with the specification or agreement. Sub topic 1-2: Issue 1-2: Issues for SSB and data configured with different SCSs have been discussed. And the standardization work was done. For n40, 15KHz SCS SSB only causes some difficulty in the implementation especially for the case where 30KHz data SCS is used. So we prefer to add 30KHz SSB SCS. Sub topic 1-3: Issue 1-3: supportedSubCarrierSpacingDL is for subcarrier spacing used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Both view 1 and view 2 are inaccurate. In case to save your time to look up the RAN2 specification, I copy it as below. -- ASN1START -- TAG-FEATURESETDOWNLINKPERCC-START FeatureSetDownlinkPerCC ::= SEQUENCE { supportedSubcarrierSpacingDL SubcarrierSpacing, supportedBandwidthDL SupportedBandwidth, channelBW-90mhz ENUMERATED {supported} OPTIONAL, maxNumberMIMO-LayersPDSCH MIMO-LayersDL OPTIONAL, supportedModulationOrderDL ModulationOrder OPTIONAL } -- TAG-FEATURESETDOWNLINKPERCC-STOP -- ASN1STOP subcarrierSpacing Subcarrier spacing to be used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Corresponds to subcarrier spacing according to TS 38.211 [16], table 4.2-1. The value kHz15 corresponds to µ=0, value kHz30 corresponds to µ=1, and so on. Only the values 15 kHz, 30 kHz, or 60 kHz (FR1), and 60 kHz or 120 kHz (FR2) are applicable. For the initial DL BWP this field has the same value as the field subCarrierSpacingCommon in MIB of the same serving cell. Our point is that the feature of support of simultaneous reception of with different numerolgoies in CA for other cases, is optional. So for some CA combination, if 30KHz data SCS is used for PCell with larger channel bandwidth, the 30KHz data SCS should be used for other SCell on band n40 not matter what channel bandwidth is configured on carrier of n40, i.e., even smaller channel bandwidth. We just want to show the case where 30KHz data SCS is more preferable. The signaling referred by CMCC is clearer. We just refer to a sentence under the box of supportedSubCarreirSpacingDL. In our paper, we also provide other cases where 30KHz data SCS on n40 is more suitable. In other words, there are many scenarios where 30KHz data SCS is more easy-to-use. Sub topic 1-4: No reference for either views. Agree with CMCC, CATT We have already provided our analysis on the NBC issue. There is no impact on ASN.1. It depends on whether there is a legacy UE on market now. We encourage companies to think it from implementation point of view. 2.3GHz would become popular in future. And according to our analysis 30KHz data SCS is more preferable in the practical use case. It would be useful to reduce some complexity and ensure the system performance with 30KHz data SCS. As we said in our paper, there would be some benefits: - Since the simultaneous measurement and reception of data with different numerologies is optional feature, the 30KHz SSB is more reasonable to ensure the shorter measurement delay. - The guard band between SSB with 15KHz SCS and data with 30KHz SCS can be saved. - Even test equipment could only support the same numerology on data and SSB. If 30KHz is supported, then it is more easy to get TE. Otherwise, we need wait for a while. |
| Ericsson | General comment: the reasons for making the change is 1) wider bandwidth deployment, for which 80MHz and 60MHz channel bandwidths are specified with 30KHz SCS in n40, and 2) CA case, where the NR CA with n40 is deployed. We note that the current spectrum allocation in n40 is limited to 50 MHz in China. Regarding impact on NR carrier aggregation, more details on the actual impact of mixed numerology for SSB and data and interpretation of the associated UE capability are needed to justify changes for the cases in which the data SCS for n40 operation must be 30k. [Huawei]: we are not saying SSB for n40 must be 30KHz. We would like to add 30KHz SSB SCS on top of 15KHz SSB SCS in order to reduce the implementation complexity for both UE and BS when 30KHz data SCS is used. Issue 1-3: the applicability of the supportedSubCarrierSpacingDL for different SCS and data is unclear (the applicability to mixed numerology for data is clear). Issue 1-4: no changes agreed at present. |
| Apple | Issue 1-1: View 2 (up to implementation). Mixed numerologies are not new issue. We should stay with existing R15 conclusion and leave it to implementation. Issue 1-2: View 2 (up to implementation). We further observe that interruption requirements are already well defined in Rel-15 RRM Core requirements. Depending on UE capability on mixed numerologies between SSB and data, TS38.133/101-4 has addressed all the possible aspects to define UE behavior and the related requirements. There is no need to further discuss this. Issue 1-3: The supportedSubCarrierSpacingDL IE has been introduced since R15 and refers to data only. We don’t think there is ambiguity here. |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2000194 TS38.101-1 CR: adding 30KHz SSB SCS for n40, Huawei | Company A |
|  | Company B |
|  | Ericsson: not agreed |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic#1 | Tentative observations from the collected views: - Based on the majority view of Issue 1-1 and 1-2, Rel-15 specs has already well discussed/addressed and support mixed numerologies. And No additional standardization efforts are needed. - Based on the majority view of Issue 1-3, supportedSubCarrierSpacingDL indicates data only, thus the motivation of NR CA for adding 30k SCS to n40 SSB may be irrelevant. - Based on the collected views on all listed issues, the current situation is not enough to justify the proposed changes - Based on the collect views from companies, four companies prefer adding 30KHz SSB SCS from Rel-15, while two companies thought that no change is agreed at present - Based on the collected views, there are sided views on the proposal: two companies support addition of 30kHz SSB SCS from Rel-15, two companies support on condition that there is no NBC issue, two companies prefer no change on specs at present, one company has a concern on the potential similar issue on other bands. Tentative agreements: Candidate options: Recommendations for 2nd round: Recommended WF: - Proponents are encouraged to continue justifying the needs of adding 30k SCS support to n40 SSB e.g., by - Elaborating the case where 4 PRB guard band is needed at each side of SSB - Even in LTE, mixed numerologies were introduced, e.g., preamble with 1.25k SCS and data with 15k SCS, though it is at uplink direction. - Elaborating implementation difficulties - Further discussions on whether or not there is any NBC issue if adding 30k SCS for n40 SSB from Rel-15, and whether or not there is any field issue if adding 30k SCS for n40 from Rel-16. - Any other if possible |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2000194 TS38.101-1 CR: adding 30KHz SSB SCS for n40, Huawei | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” Return-to |

## Discussion on 2nd round (if applicable)

The below email discussion logs are recorded in a chronological order from newest to oldest:

Ericsson:

Our position has not changed; we are not ready to agree the WF. Band n40 is planned in several regions.

Qualcomm:

We do not agree with the WF as already stated. There is an NBC issue that needs further consideration. Agreeing the WF is not a compromise.

I made the comments and revisions because what was written in the background was misleading.

Huawei:

We also got the support from CITC. CITC expert seemed have problem to make comment in the email reflector. The expert's email to comment was rejected by 3GPP server. So I would like you know his support for this WF.

In my understanding, there seems a possible deployment plan in KSA for up to 100MHz on 2.3GHz.

Hi Vali,

We got your comments in the inbox. You did not make modification on the last slide with title WF. My understanding is that you still had concern on adding 30KHz.

But actually your revised version is some kind of acceptable to us given that our proposal is still there. Can we call it as a compromise? If yes, please let us know.

Because the deadline to make the last comments and upload the final version is approaching, I uploaded my previous version of WF into inbox and also provide the response version in the draft. More discussion after the deadline is expected.

Chairman:

To clarify, when I looked at the first round summary from moderator, I decided to assign the WF to capture the possible progress and agreement on this topic. It was captured in my notes “RAN4_94e_Main_chairman_report_01_0301”.

Qualcomm:

First of all, as I think we pointed out, there will already be NBC issues. Please see some more replies inline.

The current WF is not based on the discussion in the first round, it is just a Huawei view. We made some comments and changes to the WF in the drafts folder.

When was this WF assigned? The moderator summary was just saying that 2nd round discussion was encouraged? Is it possible to just ask for WFs on any topic directly to the chairman?

Huawei:

First question is whether 30kHz SSB SCS for n40 is mandatory or optional. Since there is no change in ASN.1, is it mandatory?

Second, most likely there are some similar demands to add SSB SCS for other bands in the future. How to treat it?

[Huawei]: Yes, supporting 30KHz SSB SCS for initial access for a UE is mandatory, if 30KHz is added.

So far we have no smart and generic approach to treat that. Considering some optional features mentioned in the way forward, there might be an problem in the future when LTE bands are re-farmed.

n40 is some kind of special case: firstly it has wide bandwidths unlike the other refarming bands. According to our survey globally, some operators have wide band while others have narrow spectrum holding. Secondly in our understanding, there is no commercial n40 device now.

[Qualcomm] First of all, this addition violates previous agreements of having only a single default SCS per band. There are a few exceptions but we do not want to proliferate this. The fact that there are no commercial devices now is just Huawei’s understanding, we do not believe this is true.

[Huawei]: The similar reason to allow the previous exception applies to n40. We have additional reasons. If there is NR commercial devices, please let us know some details.

To Christian, Vali,

I copy Qualcomm (Vali) response below for convenience.

We should bring this topic earlier. But really hope your experts could re-consider the situation.

From network side, even if you have plan for n40, it is OK for network only to support 15KHz SCS SSB. n40 UE will support 15KHz SSB SCS no matter whether 30KHz SSB SCS is added or not.

From UE side, since UE anyway will support n41, or other bands supporting both 15 and 30, the baseband is ready. We are not sure if there is really a big burden for UE to support 30KHz SCS SSB in n40.

[Qualcomm] This is not just about implementation burden, this also has to do with initial acquisition performance. Having multiple default SCSs is not desirable.

[Huawei]: Since on n41 both 15 and 30 needs be supported, we do not know whether there is severe problem for initial access on n41. If no for n41, we do not see why we have problem for n40.

Regarding overhead, it depends. Assuming 4 RPB guard band each side and 4 SSBs are configured, it is about 1.2% overhead for TDD with DDDSU, 10ms SSB periodicity and 60MHz bandwidth and 2.4% overhead for TDD with DDDSU, 5ms SSB periodicity and 60MHz bandwidth. And guard band is not only related to overhead but also related to implementation complexity.

By the way, did you check the situation of test equipment? I wonder if the mixed numerology can be widely supported or not.

[Qualcomm] with 20ms SSB perioidicyt we end up with 0.6% overhead which is not big. It seems that the SSB periodicity is changed depending on how good the numbers look like. Test equipment has nothing to do with this discussion.

[Huawei]: whether TE is ready would impact the deployment.

On the other hand, if we missed this opportunity, there would be some n40 devices in the market a few months later. Then we have no chance to make modification anymore. Even if we defined the new band on 2.3GHz supporting both 15 and 30, both BS and UE need to support two bands for 2.3GHz. We do not know if that is really desirable.

[Qualcomm] From our understanding, the deadline for this already passed.

[Huawei]: We are not sure what deadline means here.

Qualcomm:

We cannot agree with this WF.

First of all, the need is not very clear because the overhead generated by the guardband is a few RBs every 20ms which is very small.

If this is so urgent then the only agreeable solution for us at this time is to define a new band.

Ericsson:

We do not agree this WF. We would like request more time before agreeing any changes given that Band n40 is planned for several regions.

ZTE:

To Takuma, our intention for the comments of 'how to treat the similar situation for the other bands in future' in 1st round was that the similiar future work may adopt the similar way as band n40 to save the meeting time.

Regarding the WF itself, we also had internal discussion and we didn't see the NBC issue if 30kHz SSB SCS is introduced to Band n40 from R15, and as we mentioned in 1st, it is better to use the same SCS for SSB and data in order to get better performance. Hence we also agree with the WF and co-sign the WF.

To Xizheng, please include ZTE as a supporting company.

NTTDoCoMo:

This WF itself is fine for us.

We have two general questions.

First question is whether 30kHz SSB SCS for n40 is mandatory or    optional. Since there is no change in ASN.1, is it mandatory?

Second, most likely there are some similar demands to add SSB SCS    for other bands in the future.

How to treat it?

ZTE had the similar comment in 1st round, so we would like to clarify it.

CATT:

We agree with the analysis in this WF. Please include CATT as a supporting company.

CMCC:

We support the WF, and please add CMCC as supporting company.

Huawei:

Please find  the draft WF in the inbox.

https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_94_e/Inbox/Drafts/%232_NR_NewRAT_SysParameters/draft_R4-2002717_WF%20on%20adding%2030KHz%20SCS%20for%20n40%20SSB%20v1.0.pptx

Comments are welcome in this email thread.

Moderator:

This is to initiate the second round  discussion on Rel-15 system parameters maitenance. The following are the outcome of the first round discussion:

(1) R4-2002717 WF on adding 30 kHz SCS to n40 SSB

Source: Huawei

The corresponding return-to CRs(R4-2000194) treatment will depend on the WF.

(2) R4-2002718 CR to TS 38.101-3:  Correct the intra-band ENDC channel spacing with editorial format correction

Source: ZTE

(3) Discussion on whether the channel spacing definition is band specific or agnostic: this is essential on whether or not to refer to a different table. For the time being, we can focus on  the discussion, and will see if a separate WF is                    needed. The corresponding CRs(R4-2001784/85) will be returned according to the discussion.

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX R4-2002717 Way forward for adding 30kHz SCS to n40 SSB | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” Based on R4-2000193, it is proposed to add 30KHz SSB SCS for n40 in Rel-15 in WF R4-2002717. And the reasons are provided in it too. Huawei, HiSilicon, CMCC, CITC, Spreadtrum, CATT and ZTE supported the WF, while Qualcomm and Ericsson objected it. The comments and responses are captured in section 1.5 of the summary. The issues are not settled. More discussions are needed in the future meeting. |
| R4-2000194 TS38.101-1 CR: adding 30KHz SSB SCS for n40, Huawei | Recommended as “Noted” |

# Topic #2: Redundant sentence for the intraband EN-DC channel spacing

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2000489 CR to TS 38.101-3: Correct the intra-band ENDC channel spacing | ZTE Corporation | Proposal : remove the redundant sentence: For NR operating bands with 15 kHz channel raster, Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-5kHz, 0kHz, 5kHz} - Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-5kHz, 0kHz, 5kHz} for ∆FRaster equals to 15 kHz - Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-10 kHz, 0 kHz, 10 kHz} for ∆FRaster equals to 30 kHz |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: Redundancy confirmed

- Views

  - View 1: Yes

  - View 2: No

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Huawei | Sub topic 2-1: We suggest View2. |
| Nokia | Sub topic 2-1: View 1 |
| Ericsson | Sub topic 2-1: View 1 |
| ZTE | Sub topic 2-1: View 1 In RAN4#92 meeting, it was agreed that the equations for NR operating bands with 15 kHz channel raster in TS38.101-1 were corrected related to ∆FRaster (i.e. 15 kHz and 30kHz.). For TS38.101-3, it shall be aligned with 38.101-1. Actually, we have submitted the CR to TS38.101-3 to correct the equations aligned with 101-1 in RAN4 #93 meeting and the CR was approved. However, the CR was wrongly reflected in the lastest 101-3 spec. Hence it is need to remove the redundant sentence in 101-3. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2000489 CR to TS 38.101-3: Correct the intra-band ENDC channel spacing | Company A |
|  | Company B |
|  | Ericsson: agreed. Editorial correction: The two last bullets should be sub-bullets under the 15 kHz spacing and be indented to B2. |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #2-1 | 1-st round summary as of 10:00pm UTC, Feb 26 Ericsson/Nokia/ZTE: support the change Huawei: not support The majority view confirms the redundancy and the proposed CR does not change any technical contents, only editorial change and aligned to the similar approved change in TS 38.101-1. Recommend WF: - Revise the proposed CR to correct the format (indenting bullets) and approve the revised CR Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Clear enough in the first round.

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2002718 | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” Recommended as “Agreeable” |

# Topic #3: Wording and reference correction for CA channel spacing

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2001783 CR for 38.101-1 channel space for CA_Rel15, R4-2001785 CR for 38.101-2 channel space for CA_Rel15 | Huawei, HiSilicon | Proposal 1: Add “common” to µ Proposal 2: Referring to Table 5.3.2-1 instead of 5.3.5-1 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-1: Necessity to add “common” to µ

- Views

  - View 1: Yes, more clear

  - View 2: No, it is clear enough for the current wording “μ0  is the common largest μ value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths”

### Sub-topic 3-2

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-2: which is preferred, Table 5.3.5-1 or Table 5.3.2-1?

- Views

  - View 1: Table 5.3.5-1 as it is now, since this allows for band specific definition.

  - View 2: Table 5.3.2-1 as proposed, since it applies to all bands.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 3-1: Skyworks: the overall definitions of the channel spacing, guard bands are measurement bandwidth is also discussed within Thread #18 for NR UL intra-band contiguous CA. we believe an consistant specification and notation is needed throughout. We could agree to proposed CR once the overall view is consistent and shared amongst companies. We anyhow agree that current spec wording needs clarification. For which table to refer to, I believe it is ambiguous since the raster is defined in one table and the common largest mu in another. ZTE: It seems misunderstanding exist among companies since some discussion on the measurement channel bandwidth of ACLR in #18. In thread #18, company also provide the proposals on how to improve this sentence, hence it shall be discussed together. Sub topic 3-2: …. ZTE: The original table (i.e. Table 5.3.5-1) is better. The table 5.3.2-1 is for all band. However, we should consider the supported SCS for the channel bandwidth supported for a certain band for intra-band CA operation. For example, for band n5 operated in intra-band CA. According to Table 5.3.5-1, 60kHz is not supported for all the channel bandwidths supported in band n5, so μ0 is 1 (SCS=30kHz) for 10M+10M CA. However, μ0 may be interpreted as 2 (SCS=60kHz) according to table 5.3.2-1 since 60kHz SCS is supported for 10MHz. Others: |
| Qualcomm | Sub topic 3-1: We agree with view 2, the change is not needed. Wording is already clear enough, adding common would make the text redundant Sub topic 3-2: we agree with view 1, no change is needed. |
| Huawei | Sub topic 3-1: common wording is we have agreed in the WF approved in RAN4 #91 meeting, we suggest to follow the exact description in the approved WF. We don’t want to have misunderstanding for audiences of TS 38.101. Sub topic 3-2: it is a necessary change because table 5.3.5-1 is updated with each version of TS 38.101, but the UE cannot adjust the table accordingly. Table 5.3.2-1 is relative fixed. Nominal channel space is just a reference space for UE and gNB verify the contiguous issue, the information from table 5.3.5-1 on calculating nominal channel space is redundant. |
| Nokia | Support Skyworks WF to further check the issue. |
| Ericsson | Sub topic 3-1: the CR is proposing a change of the definition of the CA carrier spacing, it’s not a clarification. The proposed change is not in accordance with the agreement at RAN4#91: the SCS used for determining the CA spacing is the largest supported by both CHBW in the band, hence band specific (and thus the reference to 5.3.5-1). It was verified that this definition does not imply any overlap between adjacent carriers. View 2 is the only possible, there should be no changes of the CA carrier spacing (specification of the existing definition was already considered late in May 2019). Sub topic 3-2: (see above) View 1. |
| Apple | Issue 2-2: If there is a dependency on a discussion in Topic 18, as Skyworks point out, then we should await the outcome of that discussion before proceeding. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2001783 CR | Ericsson: not agreed. |
|  | Company B |
| R4-2001785 CR | Ericsson: not agreed. |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | 1-st round summary as of 10:00pm UTC, Feb 26 - Different views on the need of the proposed changes over the current specs: - Skyworks/Nokia: need clarification and encourage consistent specs and notations with NR UL intra-band contiguous CA - Ericsson/ZTE/Qualcomm: clear enough, no need to change. The proposed change is not aligned with the agreements - Huawei: Need change to align with the agreements Tentative agreements: Candidate options: Recommendations for 2nd round: Recommend WF: - Further discussion to align common understanding on the original agreements - Focus on whether the channel spacing definition is band specific or agnostic - the proposed CR is return to… |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2001783 CR for 38.101-1 channel space for CA_Rel15, | recommended as return to |
| R4-2001785 CR for 38.101-2 channel space for CA_Rel15 | recommended as return to |

## Discussion on 2nd round (if applicable)

No more comments received implying that companies views do not change, and it seems not feasible to reach consensus.

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2001783/1785 | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” Recommended as “to be noted”. |