3GPP TSG-RAN4 Meeting #94-e	R4-2002219

# Online, , 24th Feb 2020 - 6th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.171 | CR | 0009 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of the Note 1 in the Power level and satellite allocation table for the Sensitivity Coarse time assistance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2020-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current note in the Power level and satellite allocation table for the Sensitivity Coarse time assistance requirements restricts the initial acquisition strategy that a UE manufacturer can use and the original rationale for this no longer applies. The new note allows the UE manufacturer freedom to use other strategies if desired. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Note 1 is modified to allow the UE manufacturer freedom to use other initial acquisition strategies if desired. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Note 1 will remain unnecessarily restrictive. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Cover sheet revised from R4-2000148 |  |  |  |  |  |  |  |  |

# 6	A-GNSS minimum performance requirements (UE supports other or additional GNSSs)

## 6.0	Introduction

The minimum performance requirements specified in clause 6 apply for UEs that support other A-GNSSs than GPS L1 C/A, or multiple A-GNSSs which may or may not include GPS L1 C/A. The requirements for UEs that support A-GPS L1 C/A only are specified in clause 5.

The A-GNSS minimum performance requirements are defined by assuming that all relevant and valid assistance data is received by the UE in order to perform GNSS measurements and/or position calculation. This clause does not include nor consider delays occurring in the various signalling interfaces of the network.

In the following clauses the minimum performance requirements are based on availability of the assistance data information and messages defined in annexes D and E.

## 6.1	Sensitivity

A sensitivity requirement is essential for verifying the performance of A-GNSS receiver in weak satellite signal conditions. In order to test the most stringent signal levels for the satellites the sensitivity test case is performed in AWGN channel. This test case verifies the performance of the first position estimate, when the UE is provided with only coarse time assistance and when it is additionally supplied with fine time assistance.

### 6.1.1	Coarse time assistance

In this test case 6 satellites are generated for the terminal. AWGN channel model is used.

Table 6.1: Test parameters

| System | Parameters | Unit | Value |
| --- | --- | --- | --- |
|  | Number of generated satellites per system | - | See Table 6.2 |
|  | Total number of generated satellites | - | 6 |
|  | HDOP range |  | 1.4 to 2.1 |
|  | Propagation conditions | - | AWGN |
|  | GNSS coarse time assistance error range | seconds | 2 |
| BDS | Reference high signal power level | dBm | -136 |
|  | Reference low signal power level | dBm | -145 |
| Galileo | Reference high signal power level | dBm | -142 |
|  | Reference low signal power level | dBm | -147 |
| GLONASS | Reference high signal power level | dBm | -142 |
|  | Reference low signal power level | dBm | -147 |
| GPS(1) | Reference high signal power level | dBm | -142 |
|  | Reference low signal power level | dBm | -147 |
| NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. |  |  |  |

Table 6.2: Power level and satellite allocation

|  |  | Satellite allocation for each constellation |  |  |
| --- | --- | --- | --- | --- |
|  |  | GNSS-1(1) | GNSS-2 | GNSS-3 |
| Single constellation | High signal level | 1 | - | - |
|  | Low signal level | 5 | - | - |
| Dual constellation | High signal level | 1 | - | - |
|  | Low signal level | 2 | 3 | - |
| Triple constellation | High signal level | 1 | - | - |
|  | Low signal level | 1 | 2 | 2 |
| Note 1: GNSS-1, i.e. the system having the satellite with high signal level, shall be selected by the device manufacturer. |  |  |  |  |

#### 6.1.1.1	Minimum requirements (Coarse time assistance)

The position estimates shall meet the accuracy and response time specified in Table 6.3.

Table 6.3: Minimum requirements (coarse time assistance)

| System | Success rate | 2-D position error | Max response time |
| --- | --- | --- | --- |
| All | 95 % | 100 m | 20 s |