3GPP TSG-RAN WG4 Meeting #94-e		R4-2002477
Electronic Meeting, 24 Feb. – 6 Mar., 2020

Agenda item:	10.2.3.2

Source:	Keysight Technologies, Spirent Communications

Title:	PSP Correlation between two CE Vendors

Document for:	Information

# Introduction

This contribution provides a brief overview of a PSP correlation exercise between the two CE vendors that have provided PSP simulations in the past.

# Discussion

CE vendors were encouraged to validate and correlate their respective PSP simulations after some discrepancies were observed [1][2][3].

Keysight and Spirent subsequently worked offline on a four-probe system layout published in [4] with the following probe locations

The following simulation assumptions were made:

- Center Frequency: 28GHz

- Scenario&Channel Models: InO&CDL-A and UMi&CDL-C

- Simulated Antenna Pattern Offset: 0cm and 9.17cm (this offset radius is selected such that the entire virtual 4x4 array stays within the 10cm radius test zone)

- Probe Pattern: Omnidirectional

- Range Lengths: 0.75m and 1m

It should furthermore be noted that a 300o (0o) offset direction of the channel model coordinate system in AZ (EL) with respect to the global coordinate system was used and DUT is shifted by of 9.17cm from the centre of the test zone in the direction of 300o.

The PSP simulation results are tabulated in Table 1 and show very good alignment between the two companies.

Table 1: PSP simulation result comparison

| CE Vendor | Array Offset | KS Simulation with Spirent’s 4 probes @1.0m |  | KS Simulation with Spirent’s 4 probes @0.75m |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PSP for InO&CDL-A | PSP for UMi&CDL-C | PSP for InO&CDL-A | PSP for UMi&CDL-C |
| Keysight | 0cm | 92.10% | 88.60% | 92.20% | 88.60% |
|  | 9.17cm | 82.20% | 83.50% | 77.20% | 80.40% |
| Spirent | 0cm | 92.15% | 88.82% | 92.22% | 88.68% |
|  | 9.17cm | 82.19% | 83.20% | 77.17% | 80.29% |

Observation 1: The PSP simulations between the two CE vendors that have provided PSP simulations in the past are aligned

Additionally, the 2 strongest beam locations published in [2] were double checked and confirmed by Keysight. It should be noted that for NR FR2 MIMO, only the 1 strongest beam is generated by the BS. Clearly, very good agreement was observed.

(a)

(b)

Figure 1: Comparison of two strongest beam locations, a) Spirent (from [2]), b) Keysight

Observation 2: Beamforming assumptions are aligned between the two CE vendors that have provided PSP simulations in the past

# Conclusion

The following observations and proposals were made in this contribution

Observation 1: The PSP simulations between the two CE vendors that have provided PSP simulations in the past are aligned

Observation 2: Beamforming assumptions are aligned between the two CE vendors that have provided PSP simulations in the past

# References

- R4-1915062, Range length and probe layout considerations in 5G NR FR2, Spirent Communications, 3GPP TSG RAN WG4 Meeting #93, November 2019

- R4-1915072, Sensitivity of PSP to UE position in test volume, Spirent Communications, 3GPP TSG RAN WG4 Meeting #93, November 2019

- R4-1916175, System Implementation of FR2 3D MPAC Systems, Keysight Technologies, 3GPP TSG RAN WG4 Meeting #93, November 2019

- R4-1907949, On the probe layout on FR2 MPAC systems, Spirent Communications, 3GPP RAN WG4 Meeting #92, August 2019