3GPP TSG-RAN WG4 Meeting #94-e	R4-2002501

Electronic Meeting, Feb.24th - Mar.6th 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4892 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 36.104 Updates of PRACH performance requirements for enhanced HST scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_high_speed_enh2-Perf |  |  |  |  |  | Date: |  |  | 2020-3-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to update requirements for PRACH restricted set type B under high speed train conditions assuming a UE velocity of up to 500km/h. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update SNR values for PRACH restricted set type B performance with 1944 Hz frequency offset |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TBDs are remained. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.141 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of text proposal-------------

8.4.2.1	Minimum requirements

The probability of detection shall be equal to or exceed 99% for the SNR levels listed in Tables 8.4.2.1-1 to 8.4.2.1-5.

The requirements for Burst format 4 are optional and only valid for base stations supporting TDD. The requirements for high speed mode restricted set type A (table 8.4.2.1-2) and high speed mode restricted set type B (table 8.4.2.1-5) are only valid for the base stations supporting high speed mode restricted set A and restricted set type B respectively.

The requirements for coverage enhancement (Tables 8.4.2.1-3 and 8.4.2.1-4) are only valid for the base stations supporting coverage enhancement.

Table 8.4.2.1-1 PRACH missed detection requirements for Normal Mode

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 | Burst format 4 |
| 1 | 2 | AWGN | 0 | -14.2 | -14.2 | -16.4 | -16.5 | -7.2 |
|  |  | ETU 70 Low* | 270 Hz | -8.0 | -7.8 | -10.0 | -10.1 | -0.1 |
|  | 4 | AWGN | 0 | -16.9 | -16.7 | -19.0 | -18.8 | -9.8 |
|  |  | ETU 70 Low* | 270 Hz | -12.1 | -11.7 | -14.1 | -13.9 | -5.1 |
|  | 8 | AWGN | 0 | -19.8 | -19.4 | -21.5 | -21.3 | -11.8 |
|  |  | ETU 70 Low* | 270 Hz | -16.3 | -15.9 | -17.8 | -17.5 | -8.6 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |

The requirements in Table 8.4.2.1-2 shall not be applied to Local Area BS and Home BS.

Table 8.4.2.1-2 PRACH missed detection requirements for High speed Mode restricted set type A

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | AWGN | 0 | -14.1 | -14.2 | -16.3 | -16.6 |
|  |  | ETU 70 Low | 270 Hz | -7.4 | -7.3 | -9.3 | -9.5 |
|  |  | AWGN | 625 Hz | -12.4 | -12.3 | -14.4 | -14.4 |
|  |  | AWGN | 1340 Hz | -13.4 | -13.5 | -15.5 | -15.7 |
|  | 4 | AWGN | 0 | -16.9 | -16.6 | -18.9 | -18.8 |
|  |  | ETU 70 Low | 270 Hz | -11.8 | -11.4 | -13.7 | -13.7 |
|  |  | AWGN | 625 Hz | -14.9 | -14.6 | -16.8 | -16.8 |
|  |  | AWGN | 1340 Hz | -15.9 | -15.5 | -17.8 | -17.8 |
|  | 8 | AWGN | 0 | -19.3 | -19.1 | -20.9 | -21.0 |
|  |  | ETU 70 Low | 270 Hz | -15.6 | -15.1 | -17.0 | -17.0 |
|  |  | AWGN | 625 Hz | -17.7 | -17.4 | -19.3 | -19.4 |
|  |  | AWGN | 1340 Hz | -18.7 | -18.4 | -20.5 | -20.5 |

Table 8.4.2.1-3 PRACH missed detection requirements for coverage enhancement (PRACH frequency hopping OFF)

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | AWGN | 0 | 4 | - | - | -21.3 | -21.1 |
|  |  |  |  | 8 | -21.7 | -21.3 | - | - |
|  |  |  |  | 16 | - | - | -25.1 | -25.0 |
|  |  |  |  | 32 | -25.6 | -25.3 | - | - |
|  |  | EPA1 Low | 270 Hz | 4 | - | - | -12.1 | -11.7 |
|  |  |  |  | 8 | -13.0 | -12.3 | - | - |
|  |  |  |  | 16 | - | - | -17.2 | -17.2 |
|  |  |  |  | 32 | -19.0 | -18.6 | - | - |
| Note 1: Under fading channels, the PRACH detection performance may be significantly different with different PRACH Configuration Indexes. The requirements in this table are defined based on the simulation results with PRACH Configuration Indexes (3, 19, 35, 51) for Format 0, Format 1, Format 2, and Format 3 respectively. |  |  |  |  |  |  |  |  |

Table 8.4.2.1-4 PRACH missed detection requirements for coverage enhancement (PRACH frequency hopping ON)

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | EPA1 Low | 270 Hz | 4 | - | - | -15.5 | -15.3 |
|  |  |  |  | 8 | -16.2 | -15.8 | - | - |
|  |  |  |  | 16 | - | - | -20.1 | -20.2 |
|  |  |  |  | 32 | -21.3 | -21.1 | - | - |
| Note 1: Under fading channels, the PRACH detection performance may be significantly different with different PRACH Configuration Indexes. The requirements in this table are defined based on the simulation results with PRACH Configuration Indexes (3, 19, 35, 51) for Format 0, Format 1, Format 2, and Format 3 respectively. Note 2: The requirements in this table are defined under the assumption that UE RF tuning during PRACH frequency hopping has no impact on the symbols in PRACH subframes and thus all symbols in PRACH subframes are available for the transmission of PRACH preambles. Note 3: The requirements in this table are defined under the assumption that the PRACH frequency offset (prach-FreqOffset-r13) is 0 and frequency hopping offset is -6, where is defined in TS36.211 [12]. Note 4: The requirements in this table apply for channel bandwidth of 5MHz, 10MHz, 15MHz or 20MHz. For channel bandwidth of 3MHz, the requirements in Table 8.4.2.1-3 apply. |  |  |  |  |  |  |  |  |

Table 8.4.2.1-5 PRACH missed detection requirements for High speed Mode restricted set type B

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | AWGN | 0 | -14.5 | -14.1 | -16.7 | -16.8 |
|  |  | AWGN | 625 | -12.0 | -11.7 | -13.9 | -13.9 |
|  |  | ETU 70 Low | 270 Hz | -7.3 | -6.9 | -9.1 | -9.2 |
|  |  | AWGN | 1875 Hz | -11.8 | -11.4 | -13.8 | -14.0 |
|  |  | AWGN | 1944 Hz | [-11.7] | [-11.4] | [-13.8] | [-14.0] |
|  | 4 | AWGN | 0 | -17.1 | -16.6 | -19.1 | -19.1 |
|  |  | AWGN | 625 | -14.4 | -14.1 | -16.1 | -16.2 |
|  |  | ETU 70 Low | 270 Hz | -11.8 | -11.3 | -13.5 | -13.4 |
|  |  | AWGN | 1875 Hz | -14.2 | -13.8 | -15.9 | -16.3 |
|  |  | AWGN | 1944 Hz | [-14.2] | [-13.8] | [-15.9] | [-16.3] |
|  | 8 | AWGN | 0 | -19.6 | -19.1 | -21.2 | -21.2 |
|  |  | AWGN | 625 | -16.4 | -16.3 | -18.1 | -18.2 |
|  |  | ETU 70 Low | 270 Hz | -15.3 | -15.1 | -17.1 | -17.5 |
|  |  | AWGN | 1875 Hz | -16.3 | -16.0 | -18.0 | -18.4 |
|  |  | AWGN | 1944 Hz | [-16.3] | [-16.0] | [-18.0] | [-18.4] |

--------------End of text proposal-------------