3GPP TSG-RAN WG4 Meeting #94-e	R4-2002696

Electronic Meeting, Feb.24th – Mar.6th 2020

Agenda item:			8.14.1.5, 8.14.1.6, 8.14.1.8

Source:	Moderator (Nokia)

Title:	Email discussion summary for RAN4#94e_#23_NR_RF_FR2_req_enh_Part_4

Document for:	Information

# Introduction

Scope of this email discussion is listed in Table 1. No contributions were submitted to AI 8.14.1.5 thus it is out scoped from email discussion.

In this meeting following open issues will be discussed

Sub-topic 1 Intra-band non-contiguous UL CA

1.1 Will non-simultaneous UL for non-contiguous UL CA be part of REL-16

1.2 draft CR for non-contiguous uplink CA including MPR and A-MPR table format

1.3 Beam squint analysis for FR2 PC3 UEs

Sub-topic 2 Improvement of UE MPR

2.1 Will output power boosting be part of REL-16

2.2 Correction of Inner Allocation Definition for Powerclass 3

2.3 Simplification of In-Band Emissions Requirements (Editorial)

2.4 How to ensure consistency between FR1 and FR2 ACLR MBW

Sub-topic 3 Multi-band requirement framework for FR2

3.1 Is MBR definition changed

3.2 from which release if it is

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

| 23 | RAN4#94e_#23_NR_RF_FR2_req_enh_Part_4 | R16 NR FR2 RF | - Intra-band contiguous UL CA - Intra-band non-contiguous UL CA - Improvement of UE MPR - Response to RAN5 LS on MBR | 8.14.1.5, 8.14.1.6, 8.14.1.8, and R4-2000021, R4-2000022, R4-2000235, R4-2000236, R4-2002091, R4-2002104, R4-2000200, R4-2000201, R4-2000202,R4-2000526, R4-2001234 |
| --- | --- | --- | --- | --- |

# Topic #1: Intra-band non-contiguous UL CA

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2000019 | Apple Inc. | TDoc: Non-simultaneous UL for non-contiguous UL CA in FR2 Observation 1: Aggregation of non-contiguous uplink carriers in FR2 into a simultaneous transmission has the potential to introduce a significant penalty to the UL link budget. Observation 2: Aggregation of non-contiguous uplink carriers in FR2 into a non-simultaneous transmissions can give the network additional scheduling flexibility in terms of coverage spectrum utilization. Proposal 1: For SA CA scenarios FR1 (PCell) + FR2 (SCells) and EN-DC scenarios LTE (PCell) + FR1 (PScell) + FR2 (SCells), RAN4 confirms the feasibility of non-simultaneous Tx for FR2 intra-band NC UL CA and agrees to introduce a capability for the UE indicate to the network that it only supports non-simultaneous UL operation in NC UL CA. Proposal 2: Further effort on NSU requirements can proceed according to the Rel-16 NR RRM work item scope and associated work plan. Proposal 3: Non-simultaneous Tx for UL intra-band non-contiguous CA can support operation with the same frequency separation as the DL CCs; thus, the entire frequency separation class table can be applicable to UL CA with non-simultaneous operation. |
| R4-2000509 | Nokia, Nokia Shanghai Bell | TDoc: FR2 nc-in-ca MPR This is a discussion paper providing MPR simulation results for non-contiguous UL CA. Simulated cases are 50+50, 100+100, 200+200 and 100+100+100 MHz. Gating factor for MPR is mainly OBW and spurious emissions in case allocation is such that all sub-blocks have very narrow allocation. It seems that MPR can be defined agnostic of modulation however such way that CP-OFDM and DFT-s-OFDM could have different MPRs. MPR is limited to max 5 dB in all cases. |
| R4-2000693 | Qualcomm Incorporated | TDoc: On using Rel-15 CA MPR table format for FR2 NC UL CA Observation 1: CABW does not distinguish between the UE’s separate capabilities to support bidirectional DL spectrum and DL-only spectrum, respectively. Observation 2: For UEs with common R/T LOs, back-off requirements depend on the size of the bidirectional spectrum and remain insensitive to the presence of additional parallel Rx chains to support DL-only spectrum. Observation 3: For Rel-16, CABW is obsolete as the determining parameter for CA MPR. Observation 4: For UEs with distinct R/T LOs, back-off requirements are tied to the Tx IF BW, which is determined by the UL CA configuration alone, and independent of DL CA configuration Observation 6: For UEs with common R/T LOs, DL frequency separation can replace CABW in the MPR tables for CA operation. Observation 7: UEs with distinct R/T LOs can also use DL frequency separation to determine MPR Observation 8: For UEs with distinct R/T LOs, if DL frequency separation replaced CABW in the MPR tables, MPR for DL frequency separation >1400MHz can adopt values used for 1400MHz Proposal 1: MPR and A-MPR tables shall be changed to depend on ‘DL frequency separation’, instead of ‘Cumulative Aggregated BW’ Proposal 2: MPR (and A-MPR, if appropriate) tables for NC CA shall accommodate DL frequency separation larger than 1400MHz. Contribution proposes a template for NC CA MPR table for Rel-16. |
| R4-2000694 | Qualcomm Incorporated | TDoc: dCR to 38.101-2: Simultaneous NC UL CA framework for Rel-16 Reason for change: Introduce framework to accommodate simultaneous Rel-16 NC UL CA feature: 1. Generalize section 5.3A for non-contiguous UL CA 2. Removal of dependence on CABW due to obsolescence R4-2000693. DL frequency separation is equivalent to CABW, and adopting DL FS decouples DL CA BW enhancement to UL CA 3. Introduction of supporting definitions Summary of change: 1. Introduce definition of ‘frequency separation’ 2. Introduce MPR requirements table for NC CA, values left for consensus in a future meeting. 3. Modify requirements that depend on CABW and replace with DL aggregated BW. 4. Modify MPR section for contiguous UL CA to make consistent with removal of CABW 5. Modify A-MPR section to make consistent with removal of CABW |
| R4-2002147 | Qualcomm Incorporated | TDoc: Beam squint analysis for FR2 PC3 UEs Proposal: RAN4 to discuss how to capture consideration for radiative degradation mechanisms like beam squint for larger frequency separation. |
| R4-2000396 From AI 8.14.1.8 | Intel Corporation | TDoc: FR2 CA MPR improvement Proposal: In Rel-16 for both UL contiguous and non-contiguous CA, MPR values for 1400MHz <CABW ≤ 2400MHz should be kept the same with their corresponding MPR values for 800MHz ≤ CABW ≤ 1400MHz. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Non-simultaneous UL for non-contiguous UL CA in FR2

Issue 1-1-1: Non-simultaneous UL for non-contiguous UL CA in FR2

- Proposals

  - Option 1: RAN4 confirms the feasibility of non-simultaneous Tx for FR2 intra-band NC UL CA and agrees to introduce a capability for the UE indicate to the network that it only supports non-simultaneous UL operation in NC UL CA.

  - Option 2: Non-simultaneous UL for non-contiguous UL CA in FR2 is not part of REL16 and is removed from the WID

- Recommended WF

  - TBA

### Sub-topic 1-2: Simultaneous UL for non-contiguous UL CA in FR2

Issue 1-2-1: MPR and A-MPR tables shall be changed to depend on ‘DL frequency separation’, instead of ‘Cumulative Aggregated BW’

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - TBA

Issue 1-2-2: MPR (and A-MPR, if appropriate) tables for NC CA shall accommodate DL frequency separation larger than 1400MHz

- Proposals

  - Option 1: Yes

  - Option 2: Yes and MPR values for 1400MHz <CABW ≤ 2400MHz should be kept the same with their corresponding MPR values for 800MHz ≤ CABW ≤ 1400MHz.

  - Option 3: No

- Recommended WF

  - TBA

Issue 1-2-3: Beam squint analysis for FR2 PC3 UEs

- Proposals

  - Option 1: RAN4 to discuss how to capture consideration for radiative degradation mechanisms like beam squint for larger frequency separation.

  - Option 2: No need to discuss how to capture consideration for radiative degradation mechanisms like beam squint for larger frequency separation.

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Qualcomm | Sub topic 1-1: Non-simultaneous UL for non-contiguous UL CA in FR2: - We request clarification on how the RF requirement would look with NSU. Which specs are affected and what changes are foreseen? Sub topic 1-2:NC UL CA: - We agree with the general idea that CABW should not drive MPR. The MPR proposal of 1-2-2 comes naturally if DL frequency separation (which is equivalent to a UE’s bidirectional spectrum) is used to drive CA MPR instead of CABW |
| Intel | ### Sub-topic 1-1: Non-simultaneous UL for non-contiguous UL CA in FR2 - Suggest adding an option to allow UE capability signalling to do non-contiguous or contiguous UL CA ### Sub-topic 1-2: Simultaneous UL for non-contiguous UL CA in FR2 Issue 1-2-2: MPR (and A-MPR, if appropriate) tables for NC CA shall accommodate DL frequency separation larger than 1400MHz - Option 2: Yes and MPR values for 1400MHz <CABW ≤ 2400MHz should be kept the same with their corresponding MPR values for 800MHz ≤ CABW ≤ 1400MHz. Issue 1-2-3: Beam squint analysis for FR2 PC3 UEs - Option 1: RAN4 to discuss how to capture consideration for radiative degradation mechanisms like beam squint for larger frequency separation. |
| Ericsson | ### Issue 1-1-1: the virtue of the NSU switching alternatives and the additional (non) capability should be assessed against to a baseline with a single contiguous UL block and use of Pcell/Scell RRC reconfiguration for switching the UL block before feasibility is confirmed. What is the expected impact on the RF specifications? |
| Apple | ### Issue 1-1-1: Option 1 is our preference. In our understanding, the intra-band contiguous requirements are applicable to the NSU capable UE. As we described in 0019, the UE supporting NSU can remove all additional constraints on CC configuration by the network: Issues 1-2-1, 1-2-2: We agree to further discuss the CABW applicability of the MPR requirements and to consider the DL Fs formulation. Issue 1-2-3: We are fine with Option 1 |
| Huawei | Sub topic 1-2: Issue 1-2-1: Option 2. Different RF architecture can be used, e.g. for “no DL-only limitation” UEs, this replacement is not correct. Issue 1-2-2: Currently Option 3. It may depends on RF architecture, before we decides on the separation class issue in NC DL CA, we are actually not sure how to define this part. We will update the information, see NC DL CA discussion thread. Issue 1-2-3: Option1. |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2000694 | Nokia: Would be good to add the agreed non-contiguous uplink CA transmitter requirements in to this dCR so that this contribution can be used as a basis to feature CR in coming meetings once the MPR is agreed. |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1-1 | Non-simultaneous UL for non-contiguous UL CA in FR2: Companies: Clarification was asked how RF requirements are impacted by NSU and a suggestion on adding an option to allow UE capability signalling to do non-contiguous or contiguous UL CA. Moderator: Is not this capability proposal the current UL CA configuration signalling as UE can signal support for contiguous and/or non-contiguous CA? Tentative agreements: None Candidate options: Situation same, both option 1 and option 2 are discussed. Recommendations for 2nd round: Apple to clarify how RF requirements would be impacted by NSU. |
| Sub-topic#1-2-1 | MPR and A-MPR tables shall be changed to depend on ‘DL frequency separation’, instead of ‘Cumulative Aggregated BW’ Companies: There was support for proposal but also one company was against as they say that Different RF architecture can be used, e.g. for “no DL-only limitation” UEs, this replacement is not correct. Tentative agreements: None Candidate options: Situation same, both option 1 and option 2 are discussed. Recommendations for 2nd round: Qualcomm to answer the comment that proposal is not valid for all RF architecture. |
| Sub-topic#1-2-2 | MPR (and A-MPR, if appropriate) tables for NC CA shall accommodate DL frequency separation larger than 1400MHz Moderator: This sub-topic is pending decision on 1-2-1. However even in case that MPR remains based on CABW there was a proposal how to define MPR beyond 1400 MHz Tentative agreements: None, need to wait 1-2-1 Candidate options: Situation same, all options are discussed. Recommendations for 2nd round: Wait 1-2-1 |
| Sub-topic#1-2-3 | Beam squint analysis for FR2 PC3 UEs Companies: All comments in favour of option 1: RAN4 to discuss how to capture consideration for radiative degradation mechanisms like beam squint for larger frequency separation. Tentative agreements: RAN4 shall discuss how to capture consideration for radiative degradation mechanisms like beam squint for larger frequency separation. Candidate options: Recommendations for 2nd round: WF assigned to Qualcomm to discuss how to proceed this topic in next RAN4 meeting. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Consideration for radiative degradation mechanisms for larger frequency separation | Qualcomm |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Improvement of UE MPR

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2000518 | Nokia, Nokia Shanghai Bell | TDoc: FR2 boosting Proposal 1: No output power boost is specified for PI/2 PBSK due to suspended IBE in REL16 Proposal 2: No output power boost is specified for QPSK due to suspended IBE in REL16 |
| R4-2002091 | Motorola Mobility España SA | TDoc: Correction of Inner Allocation Definition for Powerclass 3 This is a CAT F CR. Reason for change: There are conflicting definitions of the the inner allocations for Powerclass 3 with bandwidth configurations <= 200 MHz. The second definition of the inner allocations conflicts with first definition. Summary of change: Removes the second definition of inner allocations |
| R4-2002104 | Motorola Mobility España SA | TDoc: Simplification of In-Band Emissions Requirements This CAT D editorial CR ís not strictly speaking for MPR improvement but it was never the less allocated to this email thread. Given that it is CAT D should not be too big issue to have it here. Reason for change: The first term in the general in-band emissions requirement is no longer needed as this term is <= -25 dB and Note 1 sets the lower limit as no less than -25 dB. As a result, the first term of the general in-band emissions requirement is never used. |
| R4-2002144 | Skyworks Solutions, Inc. | TDoc: FR2 ACLR Measurement Bandwidth Definition This is a discussion paper and suggests following. Two possible options may be considered to ensure MBW is centered on TxBW. - Option 1: modify FR2 MBW according to Table 3 - Option 2: maintain existing FR2 MBW, and change the text defining the center frequency of the measurement BW for the wanted channel so that MBW becomes centered on the TxBW. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1:Output power boosting when IBE is suspended

Issue 2-1-1: Output power boosting when IBE is suspended for PI/2 PBSK

- Proposals

  - Option 1: This feature is not part of REL16 and is removed from the WID

  - Option 2: This feature is kept in WID

- Recommended WF

  - TBA

Issue 2-1-2: Output power boosting when IBE is suspended for QPSK

- Proposals

  - Option 1: This feature is not part of REL16 and is removed from the WID

  - Option 2: This feature is kept in WID

- Recommended WF

  - TBA

### Sub-topic 2-2: Correction of Inner Allocation Definition for Powerclass 3

Issue 2-2-1: Inner Allocation Definition for Powerclass 3

- Proposals

  - Option 1: CR is agreed

  - Option 2: CR is not agreed

- Recommended WF

  - Option 1

### Sub-topic 2-3: Simplification of In-Band Emissions Requirements

Issue 2-3-1: Simplification of In-Band Emissions Requirements

- Proposals

  - Option 1: CR is agreed

  - Option 2: CR is not agreed

- Recommended WF

  - Option 1

### Sub-topic 2-4: FR2 ACLR MBW

Issue 2-4-1: FR2 ACLR MBW

- Proposals

  - Option 1: modify FR2 MBW according to Table 3 in R4-2002144

  - Option 2: maintain existing FR2 MBW, and change the text defining the center frequency of the measurement BW for the wanted channel so that MBW becomes centered on the TxBW.

  - Option 3: Do nothing

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Qualcomm | Sub topic 2-1: Output power boosting when IBE is suspended: - Pi/2 BPSK. As an optional feature that the UE can support, we think there is potential for MPR reductions, even if min peak EIRP is not changed. We will bring a proposal in a future meeting, pending further study - We think power boosting with QPSK is feasible and useful for most FR2 deployments which are UL limited. Sub topic 2-4: If ACLR MBW must be changed, we prefer option 1 |
| Anritsu | Sub topic 2-4: FR2 ACLR MBW (R4-2002144) We share the same concern with Skyworks. From a viewpoint of the test case implementation, both option 1 and 2 are doable. But we prefer option 1 from the consistency between FR1 and FR2. |
| Intel | ### Sub-topic 2-1:Output power boosting when IBE is suspended Issue 2-1-1: Output power boosting when IBE is suspended for PI/2 PBSK - Option 1: This feature is not part of REL16 and is removed from the WID Issue 2-1-2: Output power boosting when IBE is suspended for QPSK - Option 1: This feature is not part of REL16 and is removed from the WID |
| Verizon | For Sub-topic 2-1: Our preference is to keep this feature in Rel-16 and maintain it in WID, at least for QPSK. The main our concern is FR2 UE uplink limitation in our deployment. And, we believe the system parameters could be potential omitted further. |
| Huawei | Sub topic 2-1: Output power boosting when IBE is suspended: Option 2. For Pi/2 BPSK power boosting, it is initiated by DMRS sequence revision in RAN1, it is under eMIMO WI, should not discuss in this sub topic. For QPSK, it can be completed with the conclusion no power boosting with IBE relaxation. Sub topic 2-4: FR2 ACLR MBW (R4-2002144) Huawei: clarify it is for Rel-15? For option1, would like to know if UE do not support 60kHz SCS, how we define the MBW? 120kHz is the reference waveform for FR2. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2002091 | Qualcomm: While we agree with the change, we wonder why v16.2 has diverged from 15.8. We think the 2 versions need to be aligned. We seek comments from other companies. |
|  | Skyworks: We agree with this change. This should have been spotted at R4#93. To Qualcomm: The changes introduced in Rel-16.2.0 are the improvement of the 0dB MPR Region, changes that were agreed to apply to Rel’16 only. |
|  | Intel: Agree with the CR |
| R4-2002104 | Qualcomm: We agree with the change |
|  | Skyworks: if the intention is to remove redundant information, our preference would be to keep IBE emission mask equations intact and modify note 1 instead. If the group prefers to change all IBE mask equations and to keep Note 1 intact, then we observe that in all power class tables of 38.101-2, the “PRB” notation is not aligned with the Note 10 notation and also not aligned with 38.101-1 Note1/Note 10: the “bar” is missing above this term. In that case a CR is needed in 38.101-2 16.2.0 Note 1 for all power classes (Tables 6.4A.2.3.2-1, 6.4A.2.3.3-1, 6.4A.2.3.4-1, 6.4A.2.3.5-1). Example: - 38.101-2 Note 1 uses notation “PRB “ and refers to Note 10 in which the following notation is used: -38.101-2 “NOTE 10:is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm“. |
|  | Intel: Agree with the CR |
|  | Huawei: prefer to keep, first item has its physical significance. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1-1 and 2-1-2 | Output power boosting when IBE is suspended Companies: There was both support to keep boosting in WID and not to keep. Tentative agreements: None Candidate options: Recommendations for 2nd round: Continue discussion in next RAN4 meeting. |
| Sub-topic#2-2-1 | Inner Allocation Definition for Powerclass 3 Companies: Support was expressed to CR R4-2002091. Question was raised on applicability to REL15 and it was answered that MPR enhancement is REL16 feature. Tentative agreements: CR R4-2002091 Candidate options: Recommendations for 2nd round: Agree CR R4-2002091 |
| Sub-topic#2-3-1 | Simplification of In-Band Emissions Requirements Companies: Two companies support the CR, one provided alternative and one prefers not to change. Tentative agreements: None Candidate options: R4-2002104 or revision of it or no change. Recommendations for 2nd round: Continue discussion in next RAN4 meeting. |
| Sub-topic#2-4-1 | FR2 ACLR MBW Companies: Support for option 1 was majority. One company as a clarification questions. Tentative agreements: Modify FR2 MBW according to Table 3 in R4-2002144 Candidate options: Recommendations for 2nd round: Skyworks to answer the questions from Huawei and if issues are solved assign WF or CR to Skyworks. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | FR2 ACLR MBW | Skyworks |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2002091 | CR is agreeable |
| R4-2002104 | CR is postponed |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: Multi-band requirement framework for FR2

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2000021 | Apple Inc. | TDoc: Multi-band requirement framework for FR2 in Rel-16 and beyond Proposal 1: RAN4 shall introduce a maximum cap to the per-band relaxation factors, such that ∆MBP,n ≤ 0.75 dB and ∆MBS,n ≤ 0.75 dB. Proposal 2: RAN4 shall introduce the changes to MBR in the Rel-15 specification. |
| R4-2000022 | Apple Inc. | TDoc: [draft] LS response on Multiband relaxation for FR2 RAN4 informs RAN5 that - RAN4 shall introduce a maximum cap to the per-band relaxation factors, such that ∆MBP,n ≤ 0.75 dB and ∆MBS,n ≤ 0.75 dB - RAN4 shall introduce the changes to MBR in the Rel-15 specification. |
| R4-2000235 | Apple Inc. | TDoc: Correction of the FR2 multi-band requirement framework This is a CAT F CR to introduce the proposals in R4-2000021 |
| R4-2000200 | Qualcomm Incorporated, Verizon, Ericsson, Sony, Samsung | TDoc: On LS from RAN5 on multi-band relaxations Observation 1: RAN5 concludes that the RAN4 framework of flexible band-specific relaxations is not feasible to implement Observation 2: RAN5 concludes that it is untenable that RAN4 retains flexibility to increase the maximum allowed MBP and MBS in the future Observation 3: Instituting an upper limit on MBP and MBS is neither feasible nor a sufficient solution Observation 4: Instituting per-band limits (MB) on multiband relaxation solves RAN5’s problems with MBR, while practically preserving cumulative relaxations (MB) and allowing future growth in supported bands. Proposal 1: Change Rel-15 multiband relaxation framework from table 6.2.1.3-4 in TS38.101-2 v15.8 to per-band allowance in table below: Band; MBP (dB); MBS (dB). n257; 0.73; 0.73. n258; 0.6; 0.7. n260; 0.51; 0.41. n261; 0.52,4; 0.74. Note 1: n260 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n260 Note 2: n261 peak relaxation is 0 dB for UE that exclusively supports n261+n260 Note 3: n257 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257 Note 4: n261 peak and spherical relaxations are 0 dB for UE that exclusively supports n261+n257. |
| R4-2000201 | Qualcomm Incorporated | TDoc: Reply to LS R5-199424 on FR2 Multiband Relaxations |
| R4-2000202 | Qualcomm, Verizon, Ericsson, Sony, Samsung | TDoc: CR to 38.101-2: Revision to Multiband Relaxations REL15 CAT F CR to introduce the content of R4-2000200 |
| R4-2000203 | Qualcomm, Verizon, Ericsson, Sony, Samsung | TDoc: CR to 38.101-2: Revision to Multiband Relaxations REL16 CAT A CR to introduce the content of R4-2000200 |
| R4-2000526 | Nokia, Nokia Shanghai Bell | TDoc: Discussion on RAN5 LS on Multiband relaxation for FR2 Proposal: Multi-band relaxations are removed from REL-16 38.101-2 and are not applicable for REL-16 and beyond UEs. Inform RAN5. |
| R4-2001234 | OPPO | TDoc: About multi-band relaxation tests Observation 1: RAN5 found the problem in multi-band relaxation, i.e. UE may declare support several bands and apply larger relaxation values but actually only certify part of the supported bands. Observation 2: GCF might confront with the issue found by RAN5 due to its certification mechanism, while PTCRB can solve it by testing all the bands that UE supports. Observation 3: Any changes to the multi-band relaxation will probably cause other requirements are impacted. Proposal 1: Encourage GCF could adopt flexible solutions in their certification, for example like PTCRB does to avoid the multi-band relaxation certification issue. Observation 4: RAN5 could use relatively larger relaxation value at the beginning to avoid of re-evaluating issue. Proposal 2: Encourage RAN5 to adopt a relatively larger values from the testability perspective at beginning to avoid possible future new bands caused testability re-evaluation issue. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: Multi-band requirement framework for FR2

Issue 3-1-1: Does Multi-band requirement framework needs some enhancements

- Proposals

  - Option 1: No because GCF could adopt flexible solutions in their certification and RAN5 adopt a relatively larger values from the testability perspective at beginning

  - Option 2: Yes, some enhancements are needed

- Recommended WF

  - TBA

Issue 3-1-2: If Multi-band requirement framework needs some enhancements

- Proposals

  - Option 1: Keep current format but introduce a cap to the per-band relaxation factors

  - Option 2: Change the format as in R4-2000200. Relaxation is defined per band and it is same no matter what band combinations UE supports, with the exceptions listed in notes. Change is from REL-15

  - Option 3: Remove the Multi-band requirement framework from REL16 specification

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| Qualcomm | Sub topic 3-1: Does MBR framework need to be changed: Under the circumstances, we think it is RAN4 responsibility to accommodate RAN5 request. While details differ between proposals, there seems to be wide consensus on how to reframe MBR to address RAN5 concerns. We believe the reframing should be applicable to rel-15. We think the CR proposals are close and convergence is possible in this meeting. |
| OPPO | Issue 3-1-1: Suggest option 1, our understanding is that the UE certification issue can be solved by solutions like PTCRB has done, i.e. test the bands that UE support, while the RAN5 test ability issue can be done by utilizing a relative larger relaxation values to avoid re-open the discussion in RAN5. About the exact value, it seems RAN5 applies the max MBR to single band to calculate the testability. If this is the case, then the cap per-band that RAN5 can use could up to 1.7dB which is the max MBR now in Rel-15. RAN4 could inform RAN5 about this if needed. Finally, changing RAN4 multi-band relaxation requirements in Rel-15 shall be avoided. |
| Intel | Issue 1-1: Rather than an enhancement, perhaps it is more fitting to say framework modification or simplification. From our perspective, if Option 1 is found to be too difficult, we are ok to simplify the existing relaxations and address RAN5’s concerns. Issue 1-2: We are not ok with Option 3. We are ok to further discuss Option 1 and Option 2, as the options are well aligned following a per-band relaxation approach to address RAN5’s concerns and simplify the current multi-band framework. We are open to either option 1 or option 2; values presented in each option are reasonable. We have a slight preference for option 2, as the per-band relaxation values are different for each band in this option, allowing us to define the per-band relaxation values of new bands as they are introduced. |
| NTT DOCOMO, INC. | Sub-topic 3-1: For Rel-16, we would like to remove the multi-band relaxation requirement. For Rel-15, in order to solve the issue raised from RAN5 (R5-199424), we should specify per band relaxation without multi-band framework. Otherwise, RAN5 cannot fully ensure correct conformance test implementation of multi-band relaxation as described in problem description 1 in R5-199424. We would like to note that even if we put per band limitation while multi-band relaxation framework still exist, the issue may get smaller but is not solved at fundamental level. We have a comment on R4-2000200. Per band relaxation values are estimated based on Table 2.2.2-2, but some multi-band relaxation values in current specification seems to have errors and should be corrected regarding the relaxation value including n257 and n261, that is, the relaxation value of Band X+n257+n261 should be the same that of Band X + n257 since the frequency range of n261 is covered by n257. This is an error of current specification, but your proposed values may change accordingly if these values are corrected. |
| Nokia | Issue 3-1-1: We would prefer option 2. Reason being that RAN5 seeks help and secondly we think that maintaining the MBR scheme in future when more FR2 bands come will be significant burden. |
| MediaTek | Sub topic 3-1: Issue 3-1-1: Does Multi-band requirement framework needs some enhancements The reason that why multiband relaxation was defined as a pool for all supported bands is because during core requirement discussion, companies thought it is needed to reserve the UE implementation flexibility in my understanding. Moreover, while RAN4 can clearly define the multiband relaxation framework and rule, it doesn’t make sense to say there is potential certification issue. Issue 3-1-2: If Multi-band requirement framework needs some enhancements We are not okay for Option-3, but we are okay to discuss “cap” concept as Option 1. |
| Ericsson | Issue 3-1-1: With regard to LS from RAN5, enhancement of multiband relaxation framework is necessary for both Rel-15 and Rel-16. Issue 3-1-2: option 2 |
| Verizon | For Issue 3-1: We prefer to remove the multi-band relaxation requirements and specify the requirement per band. Actually, RAN5-suggested per-band relaxations proves a good platform to recast multiband relaxations with implementation criteria. The intention to adopt a relatively larger values from the testability is not helpful to implement the criteria requirement and the future scalability. |
| Apple | Issue 1-1: Based on the RAN5 LS, Option 2 seems to be the correct approach to address the concerns. Issue 1-2: We would like to echo the Qualcomm comments that convergence is possible based on Options 1 and 2. We do not view Option 3 as a feasible approach. |
| SONY | Issue 3-1-1: Option 2, enhancement is needed. Issue 3-1-2: Option 2. We think the CR proposal is reasonable and potential agreement can be made during this meeting. |
| Huawei | Issue 1-1: suggest option 1. But open on the enhancement. Introduce a cap seems better than define a per-band relaxation. For the cap value, we doubt whether TE can measure with 0.05dB granularity. And MBR is for design flexibility, prefer a larger upper cap than 0.75dB. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2000235 | Nokia: Prefers to have more discussion how to solve RAN5 issues and simplify RAN4 work. For example, solution proposed in R4-2000202. |
|  | Company B |
| R4-2000202 | Nokia: We support this simplification of MBR framework |
|  | Company B |
| R4-2000203 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1-1 | Does Multi-band requirement framework needs some enhancements Companies: Most of the companies think that RAN4 needs to make a change to MBR while minority thinks RAN5 can solve the issues. Tentative agreements: Candidate options: Recommendations for 2nd round: RAN4 discusses how to change the MBR framework. |
| Sub-topic#3-1-2 | If Multi-band requirement framework needs some enhancements how to do those Companies: All three options got support. Tentative agreements: None Candidate options: Recommendations for 2nd round: SONY to prepare WF to agree how to change the MBR framework in order to solve RAN5 problems. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on multiband relaxation framework | SONY |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2000235 | return to |
| R4-2000202 | return to |
| R4-2000203 | return to |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |