3GPP TSG-RAN WG4 Meeting #94bis-e                                R4-2003156

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI

Title:	TP for TR 37.716-21-21: DC_1-41_n3-n257

Agenda item:			8.7.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.716-21-21 to include DC_1A-41A_n3A-n257A, DC_1A-41C_n3A-n257A, DC_1A-41A_n3A-n257I and DC_1A-41C_n3A-n257I according to the request in [1].

# 2. Reference

- RP-200080, Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL), LG Electronics.

# 3. Text Proposal

<Start of Text Proposal>

## 7.x	DC_1-41_n3-n257

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A_n3A-n257A DC_1A-41C_n3A-n257A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 7.x.2	Channel bandwidths per operating band for DC

Table 7.x.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  |  |  |  |  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  |  |  | E-UTRA and NR Band |  | SCS (kHz) |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-41A_n3A-n257A |  | DC_41A_n3A DC_41A_n257A |  |  |  | 1 |  |  | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 470 |  |
|  |  |  |  |  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n3 |  |  | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C_n3A-n257A |  | DC_41A_n3A DC_41A_n257A DC_41C_n3A DC_41C_n257A |  |  |  | 1 |  |  | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 490 |  |
|  |  |  |  |  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n3 |  |  | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41A_n3A-n257I |  | DC_41A_n3A DC_41A_n257A DC_41A_n257I |  |  |  | 1 |  |  | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 470 |  |
|  |  |  |  |  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n3 |  |  | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-41C_n3A-n257I |  | DC_41A_n3A DC_41A_n257A DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257I |  |  |  | 1 |  |  | Please see BCS of LTE CA in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 490 |  |
|  |  |  |  |  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n3 |  |  | Please see BCS of NR CA in TS 38.101-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | n257 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.x.3	Co-existence studies

Co-existence studies of DC_1-41_n3-n257 and protected bands are already covered in the constituent fall-back modes.

### 7.x.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values DC_1-41_n3-n257 are given in the tables below, numbers come from  DC_1-41_n3.

Table 7.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3-n257 | 1 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3-n257 | 1 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n257 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.x.5	MSD

All MSD requirements for DC_1-41_n3-n257 are already covered by constituent fallback modes. No additional MSD requirement is needed.

<End of Text Proposal>