3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2003236

e-meeting 20th – 30th April 2020

Source: 	Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc.

Title: 					Sub-block edges and ACLR MBW for FR1 contiguous UL CA

Agenda Item: 			6.13.1.3

Document for:	Approval

# 1	Introduction

Previously, it has been observed for FR1 contiguous UL CA that the current formulas in 38.101-1 V16.3.0 may result in aggregated channel bandwidths greater than the sum of CC bandwidths. This is not acceptable. In this contribution, we propose new, simple formulas for Foffset,low and Foffset,high to solve this problem. The resulting aggregated channel bandwidth is independent of the applied SCS values.

The CA channel spacing depends on the operating band, which makes it hard to find MPR values that are guaranteed to be valid for all operating bands. We propose a way to find sufficient MPR values without simulating operating bands separately.

Previously proposed formulas for CA ACLR measurement bandwidth may not cover all RBs in all cases. We propose new formulas for the CA ACLR measurement bandwidth and frequency offset. These are independent of the used SCS and guarantee that all RBs are covered in the ACLR measurement.

# 2	Discussion

## 2.1	Sub-block edges

In [1], we pointed out problems with the current formulas for Foffset,low and Foffset,high defined in TS 38.101-1 subclause 5.3A.3. The aggregated channel bandwidth may exceed the sum of channel bandwidth, which is not acceptable.

TS 38.101-1 subclause 5.4A.1 specifies the nominal channel spacing as follows:

### 5.4A.1	Channel spacing for CA

For intra-band contiguous carrier aggregation with two or more component carriers, the nominal channel spacing between two adjacent NR component carriers is defined as the following unless stated otherwise:

For NR operating bands with a 100 kHz channel raster:

while for NR operating bands without a 100 kHz channel raster:

with

n = µ0

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective NR component carriers according to Table 5.3.2-1 with values in MHz, μ0  is the largest μ value among the subcarrier spacing configurations supported in the operating band for both of the channel bandwidths according to Table 5.3.5-1 and GBChannel(i) is the minimum guard band for channel bandwidth i according to Table 5.3.3-1 for the said μ value with μ as defined in TS 38.211.

Here, GBChannel(1)-GBChannel(2) is a guard exchange term. It reduces the channel spacing in order to make room for increasing the narrower outer CC guard to make the sub-block guard widths equal.

TS 38.101-1 subclause 5.3A.3 currently specifies:

The lower and upper frequency offsets depend on the transmission bandwidth configurations of the lowest and highest assigned edge component carrier and are defined as

Foffset,low = (NRB,low*12 + 1)*SCSlow/2 + BWGB (MHz)

Foffset,high = (NRB,high*12 - 1)*SCShigh/2 + BWGB (MHz)

BWGB = max(BWGB,Channel(k))

BWGB,Channel(k) is the minimum guard band defined in clause 5.3.3 of carrier k, while NRB,low and NRB,high are the transmission bandwidth configurations according to Table 5.3.2-1 for the lowest and highest assigned component carrier, SCSlow and SCShigh are the sub-carrier spacing for the lowest and highest assigned component carrier respectively.

The current formulas of Foffset,low and Foffset,high are problematic because the used minimum guard band widths are different from those used for the nominal channel spacing in subclause 5.4A.1. As a result of this discrepancy, the aggregated channel bandwidth may exceed the sum of CC bandwidths, which is unacceptable. Furthermore, the sub-block edges should not depend on the used SCSs. In general, the edges of frequency channel should not depend on what is happening inside the channel.

To overcome this problem, we propose to rewrite the formulas of Foffset,low and Foffset,high in Subclause 5.3A.3 in a simple and straightforward way which guarantees that the aggregated channel bandwidth cannot exceed the sum of CC bandwidths.

Proposal 1: Specify Foffset,low and Foffset,high as follows:

Define the guard exchange term as

GB=GBChannel(2)-GBChannel(1)

where GBChannel(n) (n=1, 2) are the minimum guard band widths used in subclause 5.4A.1.

If GB>0,

Foffset,low=BWChannel12+GB

Foffset,high=BWChannel22,

else

Foffset,low=BWChannel12

Foffset,high=BWChannel22+GB.

Properties of Proposal 1:

- The absolute guard exchange term GB is the same as the one in the formula of nominal channel spacing under subclause 5.4A.1.

- The formulas result in approximately equal guard band widths at the sub-block edges for SCS=2μ0∙15 kHz, with μ0 as defined in TS 38.101-1 subclause 5.4A.1. For other SCSs, the sub-block guards differ in most cases.

- The aggregated channel bandwidth will never exceed the sum of CC bandwidths. This is guaranteed by using the same guard exchange term (same SCS) in the formulas of both nominal channel spacing and Foffset,low and Foffset,high.

- The aggregated channel bandwidth is always
 BWChannel_CA=GBChannel(1)+GBChannel(2)FRaster_CAFRaster_CA, 
where FRaster_CA=LCMFRaster, 20∙15 kHz, FRaster is the channel raster of the operating band, and LCM(∙) stands for least common multiple.

## 2.2	Implications on determining the MPR

Specified MPR values must be valid for all operating bands. Therefore, it is unfortunate that the nominal channel spacing and sub-block guards depend on the operating band. It is infeasible to run simulations for the MPR separately for each operating band. Furthermore, the specified MPR should be valid also for operating bands possibly specified in the future. Therefore, what is needed is a method to simulate or measure the MPR in a way that the results will be valid for all current and future operating bands.

This problem can be solved by using modified definitions for nominal channel spacing as well as Foffset,low and Foffset,high.

- In the definition of the guard exchange term, use the minimum guard bandwidths of the SCS which minimizes GB. This maximizes the channel spacing and thus maximizes the reach of spectral regrowth and IMD.

- In the definition of Foffset,low and Foffset,high, use the above-mentioned modified GB. This minimizes the guard band of one of the edge CCs, again contributing to finding the highest MPR among operating bands.

Properties of the parameters modified for determining the MPR:

- The combination of nominal channel spacing and sub-block guards might match the specified ones at none of the operating bands.

- The channel spacing may be larger than the specified nominal channel spacing for some operating bands.

- The guard on either edge of the sub-block may be narrower than specified for some operating bands.

- The aggregated channel bandwidth never exceeds the sum of CC bandwidths.

## 2.3	ACLR measurement bandwidth and frequency offsets

If the WF [2], the proposed ACLR MBW might not cover all RBs. Especially if applied in measuring the transmitted power, this would cause erroneous results for some allocations.

To simplify CA ACLR measurements, it is preferable to define a single measurement bandwidth for each bandwidth combination. This must be a superset of all RBs in all SCS combinations. To achieve this, we can reuse the single-channel ACLR MBWs, which were designed to cover all RBs of all SCSs.

ACLR alternative A: asymmetric MBW

MBWACLR=MBWACLR,1/2+FC,2-FC,1+MBWACLR,2/2,

where MBWACLR,n is the single-channel ACLR MBW for the channel bandwidth of nth CC as defined in TS 38.101-1 Table 6.5.2.4.1-1, and FC,2-FC,1 is the channel spacing.

However, when properly aligned with the sub-block, the measurement center does not necessarily coincide with the sub-block center. Therefore, the offset from the center of transmitted sub-block to the ACLR MBW center must be specified separately above and below the sub-block.

After simplification, the offsets between the centers of the sub-block and ACLR MBW for the adjacent channels below and above the sub-block are

offsetACLR,low=BWChannel_CA-displacement

offsetACLR,high=BWChannel_CA+displacement,

respectively. Here, BWChannel_CA is the aggregated channel bandwidth, and

displacement=MBWACLR,2-MBWACLR,14-Foffset,high-Foffset,low2

is a displacement due to the asymmetry of the ACLR MBW.

ACLR alternative B: symmetric MBW

Another approach would be to define the ACLR MBW to be symmetric with respect to the sub-block center. This would allow to use the same frequency offset on both sides of the sub-block. However, since this MBW should be a superset of the asymmetric MBW of Alternative A, the symmetric MBW must be wider, resulting in somewhat lower measured ACLR values.

Symmetric ACLR MBW and the respective frequency offset:

MBWACLR=BWChannel_CA-2 minFoffset,low-MBWACLR,12,  Foffset,high-MBWACLR,22

offsetACLR=BWChannel_CA

Notice the min∙ function, in contrast to earlier WF [2] which applied a max∙ function. The min∙ function is preferred because max∙ could leave some RBs outside the MBW.

Due to the symmetry, we prefer Alternative B.

Proposal 2: Define the CA ACLR measurement bandwidth and frequency offset as

MBWACLR=BWChannel_CA-2 minFoffset,low-MBWACLR,12,  Foffset,high-MBWACLR,22

offsetACLR=BWChannel_CA

# 3	Conclusions

We proposed new formulas for Foffset,low and Foffset,high which determine the sub-block edges in carrier aggregation. The new formulas guarantee that the aggregated channel bandwidth never exceeds the sum of CC bandwidths.

Proposal 1: Specify Foffset,low and Foffset,high as follows:

Define the guard exchange term as

GB=GBChannel(2)-GBChannel(1)

where GBChannel(n) (n=1, 2) are the minimum guard band widths used in TS 38.101-1 subclause 5.4A.1.

If GB>0,

Foffset,low=BWChannel12+GB

Foffset,high=BWChannel22,

else

Foffset,low=BWChannel12

Foffset,high=BWChannel22+GB.

The nominal channel spacing and sub-block guards depend on the operating band. However, the specified MPR must be valid for all operating bands. We proposed a way to simulate or measure MPR values that are guaranteed to be sufficient in all current and future operating bands.

We also proposed how to define the measurement bandwidth and frequency offset for CA ACLR:

Proposal 2: Define the CA ACLR measurement bandwidth and frequency offset as

MBWACLR=BWChannel_CA-2 minFoffset,low-MBWACLR,12,  Foffset,high-MBWACLR,22

offsetACLR=BWChannel_CA

# 4	References

[1] R4-2001385, Problems with CA channel arrangement, Nokia, RAN4#94

[2] R4-2002804, WF on emission requirement for intra-band contiguous UL CA, RAN4#94e