3GPP TSG-RAN WG4 Meeting # 94-e-Bis                                                      R4-2003268

Electronic Meeting, 20 – 30 Apr., 2020

Source: 	CATT

Title: 	Discussion on test parameters for NR PUSCH UL Timing

Agenda item:	6.17.2.2.3

Document for:	Approval

- Introduction

In the recent RAN4#94-e meeting, A WF[1] on NR HST requirement was approved, but there are some open issues on PUSCH UL timing shown as following:

UL TA – Test metric: - Test metric - Option 1: SNR@70% of maximum throughput for the moving UE. - Option 2: Keep decision open. Study test metrics under time estimation error.

This paper will provide our view on these issues for NR PUSCH UL timing.

- Discussion

In order to investage the test metric for NR PUSCH UL timing, the SNR @ 70%  maximum throughput and SNR @ 95% maximum throughput for moviing UE are simulated in the following three cases, where the different Timing detection period and Timing adjustment period are configurated. The different synthetic UL TA implementation errors can be simulated by configuring different Timing detection period and Timing adjustment period.  The following three cases are shown as follows:

Case 1: Timing detection period 20ms, Timing adjustment period 20ms

Case 2: Timing deteciton period 80ms, Timing adjustment period 500ms

Case 3: Timing detection period 200ms, Timing adjustment period 1000ms

Based on the simulation parameters[1,2] shown in Table A-1 in Annex, the evaluation results for test metric  for  NR PUSCH UL timing are shown in Table 2-1. The evaluation results show that the variation of SNR @70% of maximum throughput of case 2 relative to case 1 and the variation of SNR @ 70% of maximum throughput of case 3 relative to case 1 are 0.03 and 0.05 dB respectively, while the variation of SNR @95% of maximum throughput of case 2 relative to case 1 and the variation of SNR @ 95% of maximum throughput of case 3 relative to case 1 are 0.07 and 0.14 dB respectively. The variation of SNR @ 95% of maximum throughput is slightly larger than that of SNR @ 70% of maximum throughput. Compared with 70% of maximum throughput test metric, the 95% of maximum throughput test metric can more clearly reflect the UL TA implementation errors.  But the gain of 95% of maximum throughput test metric relative to 70% of maximum throughput test metric is less than 1dB. So,we propose  Option 1: SNR@70% of maximum throughput for the moving UE.

Proposal 1: Choose Option 1: SNR@70% of maximum throughput for the moving UE.

Table 2-1 Evaluation results for NR PUSCH UL timing

| Scenario | CBW | SCS | PUSCH mapping type | variation of SNR @70% of maximum throughput of Case 2 relative to Case 1 | variation of SNR @70% of maximum throughput of Case 3 relative to Case 1 | variation of SNR @95% of maximum throughput of Case 2 relative to Case 1 | variation of SNR @95% of maximum throughput of Case 3 relative to Case 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario Y | 10MHz | 15KHz | Type A | 0.03 | 0.05 | 0.07 | 0.14 |
|  | 10MHz | 15KHz | Type B | 0.03 | 0.05 | 0.06 | 0.13 |
|  | 40MHz | 30KHz | Type A | 0.01 | 0.05 | 0.03 | 0.1 |
|  | 40MHz | 30KHz | Type B | 0.02 | 0.06 | 0.03 | 0.1 |

In the agreed WF [2] in RAN4#94-e, the PUSCH mapping type B is agreed to be defined for NR PUSCH UL timing requirement. For PUSCH mapping type B with DMRS 1+1+1, l0 is defined as 0.

Proposal 2: l0 = 0 for PUSCH mapping type B with DMRS 1+1+1.

- Conclusion

This paper provides our evaluation results for test metric for NR PUSCH UL timing and gives our proposals as follows:

Proposal 1: Choose Option 1: SNR@70% of maximum throughput for the moving UE.

Proposal 2: l0 = 0 for PUSCH mapping type B with DMRS 1+1+1.

- References

- R4-1915886，Way forward on NR HST PUSCH demodulation requirements，Huawei, HiSilicon，RAN4#93

- R4-2002405，WF on Rel-16 NR HST BS demodulation requirements，Nokia, Nokia Shanghai Bell, RAN4#94-e

- Annex

Table A-1 Test parameters for NR PUSCH UL timing

| Parameter | value |
| --- | --- |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and type B |
| MCS index | MCS 16 |
| PTRS | Not configured |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| RV | {0,2,3,1} |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 |
| Front loaded DMRS start symbol | Type A: l0 = 2 Type B: l0 = 0 |
| SCS and BW | 15kHz: 10MHz; 30kHz: 40MHz |
| RB assignment | 15kHz SCS/10MHz CBW: 25 continuously allocated RBs for each UE 30kHz SCS/40MHz CBW: 50 continuously allocated RBs for each UE |
| Starting PRB index | 15kHz SCS/10MHz CBW: Moving UE 0, Stationary UE 25 30kHz SCS/40MHz CBW: Moving UE 0, Stationary UE 50 |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |
| Propagation condition | Scenario Y: AWGN 15kHz SCS: A=10us, △ω=0.13s-1 30kHz SCS: A=5us, △ω=0.26s-1 |
| SRS transmission(optional) | Disabled |
| TA detection period and TA adjusted period | Case 1: Timing detection period 20ms, Timing adjustment period 20ms Case 2: Timing deteciton period 80ms, Timing adjustment period 500ms Case 3: Timing detection period 200ms, Timing adjustment period 1000ms |
| Testing metric | SNR @70% of maximum throughput |