3GPP TSG-RAN WG4 Meeting # 94-e-Bis                                                       R4-2003274

Electronic Meeting, 20 – 30 Apr., 2020

Source: 	CATT

Title: 	Simulation results for NR PUSCH with 30% throughput test point

Agenda item:	6.18.2.1

Document for:	Discussion

- Introduction

In recent RAN4#94-e meeting,  the FR2 DMRS configuration and FR2 PTRS configuration for NR PUSCH with 30% throughput test point were determined as shown in [1] and [2].

This paper will provide our evaluation results for NR PUSCH with 30% throughput test point based on agreed simulation assumptions in WF.

- Discussion

The option with and without PTRS for FR2 in WF[1] was agreed, but the PT-RS configuration paramters need to be defined.  The PT-RS configuration defined in Rel-15 shown as following can be reused for NR PUSCH with 30% throughput test point.

| PT-RS configuration | Frequency density (KPT-RS) | 2 |
| --- | --- | --- |
|  | Time density (LPT-RS) | 1 |

Proposal: Reuse R15 PT-RS configuration KPT-RS=2, LPT-RS=1 for NR PUSCH with 30% throughput test point.

Based on above proposal and agreed simulaton assumptions[1, 2, 3, 4], the test parameters are summarized in Table A-1 in Annex.

The evaluation results for NR PUSCH with 30% throughput test point based on proposed test parameters in Table A5-1 are shown in Table 2-1 and Table 2-2.

Table 2-1 Evaluation results for NR PUSCH with 30% throughput test point with CP-OFDM at 4GHz

| waveform | Time domain resource allocation type | SCS and BW | MCS level | Propagation condition | DMRS | SNR @30% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| CP-OFDM | type A | 15kHz, 5MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 0.53 | 3.03 |
| CP-OFDM | type A | 30kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 0.44 | 2.94 |
| CP-OFDM | type B | 15kHz, 5MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 0.49 | 2.99 |
| CP-OFDM | type B | 30kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 0.37 | 2.87 |

Table 2-2 Evaluation results for NR PUSCH with 30% throughput test point with CP-OFDM, Type B at 30GHz

| waveform | SCS and BW | MCS level | Propagation condition | DMRS | PTRS | SNR @30% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Ideal | Impairment |
| CP-OFDM | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | K_PTRS=2, L_PTRS=1 | 1.28 | 3.78 |
| CP-OFDM | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | No | 0.98 | 3.48 |
| CP-OFDM | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | K_PTRS=2, L_PTRS=1 | 1.46 | 3.96 |
| CP-OFDM | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | No | 1.16 | 3.66 |
| CP-OFDM | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1+1 | K_PTRS=2, L_PTRS=1 | 0.85 | 3.35 |
| CP-OFDM | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1+1 | No | 0.45 | 2.95 |
| CP-OFDM | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1+1 | K_PTRS=2, L_PTRS=1 | 0.91 | 3.41 |
| CP-OFDM | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1+1 | No | 0.41 | 2.91 |

Based on above evaluation results for 30% throughput test point, the CRs for TS 38.104, TS38.141-1 and TS38.141-2 are provided for approval.

- Conclusion

In this contribution, we discuss the test parameters for NR PUSCH with 30% throughput test point and give our proposals as follows:

Proposal: Reuse R15 PT-RS configuration KPT-RS=2, LPT-RS=1 for NR PUSCH with 30% throughput test point.

Based on above proposals and evaluation results for 30% throughput test point, the draft CRs for TS 38.104 [5], TS38.141-1 [6] and TS38.141-2 [7] are provided for approval.

- References

- R4-2002397, Way forward on PUSCH demodulation requirements for 30% throughput, NTT DOCOMO, INC.,RAN4#94-e

- RAN4_94e_BS RF Test Demod_Chairman_report_EOM, RAN4#94-e

- RAN4_92bis_RD_chairman_report_10_Fri_EOM, RAN4#92bis

- R4-1915804,Ad Hoc minutes for NR Rel-15 and Rel-16 BS demodulation, China Telecom, Huawei, HiSilicon,RAN4#93

- R4-3003275, Draft CR for TS38.104: Introduce PUSCH performance requirements at 30% throughput test point, CATT, RAN4#94bis-e

- R4-2003276, Draft CR for TS38.141-1: Introduce PUSCH performance requirements at 30% throughput test point, CATT, RAN4#94bis-e

- R4-2003277, Draft CR for TS38.141-2: Introduce PUSCH performance requirements at 30% throughput test point, CATT, RAN4#94bis-e

- Annex

Table A5-1 Test Parameters for CP-OFDM with 30% throughput test point

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1 | 1+0, 1+1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A and B | type B |
| Frequency domain resource | Full applicable test bandwidth | Full applicable test bandwidth |
| MCS index | 16 | 16 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | MCS16:TDLC300-100 | MCS16:TDLA30-300 |
| SCS and BW | 15kHz: 5MHz; 30kHz: 10MHz; | 60kHz: 50MHz; 120kHz: 50MHz; |
| PTRS | With and without PTRS PTRS configuration: KPT-RS=2, LPT-RS=1 | With and without PTRS PTRS configuration: KPT-RS=2, LPT-RS=1 |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @30% of maximum throughput | SNR @30% of maximum throughput |