3GPP TSG-RAN WG4 Meeting #94-e-bis                                                         R4-2003309

# Electronic Meeting, 20th Apr. – 30th Apr., 2020

Source: 	CATT

Title: 	Discussion on antenna configuration for NR HST PUSCH tunnel scenario

Agenda item:	6.17.2.2.1

Document for:	Discussion

- Introduction

In RAN4#94e meeting, the antenna configuration for NR HST PUSCH tunnel scenario was discussed based on [1]. However, whether or not to introduce 1T1R requirements for the tunnel scenario has not reached consensus and needs to be further discussed. This contribution will provide our views and analysis on the antenna configuration for NR HST PUSCH tunnel scenario.

- Discussion

As per the WF [1] on NR HST BS demodulation requirements, the tentative progress on the antenna configuration for NR HST PUSCH tunnel scenario is shown as follows:

- Introduce 1T1R requirements for the tunnel scenario - Option 1: Introduce 1T1R requirements for the tunnel scenario. - Option 2: Do not introduce 1T1R requirements for the tunnel scenario. - Option 3: Introduce 1T1R requirements for the tunnel scenario, and limit tests to not cover OTA. - If 1T1R requirement is introduced: 1T1R requirement configuration - Option 1: Re-use the 1T2R requirement configuration. - FFS for next meeting. - If 1T1R requirement is introduced with OTA testing: 1T1R requirement configuration - Option 1: Same test setup for 1T1R as typically specified in TS 38.141-2, with a test procedure that includes polarization alignment. - FFS for next meeting.

In TS 36.104, 1T1R antenna configuration is required for LTE HST PUSCH tunnel scenario. In practical tunnel scenario, 1T1R is the common antenna configuration considering trade-off between expensive leakage cable and MIMO performance. Based on the discussion in the previous meetings, 1T1R antenna configuration has the deployment scenario in practical tunnel scenario proposed by operator. If only 2Rx requirement will be defined, 1Rx performance cannot be guaranteed. Therefore, 1T1R antenna configuration is proposed to be introduced for the tunnel scenario. If IT1R requirement is introduced, it is reasonable to reuse the 1T2R requirement configuration. However, there is no need to introduce OTA test for 1T1R antenna configuration.

Proposal 1: Introduce 1T1R requirements for the tunnel scenario.

Proposal 2: Re-use the 1T2R requirement configuration for 1T1R.

Proposal 3: No need to introduce OTA test for 1T1R.

- Conclusion

In this contribution, the antenna configuration for NR HST PUSCH tunnel scenario is primarily analysed. The following proposals are derived as follows:

Proposal 1: Introduce 1T1R requirements for the tunnel scenario.

Proposal 2: Re-use the 1T2R requirement configuration for 1T1R.

Proposal 3: No need to introduce OTA test for 1T1R.

- References

- R4-2002405, WF on Rel-16 NR HST BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#94e