3GPP TSG-RAN4  #94bis-e															     	R4-2003438

20th Apr – 30th Apr 2020

Source: 	Qualcomm Incorporated

Title: 	A-MPR for 10MHz V2X PSSCH PSCCH transmission TP

Agenda item:	6.4.4.1

Document for:	Approval

# Introduction

We study the required A-MPR and propose a specification structure and values.	The study is conducted against requirements signaled under NS_33 and NS_NEW.

# Simulation description and findings

All valid waveforms were simulated at  fc = 5860, 5870, …, 5920 MHz for 15, 30, and 60 kHz SCS; and all subband lengths (10,15,20,25, 50,75 and 100) . Plots of some of the simulations are shown.

From the data we conclude that A-MPR under NS_33 is higher at fc=5860 MHz due to the coexistence emission requirement under NS_33. Further we find under the new NS, NS_NEW, fc=5860 requires the same A-MPR than the other fcs.

The specification formulations in the text proposal are derived from simulations at each frequency.

Fc = 5860 MHz for Gpostconnector = 0,1, … 7

Fc = 5870 MHz for Gpostconnector = 0,1, … 7

Variation across modulation

<begin change 38.886 draft v0.6.0 >

### 8.1.3	UE maximum output power with additional requirements

To comply with the EN 302 571 standard  emission mask, RAN4 needs to derive A-MPR requirements considering with the additional A-SEM and A-SE requirements in the EN 302 571 standard as shown in Table 8.1.3-1 and Table 8.1.3-2.

The NR V2X UE shall satisfy the additional SEM and additional SE requirements when NS_33 or NS_NEW are configured from pre-configured radio parameters, or the cell has messaged the NS.

Table 8.1.3-1: Additional SEM requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to a conducted requirement depending on the post antenna connector gain,  Gpost connector , declared by the UE following the principle described in Annex G in TS38.101-1.

Table 8.1.3-2: Additional SE requirements

| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43 |
| NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. |  |  |  |  |  |  |  |

Also, FCC regulatory requirements for 40MHz in the ITS spectrum are shown in Table 8.1.3-3

Table 8.1.3-3: Additional SEM requirements for 40MHz channel bandwidth (fc =5885 MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0 - 2 | -32 | 100kHz |
| 2-10 | -36 | 100kHz |
| 10-20 | -38 | 100kHz |
| 20-40 | -43 | 100kHz |
| 40 - 100 | -50 | 100kHz |

To comply with the regional regulatory requirements, RAN4 specifies the A-MPR requirements according to different network signalling by pre-configured radio parameters, or when the cell messages the NS to the NR V2X UE.

The allowed additional Maximum Power Reduction (A-MPR) for the maximum output power due to higher order modulation and transmit bandwidth configuration (resource blocks) will be specified in TS38.101-1 as below.

#Editorial note: Table to be updated reflecting the table numbers once pCRs for this clause are agreed and are collated

Table 8.1.3-4: Additional Maximum Power Reduction (A-MPR) for NR V2X

| Network Signalling value | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_33 | Table 8.1.3-1 (A-SEM) Table 8.1.3-2 (A-SE) | n47 | 10 | Table 8.1.3-5 Table 8.1.3-6 Table 8.1.3-7 |  |
| NS_48 | Table 8.1.3-3 (A-SEM) | n47 | 40 | Table 8.1.3-8 |  |

Table 8.1.3-5: A-MPR for NS_33 (15kHz SCS)

| Carrier frequency(MHz) | Resources Blocks (NRB) Note1 | Start Resource Block | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | [≤15] | [0] | [20] |  |  |
|  | [15> and ≤25] | [0] | [18.5] |  |  |
|  | [25 > and ≤40] | [0] | [17] |  |  |
|  | [> 40] | [0] | [15.5] |  |  |
|  | [≤ 40] | [10] | [12] |  |  |
|  | [≤ 30] | [15] | [9.5] |  |  |
|  | [≤ 30] | [20] | [7.5] |  |  |
|  | [≤ 25] | [25] | [6] |  |  |
|  | [≤ 20] | [> 30] | [5] |  |  |
| 5870, 5910, 5920, 5880, 5890, 5900 | [≤ 50] | [0] | [3] |  | [4] |
|  | [≤ 40] | [10] | [2.5] | [2.5] | [4] |
|  | [≤ 20] | [15, 20, 40] | [1] | [2] | [4] |
|  | [20 > ] | [20] | [2.5] | [2.5] | [4] |
|  | [≤ 25] | [25, 30] | [2.5] | [2.5] | [4] |
| Note 1: The resource blocks are limited by LCRB. LCRB = {10,15,20,25,30,40,45,50}. |  |  |  |  |  |

| Carrier frequency(MHz) | A-MPRStep | LCRB (Note1) | RBStart (Note2) | A-MPRBase (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 58603 | 1.1 | ≤15 | 0 | 23 |  |  |
|  | 1.1 | > 15 and ≤25 | 0 | 20 |  |  |
|  | 0.875 | > 25 and ≤40 | 0 | 18 |  |  |
|  | 0.875 | > 40 | 0 | 16 |  |  |
|  | 0.25 | ≤ 40 | 10 | 9 |  |  |
|  | 0.25 | ≤ 30 | 15 | 8 |  |  |
|  | 0.25 | ≤ 30 | 20 | 7 |  |  |
|  | 0.25 | ≤ 25 | 25 | 6.5 |  |  |
|  | 0.25 | ≤ 20 | ≥ 30 | 6.5 |  |  |
| 58604,5870, 5910, 5920, 5880, 5890, 5900 | 0.25 | ≤ 50 | 0 | 5 |  | 6.5 |
|  | 0.25 | ≤ 40 | 10 | 4 | 4 | 6.5 |
|  | 0.25 | ≤ 20 | 15, 20, 40 | 4 | 4 | 6.5 |
|  | 0.25 | > 20 | 20 | 4 | 4 | 6.5 |
|  | 0.25 | ≤ 25 | 25, 30 | 4 | 4 | 6.5 |
| NOTE 1: LCRB is a value from the set {10,15,20,25,30,40,45,50}. NOTE 2: RBStart is constrainted to integer multiples of LCRB, including 0. NOTE 3: Applicable for NS_33 NOTE 4: Applicable for NS_NEW |  |  |  |  |  |  |

Table 8.1.3-6: A-MPR for NS_33 (30kHz SCS)

| Carrier frequency(MHz) | Resources Blocks (NRB) Note1 | Start Resource Block | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | [10,15] | [0] | [19] |  |  |
|  | [20] | [0] | [17] |  |  |
|  | [10] | [10] | [9] |  |  |
| 5870, 5910, 5920, 5880, 5890, 5900 | [≤ 20] | [0] | [2.5] |  | [4] |
|  | [10] | [10] | [1.5] | [2] | [4] |
| Note 1: The resource blocks are limited by LCRB. LCRB = {10,15,20}. |  |  |  |  |  |

| Carrier frequency(MHz) | A-MPRStep | LCRB (Note1) | RBStart (Note2) | A-MPRBase (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 58603 | 1 | 10,15 | 0 | 22 |  |  |
|  | 1 | 20 | 0 | 19 |  |  |
|  | 0.14 | 10 | 10 | 6 |  |  |
| 58604,5870, 5910, 5920, 5880, 5890, 5900 | 0.14 | ≤ 20 | 0 | 5 |  | 6.5 |
|  | 0.28 | 10 | 10 | 3 | 3.5 | 6.5 |
| NOTE 1: LCRB is a value from the set {10,15,20}. NOTE 2: RBStart is constrainted to integer multiples of LCRB, including 0. NOTE 3: Applicable for NS_33 NOTE 4: Applicable for NS_NEW |  |  |  |  |  |  |

Table 8.1.3-7: A-MPR for NS_33 (60kHz SCS)

| Carrier frequency(MHz) | Resources Blocks (NRB) | Start Resource Block | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | [10] | [0] | [18] |  |  |
| 5870, 5910, 5920 |  |  | [5] |  |  |
| 5880, 5890, 5900 |  |  | [2.5] |  | [4.5] |

| Carrier frequency(MHz) | A-MPRStep | LCRB (Note1) | RBStart (Note2) | A-MPRBase (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 58603 | 1 | 10 | 0 | 22 |  |  |
| 58604, 5870, 5880, 5890, 5900, 5910, 5920 | 0.28 | 10 | 0 | 4 |  | 6.5 |
| NOTE 1: LCRB = 10 is the only allowed value. NOTE 2: RBStart = 0 is the only allowed value. NOTE 3: Applicable for NS_33 NOTE 4: Applicable for NS_NEW |  |  |  |  |  |  |

Table 8.1.3-8: A-MPR for NS_48

| Carrier frequency(MHz) | Modulation | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocation |
| 5885 | QPSK | ≤ (9.5 + ΔNote1) | [≤ 8.0] | [≤ 3.5] |
|  | 16QAM |  | [≤ 8.0] | [≤ 3.5] |
|  | 64QAM |  | [≤ 8.5] | [≤ 4.5] |
|  | 256QAM |  | [≤ 8.5] | [≤ 6.0] |
| Note1: Δ is 0, 3, and 5 for 60kHz, 30kHz, and 15kHz SCS, respectively. |  |  |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [3].

RBStart,Low = max(1, floor(NRB /3.5))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High , and

LCRB ≤ ceil(NRB/3.5)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ floor(NRB*0.2) RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

<end change>