3GPP TSG-RAN4  #94bis-e															     	R4-2003439

20th Apr – 30th Apr 2020

Source: 	Qualcomm Incorporated

Title: 	A-MPR for 10MHz V2X PSFCH transmission TP

Agenda item:	6.4.4.1

Document for:	Approval

# Introduction

A UE may simultaneously transmit multiple PSFCH signals. We study the required A-MPR for multiple transmissions and propose a specification structure and values. A parameter call Ngap from [TBD] is used. Ngap is based on the idea that wider spaced transmissions create wider IMs, and emissions specs are tougher farther out. Ngap should have some correlation to MPR and A-MPR.

# Simulation Description

Simulations were done for PSFCH transmissions ranging from 1 to 5, for 10MHz channel bandwidth and SCS values 15 kHz and 30 kHz. The transmissions are 1RB allocated in a non-contiguous fashion. The total transmit power was limited to 23dBm and transmissions were assigned equal power.

All possible waveforms were simulated for channel bandwidth/SCS combinations with a smaller number of transmissions. We simulated 10,000+ waveforms for bandwidth/SCS combination with a larger number of transmissions. Overall 500,000+ waveforms were studied.

## Simulation result details

### A-MPR

We performed simulations for many waveforms, between 500,000 and 1,000,000. Required backoff was found to be correlated with the span of the outermost active RBs in most cases. Subcarrier spacings of 15 and 30 kHz behaved similarly.

For non-contiguous allocation for simultaneous PSFCH transmission for NR V2X the additional maximum power reduction when NS_33 is signaled will be specified as follows

For 10 MHz channel bandwidth and fc = 5860 MHz; A-MPR_PSFCH = 22

For 10 MHz channel bandwidth and fc = 5870, 5880, 5890, 5900, 5910, or 5920 MHz, and 2 <= transmission <=3 ; A-MPR_PSFCH = CEIL {6 + 4 * NGap/ NRB, 0.5}

For 10 MHz channel bandwidth and fc = 5870, 5880, 5890, 5900, 5910, or 5920 MHz, and transmission >=4 ; A-MPR_PSFCH = CEIL {5 + 3 * NGap/ NRB, 0.5}

For non-contiguous allocation for simultaneous PSFCH transmission when NS_NEW is signaled for NR V2X will be specified as follows

For 10 MHz channel bandwidth, fc = 5860, 5870, 5880, 5890, 5900, 5910, or 5920 MHz, and 2 <= transmission <=3 ; A-MPR_PSFCH = CEIL {6 + 4 * NGap/ NRB, 0.5}

For 10 MHz channel bandwidth and fc = 5860, 5870, 5880, 5890, 5900, 5910, or 5920 MHz, and transmission >=4 ; A-MPR_PSFCH = CEIL {5 + 3 * NGap/ NRB, 0.5}

Where NGap is the difference, in units of RBs, between the outermost active RBs and CEIL{N, 0.5} means rounding N upwards to closest 0.5dB.

### The effect of Gpostconnector

The coexistence emission limit is specified in EIRP, and Gpostconnector is as a factor in simulation to ensure radiated compliance.

For fc = 5860  with ETSI including the coexistence requirement (NS_33), the coexistence requirements dominate. We find no correlation between Ngap and Gpostconnector so Ngap is not used in formulating A-MPR.

fc = 5860 MHz 15kHz and 30 kHz SCS

fc = 5870 15kHz SCS and single transmission

For fc = 5870, 5880, 5890, 5900, 5910, and 5920 MHz there is a positive correlation between Gpostconnector and A-MPR. Gpostconnector is used to scale the A-MPR from its 0 dB Gpostconnector A-MPR. The slope we have chosen is a transmission and SCS dependent value.

For fc = 5860 with NS_NEW, the same formulation applies.

MA = A-MPRBase + Gpostconnector * A-MPRStep

For 5860 MHz under NS_33

A-MPRStep = 0 and transmissions > 0

For 5870, 5880, 5890, 5900, 5910, 5920 MHz under NS_33 or NS_NEW

A-MPRStep = 0.5 for 15kHz SCS and transmissions = 2 or 3,

A-MPRStep = 0.375 for 15kHz SCS and transmissions >=4 and for 30 kHz SCS with transmissions = 2 or 3

A-MPRStep = 0.2 for 30kHz SCS and transmissions >=4

For 5860 MHz under NS_NEW

A-MPRStep is the same as 5870 MHz under NS_NEW

For single transmission at fc = 5870, 5880, 5890, 5900, 5910, and 5920; and under NS_33 or NS_NEW

A-MPRStep = 0.8

For single transmission at fc 5860 MHz under NS_NEW,

A-MPRStep = 0.8

# Text Proposal

<begin changes to TR 38.886 draft v0.6.0>

### 8.1.3	UE maximum output power with additional requirements

To comply with the EN 302 571 standard  emission mask, RAN4 needs to derive A-MPR requirements considering with the additional A-SEM and A-SE requirements in the EN 302 571 standard as shown in Table 8.1.3-1 and Table 8.1.3-2.

The NR V2X UE shall satisfy the additional SEM and additional SE requirements when NS_33 or NS_NEW are configured from pre-configured radio parameters, or the cell has messaged the NS.

Table 8.1.3-1: Additional SEM requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to a conducted requirement depending on the post antenna connector gain,  Gpost connector , declared by the UE following the principle described in Annex G in TS38.101-1.

Table 8.1.3-2: Additional SE requirements

| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43 |
| NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. |  |  |  |  |  |  |  |

Also, FCC regulatory requirements for 40MHz in the ITS spectrum are shown in Table 8.1.3-3

Table 8.1.3-3: Additional SEM requirements for 40MHz channel bandwidth (fc =5885 MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0 - 2 | -32 | 100kHz |
| 2-10 | -36 | 100kHz |
| 10-20 | -38 | 100kHz |
| 20-40 | -43 | 100kHz |
| 40 - 100 | -50 | 100kHz |

To comply with the regional regulatory requirements, RAN4 specifies the A-MPR requirements according to different network signalling by pre-configured radio parameters, or when the cell messages the NS to the NR V2X UE.

The allowed additional Maximum Power Reduction (A-MPR) for the maximum output power due to higher order modulation and transmit bandwidth configuration (resource blocks) will be specified in TS38.101-1 as below.

#Editorial note: Table to be updated reflecting the table numbers once pCRs for this clause are agreed and are collated

Table 8.1.3-4: Additional Maximum Power Reduction (A-MPR) for NR V2X

| Network Signalling value | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_33 | Table 8.1.3-1 (A-SEM) Table 8.1.3-2 (A-SE) | n47 | 10 | Table 8.1.3-5 Table 8.1.3-6 Table 8.1.3-7 |  |
| NS_48 | Table 8.1.3-3 (A-SEM) | n47 | 40 | Table 8.1.3-8 |  |

Table 8.1.3-5: A-MPR for NS_33 (15kHz SCS)

| Carrier frequency(MHz) | Resources Blocks (NRB) Note1 | Start Resource Block | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | [≤15] | [0] | [20] |  |  |
|  | [15> and ≤25] | [0] | [18.5] |  |  |
|  | [25 > and ≤40] | [0] | [17] |  |  |
|  | [> 40] | [0] | [15.5] |  |  |
|  | [≤ 40] | [10] | [12] |  |  |
|  | [≤ 30] | [15] | [9.5] |  |  |
|  | [≤ 30] | [20] | [7.5] |  |  |
|  | [≤ 25] | [25] | [6] |  |  |
|  | [≤ 20] | [> 30] | [5] |  |  |
| 5870, 5910, 5920, 5880, 5890, 5900 | [≤ 50] | [0] | [3] |  | [4] |
|  | [≤ 40] | [10] | [2.5] | [2.5] | [4] |
|  | [≤ 20] | [15, 20, 40] | [1] | [2] | [4] |
|  | [20 > ] | [20] | [2.5] | [2.5] | [4] |
|  | [≤ 25] | [25, 30] | [2.5] | [2.5] | [4] |
| Note 1: The resource blocks are limited by LCRB. LCRB = {10,15,20,25,30,40,45,50}. |  |  |  |  |  |

Table 8.1.3-6: A-MPR for NS_33 (30kHz SCS)

| Carrier frequency(MHz) | Resources Blocks (NRB) Note1 | Start Resource Block | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | [10,15] | [0] | [19] |  |  |
|  | [20] | [0] | [17] |  |  |
|  | [10] | [10] | [9] |  |  |
| 5870, 5910, 5920, 5880, 5890, 5900 | [≤ 20] | [0] | [2.5] |  | [4] |
|  | [10] | [10] | [1.5] | [2] | [4] |
| Note 1: The resource blocks are limited by LCRB. LCRB = {10,15,20}. |  |  |  |  |  |

Table 8.1.3-7: A-MPR for NS_33 (60kHz SCS)

| Carrier frequency(MHz) | Resources Blocks (NRB) | Start Resource Block | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | [10] | [0] | [18] |  |  |
| 5870, 5910, 5920 |  |  | [5] |  |  |
| 5880, 5890, 5900 |  |  | [2.5] |  | [4.5] |

Table 8.1.3-8: A-MPR for NS_48

| Carrier frequency(MHz) | Modulation | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocation |
| 5885 | QPSK | ≤ (9.5 + ΔNote1) | [≤ 8.0] | [≤ 3.5] |
|  | 16QAM |  | [≤ 8.0] | [≤ 3.5] |
|  | 64QAM |  | [≤ 8.5] | [≤ 4.5] |
|  | 256QAM |  | [≤ 8.5] | [≤ 6.0] |
| Note1: Δ is 0, 3, and 5 for 60kHz, 30kHz, and 15kHz SCS, respectively. |  |  |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [3].

RBStart,Low = max(1, floor(NRB /3.5))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High , and

LCRB ≤ ceil(NRB/3.5)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ floor(NRB*0.2) RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

For PSFCH transmission, the A-MPR shall be specified as:

A-MPRBase = CEIL { MA_PSFCH_MIN + (MA_PSFCH_MAX  - MA_PSFCH_MIN) * NGap/ NRB, 0.5}

Where NGap is the difference, in units of RBs, between the outermost active RBs and CEIL{N, 0.5} means rounding N upwards to closest 0.5dB

and MA_PSFCH_MAX , MA_PSFCH_MIN  , and A-MPRStep  are specified in Tables TBD1 and TBD2

Table TBD1: PSFCH A-MPR for NS_33 or NS_NEW and 15kHz SCS

| Carrier frequency(MHz) | MA_PSFCH_MIN | MA_PSFCH_MAX | A-MPRStep | Number of transmissions |
| --- | --- | --- | --- | --- |
| 58601 | 19 | 19 | 0 | N =1 |
| 58601 | 18 | 18 | 0 | N =2 |
| 58601 | 22 | 22 | 0 | N =3 |
| 58601 | 21 | 21 | 0 | N =4 |
| 58601 | 19 | 19 | 0 | N >= 5 |
| 58602, 5870, 5880, 5890, 5900, 5910, 5920 | 5 | 5 | 0.8 | N =1 |
| 58602, 5870, 5880, 5890, 5900, 5910, 5920 | 6 | 10 | 0.5 | 2 ≤ N ≤ 3 |
| 58602, 5870, 5880, 5890, 5900, 5910, 5920 | 5 | 8 | 0.375 | N >= 4 |
| NOTE 1: For NS_33 only NOTE 2: For NS_NEW only |  |  |  |  |

Table TBD2: PSFCH A-MPR for NS_33 or NS_NEW and 30kHz SCS

| Carrier frequency(MHz) | MA_PSFCH_MIN | MA_PSFCH_MAX | A-MPRStep | Number of transmissions |
| --- | --- | --- | --- | --- |
| 58601 | 19 | 19 | 0 | N =1 |
| 58601 | 17 | 17 | 0 | N =2 |
| 58601 | 22 | 22 | 0 | N =3 |
| 58601 | 20 | 20 | 0 | N =4 |
| 58601 | 19 | 19 | 0 | N >= 5 |
| 58602, 5870, 5880, 5890, 5900, 5910, 5920 | 5 | 5 | 0.8 | N =1 |
| 58602, 5870, 5880, 5890, 5900, 5910, 5920 | 4 | 8 | 0.375 | 2 ≤ N ≤ 3 |
| 58602, 5870, 5880, 5890, 5900, 5910, 5920 | 4.5 | 6 | 0.2 | N >= 4 |
| NOTE 1: For NS_33 only NOTE 2: For NS_NEW only |  |  |  |  |

<end change>