3GPP TSG-RAN WG4 Meeting #94-e-Bis 	R4-2003447

Electronic Meeting, 20 – 30 April 2020

Source:	Ericsson

Title:	Discussion on PUSCH demodulation requirements at 30% throughput test point

Agenda item:	6.18.2.1

Document for:	Discussion

# Introduction

From NR performance requirements enhancement discussion in RAN4#92bis, the study on tests coverage of PUSCH demodulation requirements at 30% throughput test point was introduced. Companies also discussed the detailed parameter assumptions in the last meeting. In this paper, we discuss this topic further and deliver simulation both ideal and implemental results for limited test cases.

# Discussion

## 2.1 Background

In RAN4#92bis and RAN4#93 meeting, most of parameter assumptions are agreed and there are only DM-RS and PT-RS configuration issue need further discussions. The following is the agreements according to the chairman’s notes of the last meeting.

- Antenna Configuration

1T2R for both FR1 and FR2.

- Bandwidth and SCS

FR1: 5MHz bandwidth (15kHz SCS); 10MHz bandwidth (30kHz SCS)

FR2: 50MHz bandwidth (60kHz and 120kHz SCS)

- MCS and Number of allocated PRBs

Use MCS16 instead of MCS2 for full PRB allocation.

- Waveform

Only CP-OFDM for MCS16.

- PUSCH mapping type

FR1: both Type A and Type B (only one is tested)

FR2: Type B

- DM-RS configuration

FR1: DM-RS 1+1

FR2: there are 3 options:

Option 1: 1+1 and 1+0

Option 2: 1+1

Option 3: 1+0

- Channel model

FR1: TDLC300_100 for MCS16

FR2: TDLA30_300 for MCS16

- PT-RS configuration

FR1: With PT-RS

FR2: there are 3 options:

Option 1: With and without PT-RS

Option 2: With PR-RS

Option 3: Without PT-RS

## 2.2 simulation result

Here we deliver our simulation results based on assumptions above including all options of DM-RS and PT-RS configurations.

Table 2.2-1 Summary of simulation results

| BW (MHz) | SCS (kHz) | Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | PUSCH mapping type | Additional DM-RS position | PT-RS | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | ideal | implementation |
| 5 | 15 | 1 | 2 | Normal | TDLC300_100 Low | 30 % | G-FR1-A4-8 | A | pos1 | No | 0.6 | 2.93 |
|  |  |  |  |  |  |  |  | B |  |  | 0.6 | 2.64 |
| 10 | 30 | 1 | 2 | Normal | TDLC300_100 Low | 30 % | G-FR1-A4-11 | A | pos1 | No | 0.5 | 2.51 |
|  |  |  |  |  |  |  |  | B |  |  | 0.5 | 2.91 |
| 50 | 60 | 1 | 2 | Normal | TDLA30-300 Low | 30 % | G-FR2-A4-1 | B | pos0 | Yes | 0.9 | 4.18 |
|  |  |  |  |  |  |  |  |  |  | No | 0.65 | 3.93 |
|  |  |  |  |  |  |  | G-FR2-A4-11 |  | pos1 | Yes | 0.5 | 3.69 |
|  |  |  |  |  |  |  |  |  |  | No | 0.25 | 3.03 |
|  | 120 | 1 | 2 | Normal | TDLA30-300 Low | 30 % | G-FR2-A4-3 | B | pos0 | Yes | 1.05 | 3.5 |
|  |  |  |  |  |  |  |  |  |  | No | 0.8 | 3.52 |
|  |  |  |  |  |  |  | G-FR2-A4-13 |  | pos1 | Yes | 0.55 | 3.37 |
|  |  |  |  |  |  |  |  |  |  | No | 0.3 | 2.99 |

- Observation

- For FR2, DM-RS 1+0 brings no benefits compared to DM-RS 1+1 since more DM-RS gives better channel estimation performance. Considering DM-RS 1+1 is supported by almost all BS, this test should take DM-RS 1+1 as preferred.

- The with PT-RS cases have a little worse performance than without ones because PT-RS will consume more REs, and this phenomenon is also seen in 70% throughput requirements.

It would be better to have both PT-RS requirements for different BS implementation.

Proposal 1: Taking only DM-RS 1+1 configuration for FR2 30% throughput test cases.

Proposal 2: Taking both with and without PT-RS requirements for FR2 30% throughput test cases.

# Conclusion

For remained issue on DM-RS and PT-RS configurations for FR2 cases, we propose

Proposal 1: Taking only DM-RS 1+1 configuration for FR2 30% throughput test cases.

Proposal 2: Taking both with and without PT-RS requirements for FR2 30% throughput test cases.

# References

[1]	R4-1914543, “On parameters for 30% throughput requirement”, Ericsson