3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2003448

Electronic Meeting, 20 – 30 April 2020

Source:	Ericsson

Title:	NR PUSCH UL timing adjustment simulation results for 95% of maximum throughput test metric

Agenda item:	6.17.2.2.3

Document for:	Discussion

# Introduction

In this contribution, we presented our simulation results for NR HST PUSCH UL timing adjustment at 95% of maximum throughput test metric.

# Simulation assumptions and simulation results

In this section, our presented our simulation assumptions based on [1] in Table 1 for UL timing adjustment. The feasibility of test metric at 70% of maximum throughput is shown in Table 2. The ideal simulation results and results with impairments are given in Table 3 and Table 4 respectively.

| Parameters | Values |
| --- | --- |
|  | FR1 – Scenario Y |
| Channel model | Stationary UE: AWGN Moving UE: AWGN |
| Number of Tx and Rx | 1Tx 2Rx |
| Number of DMRS symbols | (1+1+1) DM-RS symbols with l0 = 2 for PUSCH mapping type A, and l0 = 0 for PUSCH mapping type B |
| (A, Δω) | SCS = 15 kHz: (10 µs, 0.13 s-1) SCS = 30 kHz: (5 µs, 0.26 s-1) |
| System bandwidth | SCS = 15 kHz: 10MHz SCS = 30 kHz: 40 MHz |
| Frequency domain resource | SCS = 15 kHz: 25 PRBs for each UE SCS = 30 kHz: 50 PRBs for each UE |
| MCS index | 16 |
| SRS (optional) | SCS = 15 kHz: single region with length of 40 PRBs SCS = 30 kHz: single region with length of 80 PRBs |
| Testing metric | SNR @95% of maximum throughput |

Table 1. Simulation assumptions for UL timing adjustment at 350 km/h, scenario Y [1].

Table 2. Performance degradation due to TO without UL TA.

| Scenario | Performance degradation [dB] |
| --- | --- |
| Scenario Y (SCS = 30 kHz and BW = 40 MHz with PUSCH mapping type A) | @70% of maximum throughput = ~ 1 |
|  | @95% of maximum throughput = ~ 4 |

In the simulation results, it is observed that the performance degradation at 70% of maximum throughput for scenario Y is approximately 1dB. When taking into account the deviations in results from all the companies, the requirements at 70% of maximum throughput could become obscured and may not be able to achieve the intention of testing UL TA performance at 350 km/h.

Observation 1: Performance requirements at 70% of maximum throughput may not be able to achieve the intention of testing UL TA performance for scenario Y.

Table 3. Ideal simulation results for NR UL timing adjustment at MCS = 16.

| Scenario | SCS / BW | PUSCH mapping type | SNR [dB] @ 95% of maximum throughput |
| --- | --- | --- | --- |
| Scenario Y | 15 kHz / 10 MHz | A | 6.2 |
|  |  | B | 6.2 |
|  | 30 kHz / 40 MHz | A | 6.2 |
|  |  | B | 6.2 |

Table 4. Results with impairments for NR UL timing adjustment at MCS = 16.

| Scenario | SCS / BW | PUSCH mapping type | SNR [dB] @ 95% of maximum throughput |
| --- | --- | --- | --- |
| Scenario Y | 15 kHz / 10 MHz | A | 8.5 |
|  |  | B | 8.2 |
|  | 30 kHz / 40 MHz | A | 8.3 |
|  |  | B | 8.4 |

Observation 2: Satisfactory performance could be achieved at the 95% of maximum throughput with the agreed configuration. From Ericsson’s perspective, we are okay to set the performance requirements of the current UL test case (i.e. scenario Y) based on 95% of maximum throughput. However, the 95% of maximum throughput test metric should be evaluated and decided on a case-by-case basis.

# Conclusion

In this contribution, we summarized our 95% of maximum throughput simulation results for scenario Y NR UL timing adjustment. Below, we also summarized our observation:

Observation 1: Performance requirements at 70% of maximum throughput may not be able to achieve the intention of testing UL TA performance for scenario Y.

Observation 2: Satisfactory performance could be achieved at the 95% of maximum throughput with the agreed configuration. From Ericsson’s perspective, we are okay to set the performance requirements of the current UL test case (i.e. scenario Y) based on 95% of maximum throughput. However, the 95% of maximum throughput test metric should be evaluated and decided on a case-by-case basis.

# References

R4-2002405, WF on Rel-16 NR HST BS demodulation requirements, Nokia, Nokia Shanghai Bell,…