3GPP TSG-RAN WG4 Meeting #94-e-Bis                       	 R4-2003505

Electronic Meeting, 20 – 30 Apr., 2020

Agenda Item:	6.15.1.8

Source:	MediaTek Inc.

Title:	Discussion on active spatial relation switch

Document for:	Discussion

- Introduction

In last RAN4 e-meeting, some agreements are captured in the chairman’s notes (RAN4_94e_RRM_chairman_report_05_EOM_v2) and a WF is recommended to be used as a starting point for further discussion in the next meeting. The detail open issues are shown as follow.

- Whether to define delay requirement for spatial relation info switching associated with UL SRS - Whether to define delay requirement for RRC based spatial relation info switching with DL-RS for PUCCH? - Delay requirement for MAC CE based spatial relation info switching associated with DL-RS for PUCCH - For known TCI state - Option 1: THARQ +3ms - Option 2: THARQ +3ms + time for time tracking if applicable - Option 3: UE to switch beams at THARQ + 3ms (RAN1 specification). Performance guaranteed (in terms of timing etc) after reception 1 SSB(similar to TCI state switching) - For unknown TCI state - Option 1: THARQ + 3ms + TL1-RSRP - Option 2: THARQ + 3ms + TL1-RSRP + time for time tracking if applicable - Option 3: No requirement - Delay requirement for RRC based spatial relation info switching associated with DL-RS for P-SRS - For known TCI state - Option 1: Define delay based on RRC based TCI state switching requirements - Option 2: TRRCprocessing (timing is not required) - Option 3: No requirements - For unknown TCI state - Option 1: Define delay based on RRC based TCI state switching requirements - Option 1a: TRRCprocessing + TL1-RSRP - Option 2: No requirements - Whether to define delay requirement for MAC CE based spatial relation info switching with DL-RS for SP-SRS? - When the UL signal has spatial relation to an unknown TCI-state, - Option 1: UE transmits with previous TX beam/ arbitrary beam - Option 1a: UE transmits using previous TX beam - Option 2: Up to UE implementation - Option 3: UE behaviour shall not be specified for the period until TCI state becomes known.

In this paper, we will discuss the detail requirement for active spatial relation switch.

- Framework on Active Spatial Relation Switch

The UE’s uplink channel and RS can be QCLed with a SRS or downlink RS based on the spatial relation configuration.  The first open issue is whether to define the requirement for spatial relation info switching associated with UL SRS.

  - QCLed with a UL SRS

In uplink spatial relation configuration, a different scenario is that the uplink channel or RS can be configured to be QCLed with an uplink SRS. After some QCLed links, the spatial relation shall be QCLed with SRS with its usage configured as ‘beamManagement’. Some companies suggest to deprioritize this scenario in RAN4 because of the work loading. We’re fine to only focus on the requirement QCLed with DL RS firstly in RAN4 R16.

Proposal 1: Not to define the requirement when active spatial relation is configured to associated to a SRS in R16.

  - QCLed with a DL RS

Generally, if the uplink channel or RS is configured to QCL with a downlink RS, the active spatial relation switch procedure shall be similar as the procedure of active TCI state switch as follow. The similar known and unknown condition definition from TCI state switch shall be re-used. At first, the network can configure some spatial relation lists and some measurement objects.

Figure 1. Active Spatial Relation Switch Procedure for QCLed with DL RS

Known spatial relation condition

Currently, most of companies support to re-use the known condition in TCI state switching for spatial relation. We also think these two procedures for UE is similar and it’s reasonable to save discussion time for RAN4 group on this issue and directly follow the known condition in TCI state switching.

Proposal 2: The spatial relation associated to DL RS is known if the following conditions are met:

-	During the period from the last transmission of the DL RS resource used for the L1-RSRP measurement reporting for the target spatial relation to the completion of active spatial relation switch, where the DL RS resource for L1-RSRP measurement is the DL RS in target spatial relation or QCLed to the target spatial relation with QCL type-D.

-	Spatial relation switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target spatial relation before the spatial relation switch command

-	The spatial relation remains detectable during the spatial relation switching period

-	The SSB associated with the spatial relation remains detectable during the spatial relation switching period

-	SNR of the spatial relation ≥ -3dB

Otherwise, the spatial relation is unknown.

Fine timing tracking

When RAN4 discussed the requirement of UE transmit timing, it was already agreed that the uplink timing is followed with downlink timing. When UE receives the spatial relation switch command to switch from DL RS0 to DL RS1, the uplink transmission shall follow the new DL RS1’s timing. The timing of new DL RS may be different with the old DL RS’s timing (shown in the figure below). However, UE doesn’t know this timing difference until UE finishes the fine timing tracking based on the new DL RS beam. Thus, a fine timing tracking SSB is always needed in uplink spatial relation switch.

Figure 2. Uplink spatial relation switching associated with DL RS with different timing

Proposal 3: Fine timing tracking is required for uplink spatial relation switch associated with DL RS.

Scheduling Restriction

When the spatial relation is unknown, the UE should execute the Rx beam sweeping to train the downlink spatial domain filter before transmitting the uplink signals with the same spatial domain transmission filter. During the training phase, the UE only has the previous spatial relation information (which UE adopted before receiving the switch command) and also this information is known to network. Thus, UE shall be allowed to be transmitted signals with previous spatial domain transmission filter, but the signal quality cannot be guaranteed before UE finishes the active spatial relation switching.

Proposal 4: For unknown spatial relation switch, UE is allowed to transmit signals with previous spatial domain transmission filter during the Rx beam training phase.

- Active Spatial Relation Switch for each channel and RS

In this section, we provide some discussions about the active spatial relation switch for different channels and RS

- PUCCH

- PUSCH

- SRS

  - Active Spatial Relation switch for PUCCH

The procedure of active spatial relation switch for PUCCH is similar as active TCI state switch for PDSCH in downlink which is shown as follow. RRC is required at first to configure with up to 8 spatial relations, followed by MAC CE based PUCCH spatial relation activation. MAC CE activates one of the spatial relation for PUCCH. The UE will transmit the PUCCH using a same Rx spatial domain filter as a reception of a SSB or CSI-RS or a transmission of a SRS. Thus, RAN4 shall define the MAC based active spatial relation switch when active spatial relation switches for PUCCH.

Proposal 5:  The MAC CE based spatial relation info switching associated with DL-RS for PUCCH is shown as follow.

- For known spatial relation, THARQ +3ms + time for time tracking if applicable;

- For unknown spatial relation, THARQ + 3ms + TL1-RSRP + time for time tracking if applicable.

Figure 3. PUCCH Spatial Relation Activation Procedure

  - Active Spatial Relation switch for PUSCH

The agreements for last meeting is shown as follow. RAN4 had already agreed no requirements will be defined for spatial relation switching for PUSCH.

RAN4 94-e Issue 2-4-1: Spatial relation info switching for PUSCH Agreement: No requirements are defined for spatial relation info switching for PUSCH

Proposal 6:  RAN4 had already agreed no requirements will be defined for spatial relation switching for PUSCH in RAN4 94-e meeting.

  - Active Spatial Relation switch for SRS

The SRS resources also can be periodic, semi-persistent, or aperiodic. Periodic SRS is configured by RRC. Semi-persistent SRS is activated by MAC-CE. Aperiodic SRS is triggered/activated by DCI command. The issue in spatial relation for SRS is whether and how to define the requirement in these scenarios.

The agreements for last meeting is shown as follow. RAN4 had already agreed to define the spatial relation requirement for RRC-based and DCI-based SRS.

RAN4 94-e - Issue 2-2-1: Applicability of RRC based spatial relation info switching delay Agreement: RRC based spatial relation info switch requirements are defined for P-SRS - Issue 2-3: DCI based spatial relation info switch Agreement: DCI based spatial relation info switch requirements are defined for A-SRS. For DCI based spatial relation info switch no requirements for unknown TCI state; refer to RAN1 requirement for known TCI state.

Figure 4. SRS Spatial Relation Activation Procedure

P-SRS

Proposal 7:  The RRC based spatial relation info switching associated with DL-RS for P-SRS is shown as follow.

- For known spatial relation, TRRCprocessing + time for time tracking if applicable;

- For unknown spatial relation, TRRCprocessing + TL1-RSRP + time for time tracking if applicable.

SP-SRS

From our understanding, the spatial relation switching processing for MAC-based SRS shall be the same as the requirement of MAC-based PUCCH. Thus, RAN4 shall also define the MAC-based active spatial relation switching requirement for SP-SRS.

Proposal 8: RAN4 shall define the MAC based active spatial relation switch for semi-persistent SRS and the delay requirement shall be the same as MAC based active spatial relation switch for PUCCH.

AP-SRS

RAN4 had already agreed that the DCI-based spatial relation switch requirements are defined for A-SRS. For DCI based spatial relation switch, there is no requirements for unknown TCI state; and define the requirement for known TCI state refer to RAN1 requirement.

Proposal 9: RAN4 had already agreed to define DCI-based spatial relation switch requirements for A-SRS in RAN4 94-e meeting.

- For unknown condition, there is no requirements;

- For known condition, the requirement refers to RAN1 spec.

- Conclusion

In the contribution, we discuss the issues for active spatial relation switch. We have the following observations and proposals:

Proposal 1: Not to define the requirement when active spatial relation is configured to associated to a SRS in R16.

Proposal 2: The spatial relation associated to DL RS is known if the following conditions are met:

-	During the period from the last transmission of the DL RS resource used for the L1-RSRP measurement reporting for the target spatial relation to the completion of active spatial relation switch, where the DL RS resource for L1-RSRP measurement is the DL RS in target spatial relation or QCLed to the target spatial relation with QCL type-D.

-	Spatial relation switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target spatial relation before the spatial relation switch command

-	The spatial relation remains detectable during the spatial relation switching period

-	The SSB associated with the spatial relation remains detectable during the spatial relation switching period

-	SNR of the spatial relation ≥ -3dB

Otherwise, the spatial relation is unknown.

Proposal 3: Fine timing tracking is required for uplink spatial relation switch associated with DL RS.

Proposal 4: For unknown spatial relation switch, UE is allowed to transmit signals with previous spatial domain transmission filter during the Rx beam training phase.

Proposal 5:  The MAC CE based spatial relation info switching associated with DL-RS for PUCCH is shown as follow.

- For known spatial relation, THARQ +3ms + time for time tracking if applicable;

- For unknown spatial relation, THARQ + 3ms + TL1-RSRP + time for time tracking if applicable.

Proposal 6:  RAN4 had already agreed no requirements will be defined for spatial relation switching for PUSCH in RAN4 94-e meeting.

Proposal 7:  The RRC based spatial relation info switching associated with DL-RS for P-SRS is shown as follow.

- For known spatial relation, TRRCprocessing + time for time tracking if applicable;

- For unknown spatial relation, TRRCprocessing + TL1-RSRP + time for time tracking if applicable.

Proposal 8: RAN4 shall define the MAC based active spatial relation switch for semi-persistent SRS and the delay requirement shall be the same as MAC based active spatial relation switch for PUCCH.

Proposal 9: RAN4 had already agreed to define DCI-based spatial relation switch requirements for A-SRS in RAN4 94-e meeting.

- For unknown condition, there is no requirements;

- For known condition, the requirement refers to RAN1 spec.

- Reference

[1] R4-2002245 WF on spatial relation switch, Mediatek.