3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2003596

# Electronic meeting, 20th – 30th Apr., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | xxxx | rev | - | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-2 on intra-band non-contiguous CA ordering (Rel-16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R16_intra-Core |  |  |  |  |  | Date: |  |  | 2020-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With the increasing number of NR CA combinations, the configuration table in current specification is becoming more and more complex. Therefore, retrieving a specific configuration is a very difficult and time-consuming task. According to the previous agreement proposed in R4-1904912, for intra-band non-contiguous CA with the presentation of “( )”, the sequence should be sorted alphanumerically, i.e. in order of the bandwidth class and the repetition number of sub-block. The sequence of NR CA configurations for intra-band non-contiguous CA in Table 5.5A.2-2 should be reordered. Some intra-band non-contiguous CA for band n260 / n261 with single carrier sub-block should be denoted as ‘n260A’ / ’n261A’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Reorder the sequence of NR CA configurations for intra-band non-contiguous CA. - Editorial correction for intra-band non-contiguous CA with single carrier sub-block with ‘n260A’ and ‘n261A’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The sequence of intra-band non-contiguous CA for FR2 will be in disordered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

5.5A.2	Configurations for intra-band non-contiguous CA

Configurations listed in this clause apply to downlink carrier aggregation only.

NOTE:	Sub-blocks belonging to a CA configuration can be in any order. In other words certain CA configuration acronym includes all sub-block arrangements which have exactly the same sub-block set. As an example, CA_260(3O-2P) denotes CA_260(2O-2P-O), CA_260(P-3O-P) etc. but these are not listed in tables separately.

Table 5.5A.2-1: NR CA configurations with single CA bandwidth class defined for intra-band non-contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | (BWChannel,block) (MHz) | BCS |
| CA_n257(2A) | - | n257A | n257A |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n258(2A) | - | n258A | n258A |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| CA_n258(3A) | - | n258A | n258A | n258A |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| CA_n258(4A) | - | n258A | n258A | n258A | n258A |  |  |  |  |  |  |  |  |  |  |  | 0 |
| CA_n258(5A) | - | n258A | n258A | n258A | n258A | n258A |  |  |  |  |  |  |  |  |  |  | 0 |
| CA_n260(2A) | - | n260A | n260A |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(3A) | - | n260A | n260A | n260A |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(4A) | - | n260A | n260A | n260A | n260A |  |  |  |  |  |  |  |  |  |  | 1600 | 0 |
| CA_n260(5A) | - | n260A | n260A | n260A | n260A | n260A |  |  |  |  |  |  |  |  |  | 2000 | 0 |
| CA_n260(6A) | - | n260A | n260A | n260A | n260A | n260A | n260A |  |  |  |  |  |  |  |  | 2400 | 0 |
| CA_n260(7A) | - | n260A | n260A | n260A | n260A | n260A | n260A | n260A |  |  |  |  |  |  |  | 2600 | 0 |
| CA_n260(8A) | - | n260A | n260A | n260A | n260A | n260A | n260A | n260A | n260A |  |  |  |  |  |  | 2650 | 0 |
| CA_n260(9A) | - | n260A | n260A | n260A | n260A | n260A | n260A | n260A | n260A | n260A |  |  |  |  |  | 26004 | 0 |
| CA_n260(10A) | - | n260A | n260A | n260A | n260A | n260A | n260A | n260A | n260A | n260A | n260A |  |  |  |  | 25504 | 0 |
| CA_n260(2D) | - | CA_n260D | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(2G) | - | CA_n260G | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
| CA_n260(3G) | - | CA_n260G | CA_n260G | CA_n260G |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(4G) | - | CA_n260G | CA_n260G | CA_n260G | CA_n260G |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(2H) | - | CA_n260H | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(2O) | - | CA_n260O | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
| CA_n260(3O) | - | CA_n260O | CA_n260O | CA_n260O |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(4O) | - | CA_n260O | CA_n260O | CA_n260O | CA_n260O |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(2P) | - | CA_n260P | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n260(3P) | - | CA_n260P | CA_n260P | CA_n260P |  |  |  |  |  |  |  |  |  |  |  | 900 | 0 |
| CA_n260(4P) | - | CA_n260P | CA_n260P | CA_n260P | CA_n260P |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2Q) | - | CA_n260Q | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(2A) | - | n261A | n261A |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(3A) | - | n261A | n261A | n261A |  |  |  |  |  |  |  |  |  |  |  | 750 | 0 |
| CA_n261(4A) | - | n261A | n261A | n261A | n261A |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(2D) | - | CA_n261D | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(2G) | - | CA_n261G | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
| CA_n261(3G) | - | CA_n261G | CA_n261G | CA_n261G |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(4G) | - | CA_n261G | CA_n261G | CA_n261G | CA_n261G |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(2H) | - | CA_n261H | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(2I) | - | CA_n261I | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(2O) | - | CA_n261O | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
| CA_n261(3O) | - | CA_n261O | CA_n261O | CA_n261O |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(4O) | - | CA_n261O | CA_n261O | CA_n261O | CA_n261O |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(5O) | - | CA_n261O | CA_n261O | CA_n261O | CA_n261O | CA_n261O |  |  |  |  |  |  |  |  |  | 650 | 0 |
| CA_n261(6O) | - | CA_n261O | CA_n261O | CA_n261O | CA_n261O | CA_n261O | CA_n261O |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(7O) | - | CA_n261O | CA_n261O | CA_n261O | CA_n261O | CA_n261O | CA_n261O | CA_n261O |  |  |  |  |  |  |  | 550 | 0 |
| CA_n261(2P) | - | CA_n261P | CA_n261P |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(2Q) | - | CA_n261Q | CA_n261Q |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Void NOTE 4: Void NOTE 5: Channel bandwidth per operating band defined in Table 5.3.5-1 NOTE 6: Unless otherwise stated, BCS0 is referred in each constituent CA configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.5A.2-2: NR CA configurations and bandwidth combination sets for intra-band non-contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA configuration | Uplink CA configurations | Sub-block |  | Sub-block | Sub-block | Sub-block | Sub-block |  |  | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | Sub-block | (BWChannel,block) (MHz) | BCS |
| CA_n260(A-D) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(A-2D) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(2D) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(A-G) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |  |  |  | 0 |  |  |
| CA_n260(A-2G) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(A-H) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 |  |  |  | 0 |  |  |
| CA_n260(A-2H) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(2H) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(A-I) |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |  |
| CA_n260(A-O) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |  |  |  | 0 |  |  |
| CA_n260(A-2O) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(A-3O) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(A-4O) |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(A-P) |  |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 |  |  |  | 0 |  |  |
| CA_n260(A-2P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(A-3P) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(3P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(A-4P) |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(4P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  | 0 |  |  |
| CA_n260(A-Q) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(A-2Q) |  |  |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(2A-D) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(2A-2D) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2D) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  | 0 |  |  |
| CA_n260(2A-G) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(2A-2G) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(2A-H) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1100 |  |  |  | 0 |  |  |
| CA_n260(2A-2H) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2H) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |  |  | 0 |  |  |
| CA_n260(2A-O) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(2A-2O) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(2A-3O) |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |  |  | 0 |  |  |
| CA_n260(2A-4O) |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  | 0 |  |  |
| CA_n260(2A-P) |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1100 |  |  |  | 0 |  |  |
| CA_n260(2A-2P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |  |  | 0 |  |  |
| CA_n260(2A-3P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(3P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1700 |  |  |  | 0 |  |  |
| CA_n260(2A-4P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(4P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2000 |  |  |  | 0 |  |  |
| CA_n260(2A-Q) | - | CA_n260(2A) |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  | 1200 | 0 |
| CA_n260(2A-2Q) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  | 0 |  |  |
| CA_n260(3A-G) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |  |  | 0 |  |  |
| CA_n260(3A-2G) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  | 0 |  |  |
| CA_n260(3A-O) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |  |  | 0 |  |  |
| CA_n260(3A-2O) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  | 0 |  |  |
| CA_n260(3A-3O) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1800 |  |  |  | 0 |  |  |
| CA_n260(3A-4O) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2000 |  |  |  | 0 |  |  |
| CA_n260(3A-P) |  |  |  | - |  |  |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1300 |  |  |  | 0 |  |  |
| CA_n260(3A-2P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1800 |  |  |  | 0 |  |  |
| CA_n260(3A-Q) |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  | 0 |  |  |
| CA_n260(3A-2Q) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2400 |  |  |  | 0 |  |  |
| CA_n260(4A-G) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1800 |  |  |  | 0 |  |  |
| CA_n260(4A-2G) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2000 |  |  |  | 0 |  |  |
| CA_n260(4A-O) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1800 |  |  |  | 0 |  |  |
| CA_n260(4A-2O) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2000 |  |  |  | 0 |  |  |
| CA_n260(4A-3O) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2200 |  |  |  | 0 |  |  |
| CA_n260(4A-4O) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2400 |  |  |  | 0 |  |  |
| CA_n260(4A-P) |  |  |  | - |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1900 |  |  |  | 0 |  |  |
| CA_n260(4A-2P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2200 |  |  |  | 0 |  |  |
| CA_n260(4A-Q) |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2400 |  |  |  | 0 |  |  |
| CA_n260(4A-2Q) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2400 |  |  |  | 0 |  |  |
| CA_n260(5A-O) |  |  |  | - |  |  |  |  |  |  |  | CA_n260(5A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2400 |  |  |  | 0 |  |  |
| CA_n260(5A-2O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(5A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2400 |  |  |  | 0 |  |  |
| CA_n260(5A-3O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(5A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2600 |  |  |  | 0 |  |  |
| CA_n260(5A-4O) |  |  |  | - |  |  |  |  |  |  |  | CA_n260(5A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 26002 |  |  |  | 0 |  |  |
| CA_n260(5A-P) | - | CA_n260(5A) |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  | 2300 | 0 |
| CA_n260(5A-2P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(5A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2600 |  |  |  | 0 |  |  |
| CA_n260(6A-O) |  |  |  | - |  |  |  |  |  |  |  | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 27002 |  |  |  | 0 |  |  |
| CA_n260(6A-2O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 24502 |  |  |  | 0 |  |  |
| CA_n260(6A-3O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 26002 |  |  |  | 0 |  |  |
| CA_n260(6A-P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2700 |  |  |  | 0 |  |  |
| CA_n260(6A-2P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 26502 |  |  |  | 0 |  |  |
| CA_n260(7A-O) |  |  |  | - |  |  |  |  |  |  |  | CA_n260(7A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 26502 |  |  |  | 0 |  |  |
| CA_n260(7A-2O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(7A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25002 |  |  |  | 0 |  |  |
| CA_n260(7A-3O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(7A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25502 |  |  |  | 0 |  |  |
| CA_n260(8A-O) |  |  |  | - |  |  |  |  |  |  |  | CA_n260(8A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 26002 |  |  |  | 0 |  |  |
| CA_n260(8A-2O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(8A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25502 |  |  |  | 0 |  |  |
| CA_n260(D-G) |  |  |  |  |  |  |  | CA_n260D CA_n260G |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |  |  |  | 0 |  |  |
| CA_n260(D-2G) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(D-H) |  |  | CA_n260D CA_n260H |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 |  |  |  | 0 |  |  |  |
| CA_n260(D-I) |  |  |  |  |  |  |  | CA_n260D CA_n260I |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  | CA_n260I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(D-O) |  |  |  |  |  |  |  | CA_n260D CA_n260O |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |  |  |  | 0 |  |  |
| CA_n260(D-2O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(D-P) |  |  | CA_n260D CA_n260P |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 |  |  |  | 0 |  |  |  |
| CA_n260(D-Q) |  |  |  |  |  |  |  | CA_n260D CA_n260Q |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(2D-O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2D) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(E-O) |  |  |  |  |  |  |  | CA_n260E CA_n260O |  |  |  |  |  |  |  | CA_n260E |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(E-P) |  |  |  |  |  |  |  | CA_n260E CA_n260P |  |  |  |  |  |  |  | CA_n260E |  |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8001 |  |  |  | 0 |  |  |
| CA_n260(E-Q) |  |  |  |  |  |  |  | CA_n260E CA_n260Q |  |  |  |  |  |  |  | CA_n260E |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(G-H) |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  | CA_n260H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 500 |  |  |  | 0 |  |  |
| CA_n260(G-I) |  |  |  |  |  |  |  | CA_n260G CA_n260I |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  | CA_n260I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |  |  |  | 0 |  |  |
| CA_n260(G-O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 400 |  |  |  | 0 |  |  |
| CA_n260(G-2O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |  |  |  | 0 |  |  |
| CA_n260(G-3O) | - | CA_n260G | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n260(G-4O) | - | CA_n260G | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(2G-O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |  |  |  | 0 |  |  |
| CA_n260(2G-2O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(2G-3O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(3O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(2G-4O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(4O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(3G-O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(3G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(4G-O) | - | CA_n260(4G) |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | 1000 | 0 |
| CA_n260(H-O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260H |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 500 |  |  |  | 0 |  |  |
| CA_n260(2H-O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2H) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(O-P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 500 |  |  |  | 0 |  |  |
| CA_n260(O-2P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(O-Q) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |  |  |  | 0 |  |  |
| CA_n260(O-2Q) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(2O-P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 |  |  |  | 0 |  |  |
| CA_n260(2O-2P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(2O-Q) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(2O-2Q) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(P-Q) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 700 |  |  |  | 0 |  |  |
| CA_n260(A-D-O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(A-D-2O) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(A-G-O) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n260(A-G-2O) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(A-2G-O) |  |  |  |  |  |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(A-2G-2O) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(A-O-P) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 900 |  |  |  | 0 |  |  |
| CA_n260(A-O-2P) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(A-O-Q) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(A-O-2Q) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |  |  | 0 |  |  |
| CA_n260(A-2O-P) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1100 |  |  |  | 0 |  |  |
| CA_n260(A-2O-2P) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |  |  | 0 |  |  |
| CA_n260(A-2O-Q) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(A-2O-2Q) |  |  |  |  | - |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  | 0 |  |  |
| CA_n260(A-P-Q) |  |  |  |  | CA_n260P CA_n260Q |  |  |  |  |  |  |  | n260A |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1100 |  |  |  | 0 |  |  |
| CA_n260(2A-D-O) | - | CA_n260(2A) |  |  | CA_n260D | CA_n260O |  |  |  |  |  |  |  |  |  |  | 1400 | 0 |
| CA_n260(2A-D-2O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260D |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  | 0 |  |  |
| CA_n260(2A-G-O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1200 |  |  |  | 0 |  |  |
| CA_n260(2A-G-2O) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260G |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1000 |  |  |  | 0 |  |  |
| CA_n260(2A-2G-O) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |  |  | 0 |  |  |
| CA_n260(2A-2G-2O) |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  | 0 |  |  |
| CA_n260(2A-O-P) | - | CA_n260(2A) |  |  | CA_n260O | CA_n260P |  |  |  |  |  |  |  |  |  |  | 1300 | 0 |
| CA_n260(2A-O-2P) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  | 0 |  |  |
| CA_n260(2A-O-Q) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1400 |  |  |  | 0 |  |  |
| CA_n260(2A-O-2Q) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1800 |  |  |  | 0 |  |  |
| CA_n260(2A-2O-P) |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1500 |  |  |  | 0 |  |  |
| CA_n260(2A-2O-2P) |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2P) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1800 |  |  |  | 0 |  |  |
| CA_n260(2A-2O-Q) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1600 |  |  |  | 0 |  |  |
| CA_n260(2A-2O-2Q) |  |  |  | - |  |  |  |  |  |  |  | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260(2Q) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2000 |  |  |  | 0 |  |  |
| CA_n260(3A-O-P) |  |  |  |  | CA_n260O CA_n260P |  |  |  |  |  |  |  | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n260O |  |  |  |  |  |  |  | CA_n260P |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1700 |  |  |  | 0 |  |  |
| CA_n261(A-D) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 800 |  |  |  | 0 |  |  |
| CA_n261(A-2D) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261(2D) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 |  |  |  | 0 |  |  |
| CA_n261(A-G) |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  |  |  | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 600 |  |  |  | 0 |  |  |
| CA_n261(A-2G) | - | n261A |  | CA_n261(2G) |  |  |  |  |  |  |  |  |  |  |  |  | 7501 |  |
| CA_n261(A-3G) | - | n261A |  | CA_n261(3G) |  |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
| CA_n261(A-4G) | - | n261A |  | CA_n261(4G) |  |  |  |  |  |  |  |  |  |  |  |  | 6501 | 0 |
| CA_n261(A-H) | - | n261A |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(A-2H) | - | n261A |  | CA_n261(2H) |  |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
| CA_n261(A-I) | - | n261A |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-2I) | - | n261A |  | CA_n261(2I) |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
| CA_n261(A-J) | - | n261A |  | CA_n261J |  |  |  |  |  |  |  |  |  |  |  |  | 8001 | 0 |
| CA_n261(A-K) | - | n261A |  | CA_n261K |  |  |  |  |  |  |  |  |  |  |  |  | 8001 | 0 |
| CA_n261(A-O) | - | n261A |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-2O) | - | n261A |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
| CA_n261(A-3O) | - | n261A |  | CA_n261(3O) |  |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
| CA_n261(A-4O) | - | n261A |  | CA_n261(4O) |  |  |  |  |  |  |  |  |  |  |  |  | 6501 | 0 |
| CA_n261(A-5O) | - | n261A |  | CA_n261(5O) |  |  |  |  |  |  |  |  |  |  |  |  | 6001 | 0 |
| CA_n261(A-6O) | - | n261A |  | CA_n261(6O) |  |  |  |  |  |  |  |  |  |  |  |  | 5501 | 0 |
| CA_n261(A-7O) | - | n261A |  | CA_n261(7O) |  |  |  |  |  |  |  |  |  |  |  |  | 5001 | 0 |
| CA_n261(A-P) | - | n261A |  | CA_n261P |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(A-2P) | - | n261A |  | CA_n261(2P) |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
| CA_n261(A-Q) | - | n261A |  | CA_n261Q |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(A-2Q) | - | n261A |  | CA_n261(2Q) |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
| CA_n261(2A-G) | - | CA_n261(2A) |  |  | CA_n261G |  |  |  |  |  |  |  |  |  |  |  | 0 | 0 |
| CA_n261(2A-H) |  | CA_n261(2A) |  |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
| CA_n261(2A-I) | - | CA_n261(2A) |  |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
| CA_n261(3A-G) | - | CA_n261(3A) |  |  |  | CA_n261G |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
| CA_n261(D-G) | CA_n261D CA_n261G | CA_n261D |  | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(D-H) | CA_n261D CA_n261H | CA_n261D |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(D-I) | CA_n261D CA_n261I | CA_n261D |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(D-O) | CA_n261D CA_n261O | CA_n261D |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(D-2O) | - | CA_n261D |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  |  | 7501 | 0 |
| CA_n261(D-P) | CA_n261D CA_n261P | CA_n261D |  | CA_n261P |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(D-Q) | CA_n261D CA_n261Q | CA_n261D |  | CA_n261Q |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(E-O) | CA_n261E CA_n261O | CA_n261E |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  | 800 | 0 |
| CA_n261(E-P) | CA_n261E CA_n261P | CA_n261E |  | CA_n261P |  |  |  |  |  |  |  |  |  |  |  |  | 900 | 0 |
| CA_n261(E-Q) | CA_n261E CA_n261Q | CA_n261E |  | CA_n261Q |  |  |  |  |  |  |  |  |  |  |  |  | 8001 | 0 |
| CA_n261(G-I) | - | CA_n261G |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(G-H) | - | CA_n261G |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  | 500 | 0 |
| CA_n261(G-O) | - | CA_n261G |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  | 400 | 0 |
| CA_n261(G-2O) | - | CA_n261G |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(2G-O) | - | CA_n261(2G) |  |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  | 600 | 0 |
| CA_n261(2G-2O) | - | CA_n261(2G) |  |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
| CA_n261(3G-O) | - | CA_n261(3G) |  |  |  | CA_n261O |  |  |  |  |  |  |  |  |  |  | 7001 | 0 |
| CA_n261(H-I) | - | CA_n261H |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  | 700 | 0 |
| CA_n261(A-D-H) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261D |  |  |  |  |  |  |  |  |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 |  |  |  | 0 |  |  |
| CA_n261(A-D-O) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261D |  |  |  |  |  |  |  |  |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 |  |  |  | 0 |  |  |
| CA_n261(A-D-2O) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261D |  |  |  |  |  |  |  |  |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 |  |  |  | 0 |  |  |
| CA_n261(A-G-H) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261G |  |  |  |  |  |  |  |  |  | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 |  |  |  | 0 |  |  |
| CA_n261(A-G-I) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261G |  |  |  |  |  |  |  |  |  | n261I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 |  |  |  | 0 |  |  |
| CA_n261(A-G-O) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261G |  |  |  |  |  |  |  |  |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 |  |  |  | 0 |  |  |
| CA_n261(A-G-2O) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261G |  |  |  |  |  |  |  |  |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 |  |  |  | 0 |  |  |
| CA_n261(A-2G-O) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n261O |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7001 |  |  |  | 0 |  |  |
| CA_n261(A-2G-2O) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261(2G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CA_n261(2O) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6501 |  |  |  | 0 |  |  |
| CA_n261(A-3G-O) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261(3G) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6501 |  |  |  | 0 |  |  |
| CA_n261(A-H-I) |  |  |  |  | - |  |  |  |  |  |  |  | n261A |  |  |  |  |  |  |  | CA_n261H |  |  |  |  |  |  |  |  |  | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7501 |  |  |  | 0 |  |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: Channel bandwidth per operating band defined in Table 5.3.5-1 NOTE 4: Configurations for intra-band contiguous CA defined in Table 5.5A.1-1 NOTE 5: Configurations for intra-band non-contiguous CA defined in Table 5.5A.2-1 NOTE 6: Void NOTE 7: Unless otherwise stated, BCS0 is referred in each constituent CA configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

==============================================================

<< End of changes >>