3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2003838

Electronic Meeting, 20 – 30 Apr., 2020

Source:	ZTE

Title:	UE behavior in RRC release with re-direction in NR-U

Agenda item:	6.1.5.4

Document for:	Discussion

# 1	Introduction

- In RAN4 94-e, it’s left to future study the UE behavior when L2,max is exceeded [1]. The content in WF [1] is copied below:

- In this paper, we tend to discuss this issue and provide our view.

2	UE behavior when L2 exceeds L2,max

After giving the background of discussions, we discuss the UE behavior when L2 exceeds L2,max, which means when re-directing to a cell in unlicensed band, the UE can’t manage to send RACH because of consecutive UL LBT failures. While in RAN2, there have been discussions on mechanism to handle UL LBT failures (as mentioned by some companies in previous RAN4 meetings [2]), yet it’s not supported for UEs in IDLE or INACTIVE mode to send report on consecutive UL LBT failures. Thus, existing RAN2 mechanism can’t solve the problem.

Existing RAN2 mechanism on UL LBT failure doesn’t apply to UEs in IDLE or INACTIVE mode.

If the UE is not able to transmit RACH due to consecutive UL LBT failures (L2 exceeds L2,max), then the result will be similar when L1 exceeds L1,max, which is the UE fails to camp on the target NR-U cell. In this case, the simple solution for the UE is to allow it to camp on any NR cell, just like when L1 exceeds L1,max. The UE shall complete the process of selecting another NR cell to camp on also in a certain period of time.

When L2 exceeds L2,max, the UE is allowed to camp on any NR cell. The delay requirement shall be similar to the case when L1 exceeds L1,max.

3	Conclusion

- Existing RAN2 mechanism on UL LBT failure doesn’t apply to UEs in IDLE or INACTIVE mode.

- Proposal 1: When L2 exceeds L2,max, the UE is allowed to camp on any NR cell. The delay requirement shall be similar to the case when L1 exceeds L1,max.

References

- R4-2002336

- R4-1914880