3GPP TSG-RAN WG4 #94-e-bis	R4-2003845

E-meeting, 20th – 30th April 2020

Source:	Ericsson

Title:		On FR1 and FR2 for URLLC

Agenda Item:	6.9.1.1

Document for:	Approval

# Introduction

At RAN4#94-e, parameters related to ultra-low BLER tests and demodulation requirements for low latency were discussed. One outstanding issue is whether to define requirements for FR2. In this document, we consider our view on FR2 TDD for the demodulation requirements and ultra-low BLER test.

# Discussion

The ultra-low BLER test aims to establish whether the BS or device has a basic ability to provide 10^-5 BLER. The requirement is related to ultra-reliability applications. In our understanding, it is not necessary to provide ultra-low BLER for FR2.

Proposal 1: Do not develop ultra-low BLER tests for FR2.

Although ultra-low BLER is not needed for FR2, low latency and coverage with medium reliability are still needed for some applications. FR2 inherently reduces latency due to its increased sub-carrier spacing. However, for applications with a fixed payload size as are typical for URLLC, the use of an entire subframe may provide an excessive amount of resources, considering that analogue beamforming is common for FR2 and thus all RBs need to be allocated to the same transmission. Thus, in our view reduced slot lengths with PDSCH and PUSCH mapping type B is a realistic scenario for FR2.

Considering coverage constraints, for applications with greater reliability needs than eMBB slot aggregation can also be applicable.

Proposal 2: Create BS and UE performance requirements for slot aggregation and PDSCH/PUSCH mapping type B for FR2.

# BS parameters for FR2

The link level simulation parameters assumptions for BS non-slot requirement was presented in ‎[2]. The simulations assumptions are listed in table 1 below.

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Subcarrier spacing |  | 60kHz, 120kHz |
| Carrier frequency |  | 28GHz |
| Channel model |  | TDLA30-300 Low |
| Default TDD UL-DL pattern |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| № of Tx antennas |  | 1 |
| № of Rx antennas |  | 2 |
| No of users |  | Single user |
| № of UL scheduled layers |  | 1 |
| Scheduler MCS from index table 2 for PUSCH |  | TBC |
| № of PRBs |  | 34PRbs for 120kHz, 69PRBs for 60kHz |
| HARQ | Maximum number of HARQ transmi ssions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 for 2 symbols length 1 or 2 for 7 symbols length |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PUSCH mapping type | B |
|  | Start symbol | 0 |
|  | Allocation length | 2, 7 |
| Frequency domain resource assignment | RB assignment | 34PRbs for 120kHz, 69PRBs for 60kHz |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Activation | ON, OFF |
|  | Frequency density (KPT-RS) | 2 |
|  | Time density (LPT-RS) | 1 |
| No of slots used in simulations |  | 1 000 000 |
| № of slot aggregation for TDD |  | 1 |
| Testing metric |  | 10% BLER Target |
| Confidence Level |  | 95% |
| Geometry (BLER/SNR) |  | -10: 2: +10 dB |

Table 1: Link level simulation assumptions setup for TDD FR2 non-slot PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Subcarrier spacing |  | 60kHz, 120kHz |
| Carrier frequency |  | 28GHz |
| Channel model |  | TDLA30-300 Low |
| Default TDD UL-DL pattern |  | 60 kHz and 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| № of Tx antennas |  | 1 |
| № of Rx antennas |  | 2 |
| No of users |  | Single user |
| № of UL scheduled layers |  | 1 |
| MCS |  | TBC |
| № of PRBs |  | 34PRbs for 120kHz, 69PRBs for 60kHz |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0 |
| PT-RS |  | OFF |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full bandwidth |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| No of slots used in simulations |  | 1 000 000 |
| № of slot aggregation for TDD |  | 2 |
| Testing metric |  | 1% BLER after HARQ |
| Confidence Level |  | 95% |
| Geometry (BLER/SNR) |  | -10: 2: +10 dB |

Table 2: Link level simulation assumptions setup for TDD FR2 slot aggregation PUSCH

# UE parameters for FR2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 100 |
| Subcarrier spacing |  | kHz | 120 |
| Active DL BWP index |  |  | 1 |
| Channel model |  |  | TDLA30-300 ULA Low |
| Default TDD UL-DL pattern |  |  | 3D1S1U, S=10D:2G:2U |
| PDSCH configuration | Mapping type |  | Type B |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 2 and 7 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
|  | MCS |  | TBC |
|  | MCS Table |  | Table 1 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 for 7os, 0 for 2os |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

Table 3: Link level simulation assumptions setup for TDD FR2 non-slot PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 100 |
| Subcarrier spacing |  | kHz | 120 |
| Active DL BWP index |  |  | 1 |
| Channel model |  |  | TDLA30-300 ULA Low |
| Default TDD UL-DL pattern |  |  | 3D1S1U, S=10D:2G:2U |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 |
|  | PDSCH aggregation factor |  | 2 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
|  | MCS |  | TBC |
|  | MCS Table |  | Table 3 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

Table 4: Link level simulation assumptions setup for TDD FR2 slot aggregation PDSCH

# Conclusion

We kindly ask RAN4 to agree on the following proposal:

Proposal 1: Do not develop ultra-low BLER tests for FR2.

Proposal 2: Create BS and UE performance requirements for slot aggregation and PDSCH/PUSCH mapping type B for FR2.

# References

Huawei, HiSilicon, “Way forward on NR URLLC BS performance requirements”, R4-2002429, electronic meeting, Feb 24th -March 6th, 2020

Ericsson, “Proposals for BS non-slot requirement”, R4-2001180, electronic meeting, Feb 24th -March 6th, 2020